| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0119-v1 |
| --- | --- |

| Title: | Editorial status of alpha blending text and ITU-T Last Call for VSEI v2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Editorial proposed draft text |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan (Microsoft) Ye-Kui Wang (Bytedance) | Email: Email: | garysull@microsoft.com yekui.wang@bytedance.com |
| Source: | Microsoft, Bytedance |  |  |

_____________________________

# Abstract

This contribution discusses the editorial status of alpha blending text description, particularly for VVC v2 text preparation. It reports that further refinement of the editorial description of this part of the text for VVC v2 was conducted after text was submitted for ITU-T Last Call but before corresponding text was submitted for ISO/IEC FDIS, and reports a comment may be submitted in the ITU-T Last Call comment period to request such further refinement to be included in the ITU-T version. Change-marked text is provided to show the differences.

# Text submitted for ITU-T H.274 v2

### 8.23.2	Alpha channel information SEI message semantics

The alpha channel information (ACI) SEI message provides information about alpha channel sample values and post-processing applied to the decoded alpha planes coded in auxiliary pictures of type AUX_ALPHA and one or more associated primary pictures.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i, no picture in the CVS shall be associated with an ACI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i and an ACI SEI message, the SDI SEI message shall precede the ACI SEI message in decoding order.

When an access unit contains an auxiliary picture picA in a layer, with nuh_layer_id equal to nuhLayerIdA, that is indicated as an alpha auxiliary layer by an SDI SEI message, the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA is output.

–	A CLVS containing the auxiliary picture picA ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the ACI SEI message applies.

alpha_channel_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous ACI SEI message in output order that applies to the current layer. alpha_channel_cancel_flag equal to 0 indicates that ACI follows.

Let currPic be the picture that the ACI SEI message is associated with. The semantics of ACI SEI message persist for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an ACI SEI message is output that follows the current picture in output order.

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture should be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture should not be multiplied by the interpretation sample values of the auxiliary coded picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the auxiliary picture is unspecified. Values greater than 2 for alpha_channel_use_idc are reserved for future use by ITU-T | ISO/IEC. When not present, the value of alpha_channel_use_idc is inferred to be equal to 2. Decoders shall ignore alpha channel information SEI messages in which alpha_channel_use_idc is greater than 2.

alpha_channel_bit_depth_minus8 plus 8 specifies the bit depth of the samples of the luma sample array of the auxiliary picture. alpha_channel_bit_depth_minus8 plus 8 shall be equal to the bit depth of the associated primary picture.

alpha_transparent_value specifies the interpretation sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered transparent for purposes of alpha blending. The number of bits used for the representation of the alpha_transparent_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_opaque_value specifies the interpretation sample value of an auxiliary coded picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered opaque for purposes of alpha blending. The number of bits used for the representation of the alpha_opaque_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_channel_incr_flag equal to 0 indicates that the interpretation sample value for each decoded auxiliary picture luma sample value is equal to the decoded auxiliary picture sample value for purposes of alpha blending. alpha_channel_incr_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample value that is greater than Min( alpha_opaque_value, alpha_transparent_value ) should be increased by one to obtain the interpretation sample value for the auxiliary picture sample and any auxiliary picture luma sample value that is less than or equal to Min( alpha_opaque_value, alpha_transparent_value ) should be used, without alteration, as the interpretation sample value for the decoded auxiliary picture sample value.

When alpha_channel_incr_flag is equal to 1, alpha_transparent_value shall not be equal to alpha_opaque_value and Log2( Abs( alpha_opaque_value − alpha_transparent_value ) ) shall have an integer value. A value of alpha_transparent_value that is equal to alpha_opaque_value indicates that the auxiliary coded picture is not intended for alpha blending purposes.

NOTE 1 – For alpha blending purposes, alpha_opaque_value may be greater than alpha_transparent_value, or it may be less than alpha_transparent_value. Interpretation sample values should be clipped to the range of alpha_opaque_value to alpha_transparent_value, inclusive.

alpha_channel_clip_flag equal to 0 indicates that no clipping operation is applied to obtain the interpretation sample values of the decoded auxiliary picture. alpha_channel_clip_flag equal to 1 indicates that the interpretation sample values of the decoded auxiliary picture are altered according to the clipping process described by the alpha_channel_clip_type_flag syntax element. When not present, the value of alpha_channel_clip_flag is inferred to be equal to 0.

alpha_channel_clip_type_flag equal to 0 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than ( alpha_opaque_value + alpha_transparent_value ) / 2 is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less or equal than ( alpha_opaque_value + alpha_transparent_value ) / 2 is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture luma sample. alpha_channel_clip_type_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than alpha_opaque_value is set equal to alpha_opaque_value to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less than or equal to alpha_transparent_value is set equal to alpha_transparent_value to obtain the interpretation sample value for the auxiliary picture luma sample.

NOTE 2 – When both alpha_channel_incr_flag and alpha_channel_clip_flag are equal to one, the clipping operation specified by alpha_channel_clip_type_flag should be applied first followed by the alteration specified by alpha_channel_incr_flag to obtain the interpretation sample value for the auxiliary picture luma sample.

NOTE 3 – Alpha blending composition is normally performed with a background picture B, a foreground picture F, and a decoded auxiliary coded picture A, all of the same size. Assume for purposes of example illustration that the chroma resolution of B and F have been upsampled to the same resolution as the luma. Denote corresponding samples of B, F and A by b, f and a, respectively. Denote luma and chroma samples by subscripts Y, Cb and Cr.

Define the variables alphaRange, alphaFwt and alphaBwt as follows:

alphaRange = Abs( alpha_opaque_value − alpha_transparent_value )	(53)

alphaFwt = Abs( a − alpha_transparent_value )		(54)

alphaBwt = Abs( a − alpha_opaque_value )		(55)

Then, in alpha blending composition, samples d of the displayed picture D could be calculated as follows:

dY = ( alphaFwt * fY + alphaBwt * bY + alphaRange / 2 ) / alphaRange	(56)

dCb = ( alphaFwt * fCb + alphaBwt * bCb + alphaRange / 2 ) / alphaRange	(57)

dCr = ( alphaFwt * fCr + alphaBwt * bCr + alphaRange / 2 ) / alphaRange	(58)

The samples of pictures D, F and B could also represent red, green, and blue component values (see clause 7.3). Here the Y, Cb and Cr component values are assumed. Each component, e.g., Y, is assumed for purposes of example illustration above to have the same bit depth in each of the pictures D, F and B. However, different components, e.g., Y and Cb, need not have the same bit depth in this example.

If alpha_channel_use_idc is equal to 0, the following applies:

- F would be the decoded picture obtained from the decoded luma and chroma, and A would be the decoded picture obtained from the decoded auxiliary coded picture. In this case, the indicated example alpha blending composition involves multiplying the samples of F by factors obtained from the samples of A.

- A picture format that is useful for editing or direct viewing, and that is commonly used, is called pre-multiplied-black video. When the foreground picture was F, the pre-multiplied-black video S is given by

sY  = ( alphaFwt * fY ) / alphaRange		(59)

sCb = ( alphaFwt * fCb ) / alphaRange		(60)

sCr = ( alphaFwt * fCr ) / alphaRange		(61)

- Pre-multiplied-black video has the characteristic that the picture S will appear correct if displayed against a black background. For a non-black background B, the composition of the displayed picture D may be calculated as

dY  = sY + ( alphaBwt * bY + alphaRange / 2 ) / alphaRange	(62)

dCb = sCb + ( alphaBwt * bCb + alphaRange / 2 ) / alphaRange	(63)

dCr = sCr + ( alphaBwt * bCr + alphaRange / 2 ) / alphaRange	(64)

Otherwise, when alpha_channel_use_idc is equal to 1, alpha blending composition does not involve multiplication of the samples of S by factors obtained from the samples of A, S would be the decoded picture obtained from the decoded luma and chroma, and A would again be the decoded picture obtained from the decoded auxiliary coded picture.

# Revised text prepared for FDIS ISO/IEC 23002-7

### 8.23.2	Alpha channel information SEI message semantics

The alpha channel information (ACI) SEI message provides information about alpha channel sample values and post-processing applied to the decoded alpha planes coded in auxiliary pictures of type AUX_ALPHA and one or more associated primary pictures.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i, no picture in the CVS shall be associated with an ACI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i and an ACI SEI message, the SDI SEI message shall precede the ACI SEI message in decoding order.

When an access unit contains an auxiliary picture picA in a layer, with nuh_layer_id equal to nuhLayerIdA, that is indicated as an alpha auxiliary layer by an SDI SEI message, the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

—	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA is output.

—	A CLVS containing the auxiliary picture picA ends.

—	The bitstream ends.

—	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the ACI SEI message applies.

alpha_channel_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous ACI SEI message in output order that applies to the current layer. alpha_channel_cancel_flag equal to 0 indicates that ACI follows.

Let currPic be the picture that the ACI SEI message is associated with. The semantics of ACI SEI message persist for the current layer in output order until one or more of the following conditions are true:

—	A new CLVS of the current layer begins.

—	The bitstream ends.

—	A picture in the current layer in an AU associated with an ACI SEI message is output that follows the current picture in output order.

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture should be multiplied by the interpretation sample values of the decoded auxiliary picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture should not be multiplied by the interpretation sample values of the decoded auxiliary picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the auxiliary picture is unspecified. Values greater than 2 for alpha_channel_use_idc are reserved for future use by ITU-T | ISO/IEC. When not present, the value of alpha_channel_use_idc is inferred to be equal to 2. Decoders shall ignore alpha channel information SEI messages in which alpha_channel_use_idc is greater than 2.

alpha_channel_bit_depth_minus8 plus 8 specifies the bit depth of the samples of the luma sample array of the auxiliary picture. alpha_channel_bit_depth_minus8 plus 8 shall be equal to the bit depth of the associated primary picture.

alpha_transparent_value specifies the interpretation sample value of a decoded auxiliary picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered transparent for purposes of alpha blending. The number of bits used for the representation of the alpha_transparent_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_opaque_value specifies the interpretation sample value of a decoded auxiliary picture luma sample for which the associated luma and chroma samples of the primary coded picture are considered opaque for purposes of alpha blending. The number of bits used for the representation of the alpha_opaque_value syntax element is alpha_channel_bit_depth_minus8 + 9.

A value of alpha_opaque_value that is equal to alpha_transparent_value indicates that the auxiliary coded picture is not intended for alpha blending purposes.

NOTE 1	For alpha blending purposes, alpha_opaque_value can be greater than alpha_transparent_value or it can be less than or equal to alpha_transparent_value.

alpha_channel_incr_flag equal to 0 indicates that the interpretation sample value for each decoded auxiliary picture luma sample value is equal to the decoded auxiliary picture sample value for purposes of alpha blending. alpha_channel_incr_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample value that is greater than Min( alpha_opaque_value, alpha_transparent_value ) should be increased by one to obtain the interpretation sample value for the auxiliary picture sample and any auxiliary picture luma sample value that is less than or equal to Min( alpha_opaque_value, alpha_transparent_value ) should be used, without alteration, as the interpretation sample value for the decoded auxiliary picture sample value.

When alpha_transparent_value is equal to alpha_opaque_value or Log2( Abs( alpha_opaque_value − alpha_transparent_value ) ) does not have an integer value

, alpha_channel_incr_flag shall be equal to 0.

alpha_channel_clip_flag equal to 0 indicates that no clipping operation is applied to obtain the interpretation sample values of the decoded auxiliary picture. alpha_channel_clip_flag equal to 1 indicates that the interpretation sample values of the decoded auxiliary picture are altered according to the clipping process described by the alpha_channel_clip_type_flag syntax element.

alpha_channel_clip_type_flag equal to 0 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than ( alpha_opaque_value + alpha_transparent_value ) / 2 is set equal to Max( alpha_transparent_value, alpha_opaque_value ) to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less or equal than ( alpha_opaque_value + alpha_transparent_value ) / 2 is set equal to Min( alpha_transparent_value, alpha_opaque_value ) to obtain the interpretation sample value for the auxiliary picture luma sample. alpha_channel_clip_type_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the auxiliary picture samples, any auxiliary picture luma sample that is greater than Max( alpha_transparent_value, alpha_opaque_value ) is set equal to Max( alpha_transparent_value, alpha_opaque_value ) to obtain the interpretation sample value for the auxiliary picture luma sample and any auxiliary picture luma sample that is less than or equal to Min( alpha_transparent_value, alpha_opaque_value ) is set equal to Min( alpha_transparent_value, alpha_opaque_value ) to obtain the interpretation sample value for the auxiliary picture luma sample.

When both alpha_channel_incr_flag and alpha_channel_clip_flag are equal to one, the clipping operation specified by alpha_channel_clip_type_flag should be applied first, followed by the alteration specified by alpha_channel_incr_flag, to obtain the interpretation sample value for the auxiliary picture luma sample.

Alpha blending composition is ordinarily performed with a background picture B, a foreground picture F, and a decoded auxiliary picture A, all of the same size. Assume for purposes of example illustration that the chroma resolutions of B and F, if different from the luma resolution, have been upsampled to the same resolution as the luma. Denote corresponding samples of B, F and A by b, f and a, respectively. Denote luma and chroma samples by subscripts Y, Cb and Cr. Each component, e.g., Y, is also assumed for purposes of example illustration to have the same bit depth in each of the pictures B and F. However, different components, e.g., Y and Cb, can have different bit depths in this example. The samples of pictures B and F may alternatively represent green, blue and red component values (see clause 7.3), although the formulae use the subscripts Y, Cb and Cr for the three components.

Define the variables alphaRange, alphaFwt and alphaBwt for each luma sample aY of the auxiliary picture A as follows:

alphaRange = Abs( alpha_opaque_value − alpha_transparent_value )	(53)

alphaFwt = Abs( aY − alpha_transparent_value )		(54)

alphaBwt = Abs( aY − alpha_opaque_value )		(55)

A picture format that is often useful for editing or direct viewing, and that is commonly used, is called pre-multiplied-black video. Pre-multiplied-black video has the characteristic that the decoded picture F will appear the same regardless of whether it is viewed directly without alpha blending composition or is alpha blended with a black background. The use of alpha_channel_use_idc equal to 0 corresponds with source video that is not pre-multiplied-black video, and the use of alpha_channel_use_idc equal to 1 corresponds with source video that is pre-multiplied-black video.

An example of operation of the alpha blending composition process to produce a displayed picture D with sample values d from the pictures B and F is as follows:

—	If alpha_channel_use_idc is equal to 0, the samples d of the displayed picture D are calculated as follows:

dY = ( alphaFwt * fY + alphaBwt * bY + alphaRange / 2 ) / alphaRange	(56)

dCb = ( alphaFwt * fCb + alphaBwt * bCb + alphaRange / 2 ) / alphaRange	(57)

dCr = ( alphaFwt * fCr + alphaBwt * bCr + alphaRange / 2 ) / alphaRange	(58)

—	Otherwise (alpha_channel_use_idc is equal to

1), the samples d of the displayed picture D are calculated as follows:

dY = fY + ( alphaBwt * bY + alphaRange / 2 ) / alphaRange	(59)

dCb = fCb + ( alphaBwt * bCb + alphaRange / 2 ) / alphaRange	(60)

dCr = fCr + ( alphaBwt * bCr + alphaRange / 2 ) / alphaRange	(61)

NOTE 2	In this case, it is expected that the encoder produces its pre-multipled-black source video picture S with sample values s from some original input picture T with sample values t as expressed by formulae 62 to 64, so that when the decoded picture F is a close approximation of the pre-multipled-black source video picture S, the cascaded effect of formulae 62 to 64 followed by formulae 59 to 61 is approximately the same as expressed in formulae 56 to 58.

sY = ( alphaFwt * tY ) / alphaRange		(62)

sCb = ( alphaFwt * tCb ) / alphaRange		(63)

sCr = ( alphaFwt * tCr ) / alphaRange		(64)

NOTE 3	In the event that the background picture B is represented using green, blue and red component values (see clause 7.3) in a manner such that the colour black is represented by all three component values being equal to 0, when the background picture B is black, the operation expressed by equations 59 to 61 becomes simply dY = fY, dCb = fCb, and dCr = fCr. This can help to explain the “pre-multiplied black” term, as the expressions in formulae 62 to 64 are referred to as the pre-multiplication for the black background combination.

For the case with alpha_channel_use_idc equal to 1, somewhat modified processing should be applied when the colour representation domain is different from the use of green, blue, and red colour component values, or with the use of a non-zero black level. Unless the colour black is represented by all three component values bY, bCb, and bCr being equal to 0, formulae 59 to 61 do not simplify to dY = fY, dCb = fCb, and dCr = fCr for pre-multiplied-black video content.