| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0124-v2 |
| --- | --- |

| Title: | Non-EE2: Spatial GPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Yue Yu Haoping Yu Dong Wang | Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes a method that makes use of geometric partitioning mode (GPM) in intra-prediction. This new intra-coding tool partitions a coding block into two parts and generates two corresponding intra-prediction modes. To efficiently express the partition and associated prediction information in the bit-stream, this method employs a template-reordered candidate list, where each candidate in the list comprises a combination of the partition mode and two intra prediction modes, and only signals the candidate index. It is reported that the simulation results are as follows:

XXX

Introduction

GPM is an inter-prediction tool in the VVC standard. It can combine two uni-directional predictions with a geometrical partitioning mask. In ECM, GPM has been further extended and optimized, which now supports inter-inter and inter-intra partitions. However, GPM with two intra partitions is still missing.

DIMD and TIMD are introduced in ECM, and the use of more than two intra prediction modes for a block has already been supported. TIMD can derive two intra prediction modes based on a template and combines them with a derived weight. DIMD can derive three intra prediction modes, including Planar, with a histogram and combines them with a derived weight.

The texture in a block can sometimes be rather complex. In some cases, textures that appear in a block may demonstrate different directions with diverse attributes. In addition, the transitions between these textures can be vague and subtle. When applying GPM in intra prediction, better prediction results are expected from these complex blocks.

Proposed method

This contribution proposes to extend GPM to intra prediction. Because the intra part in inter-intra GPM is called GPM intra in the ECM4.0 software [1], the proposed method is named as Spatial GPM (SGPM).

Similar to inter GPM, SGPM consists of one partition mode and two associated intra prediction modes. If these modes are directly signaled in the bit-stream, as shown in figure 1a, it would yield significant overhead bits. To express the necessary partition and prediction information more efficiently in the bit-stream, a candidate list is employed and only the candidate index is signaled in the bit-stream. Each candidate in the list can derive a combination of one partition mode and two intra prediction modes, as shown in figure 1b.

Figure 1 SGPM signaling

A template is used to generate this candidate list. The shape of the template is shown in figure 2. For each possible combination of one partition mode and two intra prediction modes, a prediction is generated for the template with the partitioning weight extended to the template, as shown in figure 2. These combinations are ranked in ascending order of their SATD between the prediction and reconstruction of the template. The length of the candidate list is set equal to 16, and these candidates are regarded as the most probable SGPM combinations of the current block. Both encoder and decoder construct the same candidate list based upon the template.

To reduce the complexity in building the candidate list, both the number of possible partition modes and the number of possible intra prediction modes are pruned. In the following test, 26 out of 64 partition modes are used, and only the MPMs out of 67 intra prediction modes are used.

Figure 2 template shape and extended weight

Simulation results

The described method is implemented in the ECM-4.0 software and is evaluated using the common test conditions as defined in [2].

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.24% | 0.01% | 0.00% | 176% | 117% |
| Class C | -0.21% | -0.09% | -0.05% | 162% | 115% |
| Class E | -0.31% | -0.17% | -0.16% | 170% | 112% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.11% | 0.07% | 0.01% | 153% | 113% |
| Class F | -0.25% | 0.11% | 0.00% | 136% | 107% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over ECM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.11% | 0.09% | 0.05% | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.03% | 0.00% | -0.16% | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusion

This contribution proposes a method to extend GPM in intra prediction, named SGPM. The proposed method employs a template-reordered candidate list and only signals the candidate index, where each candidate comprises a combination of the one partition mode and the two associated intra prediction modes. The simulation results shows that XXX. It is suggested to further investigate SGPM in the next EE2.

References

[1] ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, January 2022.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).