| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0129-v2 |
| --- | --- |

| Title: | AHG9: SEI for transparency information as auxiliary pictures for transparent screens |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas Pierre Andrivon Fabrice Le Leannec Miloš Radosavljevć Mary-Luc Champel | Tel: Email: | thomase@xiaomi.com pandrivon@xiaomi.com fabricel@xiaomi.com milosr@xiaomi.com champelmaryluc@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

It is asserted that a conventional video content displayed on a transparent display may result in a degraded experience for the viewer where important parts of the image may be lost such as logos, computer windows or characters, etc. Additionally, it is further asserted that the transparency effect perceived by the viewer of those transparent displays is controlled by the sample values of the displayed images and depends on the panel technology and the ambient lightning conditions.

Based on those observations, it is proposed to define metadata payload to signal the content creator intent with respect to how the decoded pictures should look like on a transparent display, i.e., which area should be transparent and to what degree.

To this end, following up on JVET-Y0104, a SEI signaling is presented which enables the use of a dedicated layer for the transport of the necessary information about the intended transparency to be achieved after post-decoding image operations.

# Introduction

Over the past 20 years, transparent displays significantly progressed thanks to more efficient display technologies, first based on LCD and then OLED. This allowed to explore different markets such as head-mounted display, for military and automotive sectors, or digital signage to promote products in shop to customers.

Lately, the technology, especially based on OLED-type of screen, became mature enough to be produced at large scale and for both professional as well as private end users. Examples of such models are the Xiaomi Mi TV Lux, the Panasonic or the LG presented in this contribution.

The key principle of transparent display is that the perceived transparent effect predominantly depends on the sample values in the displayed image and to a lesser degree on the ambient lightning where the display seats. Without adaptation, a conventional content displayed on a transparent display may result in a degraded experience. For instance, a logo or a person may disappear or be a hardly visible while constituting essential content for the viewer. In addition, content creator may leverage the transparency effect for narrative purposes by for instance making object or people appearing or fading away in the picture.

This contribution is a follow-up of JVET-Y0104 [1] wherein a solution based on carrying the transparency information inside SEI messages was initially proposed. However, the group’s recommendation was to investigate a solution based on auxiliary picture, as noted in the meeting report of JVET #25:

It is suggested better using an auxiliary picture, and just signal its properties in an SEI message rather than signalling the mask values in the SEI message. With the proposed method, the bit rate for the mask might become higher than for the video. Such an SEI message should be simple to define, very similar to alpha channels.

As a result, further work was carried out to evaluate this solution. In particular, this contribution presents the methodology implemented for the generation of transparency information sequences based on the JVET CTC sequences. Consequently, the contribution further provides coding test results for the compression of those transparency information sequences as coded pictures. Lastly, the measure bitrates of those sequences are compared against the bitrate for the associated video bitstreams.

Along with test results, this contribution proposes the corresponding SEI signaling for the transport of the transparency information as coded pictures in different layers based on SEI messages. Such signaling would enable the proper post-decoding operations of the decoded pictures from both layers to achieve the intended transparency effect on display.

# Experiment of coding transparency information sequences

## Transparency information sequence generation

In JVET-Y0104 [1], a simulation to estimate the bitrate needed for lossless encoding of transparency information sequence was provided. The simulation relied on considering a single manually created mask as representative for a transparency information sequence. This technique would defeat the purpose of testing an auxiliary picture-based approach if the same transparency information picture was used for the entire sequence. As a result, the new experiment required to generate sequences of transparency information for a given test sequence.

Quite evidently, generating manually a mask for each picture of a test sequence was not feasible. Therefore, it was decided to develop an automatic process for the generation of these transparency information sequences.

The developed Python script builds on existing well-known libraries that are OpenCV and PyTorch. Depending on the type of video sequence, talking person, moving persons, etc., different approaches were needed to generate relevant transparency information sequence. For instance, extracting a person talking in a video sequence illustrates well what could be a relevant transparency information sequence in this case. In another video, a semitransparent effect for the background was more pertinent to emphasize the region of interest in the video such as moving persons. In other video, the narrative effect could imply to separate near and far object in the scenes with different transparency levels. As a result, three generation modes were developed:

- Background extraction

- Object cut-out

- Depth-based mapping

The first two modes are relying on algorithms provided by the OpenCV library. The third mode is using a depth sequence, generated with the PyTorch framework, associated with the video sequence. The different script modes are illustrated in Figure 1.

Figure 1 - Transparency information sequence generation script

Because the CTC test sequences do not come with associated depth sequences, the decision was made to use an AI model called MiDaS [2] to generate an estimated depth map for each picture of those sequences, the AI model MiDaS being itself based on the PyTorch framework [3].

For information, Figure 2 depicts some advertised results of MiDaS generated depth picture.

Figure 2 - Example of MiDaS generated depth map picture

## Examples of generated transparency information sequences

In depth-based mapping, different strategies can be chosen. In the example in Figure 3, the mapping sets transparent the pixels that are far from the camera and opaque the pixels that are near the camera. Between those two extreme values, the transparency is linearly mapped based on the depth value.

| - Original sequence | - Depth map sequence | - Transparency information sequence |
| --- | --- | --- |

Figure 3 - Depth-based linear mapping example

In the object cut-out algorithm, the depth sequence is used to guide the object extraction algorithm. The resulting transparency information is binary as shown on Figure 4.

| - Original sequence | - Depth map sequence | - Transparency information sequence |
| --- | --- | --- |

Figure 4 - Object cut-out example

For background extraction case, the algorithms work well for static camera. The script can be configured to either set the background to the desired transparency level. In Figure 5, the example shows a background semitransparent and the moving persons fully opaque.

| - Original sequence | - Transparency information sequence |
| --- | --- |

Figure 5 - Background extraction example

## Coding results

Several transparency sequences were generated for BasketballDrill, RaceHorses, BasketballDrive and Johnny sequences in their respective native resolution, framerate and in 8 bits.

Table 1 provides results of encoding test done with VTM. The sequences derived from BasketballDrill, RaceHorses, BasketballDrive were encoded in random access configuration while the sequence derived from Johnny in low delay configuration.

Table 1 - Coding test results of transparency information sequences

| Sequence | QP | Bitrate (kbps) | Y-PSNR (dB) |
| --- | --- | --- | --- |
| BasketballDrill cutout 832x480@50 | 22 | 402,7320 | 66,7894 |
|  | 27 | 381,1432 | 61,3001 |
|  | 32 | 372,9792 | 56,1660 |
|  | 37 | 370,1176 | 53,8862 |
| BasketballDrill background 832x480@50 | 22 | 526,2992 | 56,3947 |
|  | 27 | 344,2712 | 50,1861 |
|  | 32 | 224,3104 | 45,6531 |
|  | 37 | 155,5128 | 42,0555 |
| RaceHorses linear depth 832x480@30 | 22 | 282,2272 | 52,2748 |
|  | 27 | 159,2096 | 48,7316 |
|  | 32 | 98,5752 | 45,3909 |
|  | 37 | 63,5640 | 42,0442 |
| BasketballDrive linear depth 1920x1080@50 | 22 | 1177,7408 | 54,2003 |
|  | 27 | 658,6872 | 51,1492 |
|  | 32 | 405,8336 | 48,0606 |
|  | 37 | 260,0840 | 44,8564 |
| Johnny cutout 1280x720@60 | 22 | 232,4784 | 73,3639 |
|  | 27 | 219,6256 | 69,9408 |
|  | 32 | 216,3168 | 61,2749 |
|  | 37 | 215,5808 | 55,4204 |

The coding tests shows that encoding the transparency information sequences at the native resolution and frame rate can be relatively small compared to the bitrate needed for the main video sequence. Bitrate could be further reduced by downsampling the transparency information sequence to a lower spatial resolution or even subsampling in the temporal domain.

It appears then reasonable to envision such transport of the transparency information as coded picture along with the main video sequence.

# Proposal

## Design overview for VVC

The auxiliary pictures concept is a well-known approach wherein each picture of a coded video sequence, called primary pictures, are associated with one auxiliary picture of an auxiliary coded video sequence. For example, in HEVC, primary pictures and auxiliary picture are respectively part of the primary picture layers and the auxiliary picture layers. Even though the VVC specification does not define the concepts of primary and auxiliary pictures, the specification does allow for the presence of different layers in the coded video sequence (CVS). A layer is composed of a set of consecutive picture units (PU) contained the consecutive access unit (AU) composing the CVS. For a given layer, there is at most one PU per AU.

Based on this brief description of the layer design in VVC, we propose to use the layer concept to encode the main video and the transparency information sequence in two separate layers of the same CVS as depicted in Figure 61. Given the design choice in VVC, these two layers are by design also part of OLSs such that the 0-th OLS contains the 0-th layer and the 1-th OLS contains both the 0-th layer and the 1-th layer.

Figure 6 - Transparency information layer

In addition, we propose to add an SEI message in the transparency information layer. The benefit of the SEI is twofold:

- interpret and execute the proper post-decoding operations after decoding both layers to achieve the intended transparency effect

- discard/ignore the transparency information layer for conventional screens

Figure 7 - Transparency information layer with SEI signalling

## Proposed specification text

D.X.1 Transparency information layer SEI message syntax

| transparency_information_layer( payloadSize ) { | Descriptor |
| --- | --- |
| til_cancel_flag | u(1) |
| if(!til_cancel_flag) { |  |
| til_persistence_flag | u(1) |
| til_transparent_luma_value_flag | u(1) |
| if(!til_transparent_luma_value_flag) { |  |
| til_transparent_luma_value | u(v) |
| } |  |
| } |  |
| } |  |

      - D.X.2 Transparency information layer SEI message semantics

The transparency information layer SEI message provides information about transparency information samples and the intended post-processing to be applied to the decoded pictures of the accompanying layer.

When an access unit contains a picture picT with nuh_layer_id equal to nuhLayerIdT and, the transparency information sample values of picT persist in output order until one or more of the following conditions are true:

—	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdT is output.

—	A CLVS containing the picture picT ends.

—	The bitstream ends.

til_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous transparency information layer SEI message in output order. til_cancel_flag equal to 0 indicates that transparency information layer parameters follow.

til_persistence_flag specifies the persistence of the transparency information layer SEI message for the current layer.

til_persistence_flag equal to 0 specifies that the transparency information layer SEI message applies to the decoded picture in the accompanying layer of the same AU.

til_persistence_flag equal to 1 specifies that the transparency information layer SEI message applies to the decoded picture in the accompanying layer of the same AU and persists for all subsequent decoded pictures in the accompanying layer in output order until one or more of the following conditions are true:

- A new CLVS of the current layer begins.

- The bitstream ends.

- A picture in the current layer in an AU associated with a transparency information layer SEI message is output that follows the current picture in output order.

til_transparent_luma_value_flag specifies the presence of til_transparent_luma_value.

til_transparent_luma_value_flag equal to 0 specifies that the value of til_transparent_luma_value is inferred to be 0.

til_transparent_luma_value_flag equal to 1 specifies that til_transparent_luma_value follows.

til_transparent_luma_value specifies the luma sample value corresponding to a full transparent effect in the adjusted picture. The number of bits used for the representation of the til_transparent_luma_value syntax element is sps_bitdepth_minus8 + 8 where sps_bitdepth_minus8 is the bit depth signalled in the accompanying layer.

      - D.9.X Use of the transparency information layer SEI message

When a transparency information layer SEI message is present in a CLVS, the following constraints apply:

- This CLVS shall not be the 0-th layer and the 0-th layer shall not contain any transparency information layer SEI message

- The CLVSS PU shall contain a transparency information layer SEI message

Let denote TransparentValue the array composed of three components equal to defined by (til_transparent_luma_value, 128, 128)

Let denote LumaTransparency[ i ][ j ] the decoded samples of a picture of the transparency information layer.

When the decoded pictures of the transparency information layer and the decoded pictures of the accompanying layer have different resolution in luma samples, a resampling operation of the decoded pictures from the transparency information layer should be applied.

The transparency attribute of a luma sample of a decoded picture is transferred to its associated samples (e.g., chroma samples) and then to the collocated, possibly converted, samples (e.g., RGB samples) before display.

Given a decoded picture of a transparency information layer of the same resolution as the decoded picture of its accompanying layer in luma samples, a post-decoding process can adjust the decoded picture of the accompanying layer by implementing the following pixel-based operation.

For c = 0,

Iadjusted[ c ][ x ][ y ] = Clip3(
						0,
						( 1 << BitDepth ) − 1,
						Idecoded[ c ][ x ][ y ] * LumaTransparency[ x ][ y ] ÷ ( (1 << 8) − 1  ) +
						TransparentValue[ c ] * ( 1 − ( LumaTransparency[ x ][ y ] ÷ ( (1 << 8 ) − 1 ) ) )

And for c=1..2,

Iadjusted[ c ][ x ][ y ] = Clip3(
						0,
						( 1 << BitDepth ) − 1,
						Idecoded[ c ][ x ][ y ] * LumaTransparency[ x * SubWidthC ][ y * SubHeightC ] ÷ ( ( 1 << 8 ) − 1 ) ) ) +
						TransparentValue[ c ] * ( 1 − ( LumaTransparency[ x * SubWidthC ][ y * SubHeightC ] ÷ ( ( 1 << 8) − 1 ) ) )

where Idecoded[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the decoded image Idecoded, Iadjusted [ c ][ x ][ y ] is the transparency-adjusted image at the same position and colour component, where c = 0..2, x = 0..PicWidthInLumaSamples − 1, and y = 0..PicHeightInLumaSamples – 1 for c = 0 and x = 0.. PicWidthInSamplesC − 1, and y = 0.. PicHeightInSamplesC – 1 for c = 1..2.

# Conclusion

We recommend the adoption of the proposed text in a suitable output document for targeting the future new edition of VVC, e.g., Technologies under Consideration or amendment of VVC.

# References

- AHG9: Transparency information SEI for transparent screens, JVET-Y0104, E. Thomas, P. Andrivon, F. Le Leannec, M.-L. Champel (Xiaomi), Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022

- Ranftl, R., Lasinger, K., Hafner, D., Schindler, K., & Koltun, V. (2020). Towards robust monocular depth estimation: Mixing datasets for zero-shot cross-dataset transfer. IEEE transactions on pattern analysis and machine intelligence.

- https://pytorch.org/hub/intelisl_midas_v2/

# Patent rights declaration(s)

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).