| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0130-v5 |
| --- | --- |

| Title: | EE2-related: Motion compensation boundary padding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang, Han Huang, Chun-Chi Chen, Yao-Jen Chang, Y. Zhang, Vadim Seregin, Muhammed Coban, Marta Karczewicz (Qualcomm) Fabrice Le Léannec, Pierre Andrivon, Miloš Radosavljevć, Emmanuel Thomas (Xiaomi) | Tel: Email: | zhizhang@qti.qualcomm.com fabricel@xiaomi.com |
| Source: | Qualcomm Incorporated, Xiaomi |  |  |

_____________________________

# Abstract

This contribution proposes motion compensation boundary padding for reference picture boundary handling. When a reference block locates partially or completely out of the picture boundary, the outside samples are replaced with the proposed motion compensation based padding. The proposed method was implemented on top of ECM-4.0 and test results are summarized as follows:

RA configuration: -0.29%(Y), -0.32%(U), -0.22%(V), 100%(EncT) and 101%(DecT)

LDB configuration: -0.07%(Y), -0.03%(U), -0.26%(V), 100%(EncT) and 102%(DecT)

A combination of the proposed method and EE2-2.2 method “Enhanced bi-directional motion compensation” was tested and simulation results compared to ECM-4.0 are as follows:

RA configuration: -0.35%(Y), -0.28%(U), -0.26%(V), 101%(EncT) and 104%(DecT)

LDB configuration: -0.09%(Y), -0.20%(U), -0.08%(V), 100%(EncT) and 103%(DecT)

The software of the proposed method is included in the submission.

# Introduction

In ECM-4.0, an extended picture area is an area surrounding the picture with a size of (maxCUwidth + 16) in each direction of the picture boundary. The pixel in the extended area is derived by repetitive boundary padding. When a reference block locates partially or completely out of the picture boundary (OOB), the repetitive padded pixel is used for motion compensation (MC), which provides less prediction efficiency.

A method “Enhanced bi-directional motion compensation” in EE2-2.2 [2] proposes to only use the reference block located inside a picture for MC for a bi-directional predicted block. However, for a uni-directional predicted block with an OOB reference block or for a bi-directional predicted block with both OOB reference samples, repetitive padded pixels are used for MC.

A method “Motion compensation boundary padding” was studied in JVET-K meeting CE4.5 [3] where the repetitive boundary padding is replaced by MC boundary padding.

# Proposed method

In the proposed method, samples outside of the picture boundary are derived by motion compensation instead of using only repetitive padding as in ECM. In the implementation, the total padded area size is increased by 64 comparing to ECM to keep MV clipping, which implements repetitive padding, non-normative.

Figure 1. MC boundary padding method

For motion compensation padding, MV of a 4×4 boundary block is utilized to derive a M×4 or 4×M padding block. The value M is derived as the distance of the reference block to the picture boundary as shown on Figure 2. If boundary block is intra coded, then MV is not available, and M is set equal to 0. If M is less than 64, the rest of the padded area is filled with the repetitive padded samples.

Figure 2. An example of deriving a M×4 padding block with a left padding direction

In case of bi-directional inter prediction, only one prediction direction, which has a motion vector pointing to the pixel position farther away from the picture boundary in the reference picture in terms of the padding direction, is used in MC boundary padding.

The pixels in MC padding block are corrected with an offset, which is equal to the difference between the DC values of the reconstructed boundary block and its corresponding reference block.

The proposed method can be used together with method in EE2-2.2: Enhanced bi-directional motion compensation [2].  When for a bi-directional predicted block, one reference block is located out of picture and the other one is inside, only the reference block located inside the picture is used for inter prediction. Otherwise, when the reference block is located outside of a picture and is a uni-directional predicted block, or when both reference blocks are located outside the picture for a bi-directional predicted block, the MC padded samples are used for inter prediction.

# Simulation results

The proposed method was implemented on top of ECM-4.0, it was also tested in combination with EE2-2.2 (Enhanced bi-directional motion compensation) [2].

The performance was evaluated using the common test conditions as defined in [4], anchor is ECM-4.0.

Table 1. Results of MC boundary padding (test 1)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.42% | -0.18% | 100% | 101% |
| Class A2 | -0.22% | -0.27% | -0.24% | 100% | 100% |
| Class B | -0.31% | -0.29% | -0.31% | 99% | 101% |
| Class C | -0.32% | -0.34% | -0.13% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.32% | -0.22% | 100% | 101% |
| Class D | -0.72% | -0.45% | -0.43% | 99% | 104% |
| Class F (optional) | -0.07% | -0.17% | -0.15% | 100% | 102% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.16% | -0.47% | 99% | 101% |
| Class C | -0.04% | 0.34% | -0.23% | 100% | 102% |
| Class E | -0.13% | -0.30% | 0.05% | 100% | 102% |
| Overall | -0.07% | -0.03% | -0.26% | 100% | 102% |
| Class D | -0.06% | 0.38% | -0.20% | 100% | 105% |
| Class F (optional) | 0.06% | 0.67% | 0.33% | 100% | 103% |

Table 2. Results of MC boundary padding and EE2-2.2b combination (test 2)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.33% | -0.06% | 102% | 104% |
| Class A2 | -0.27% | -0.36% | -0.33% | 102% | 104% |
| Class B | -0.37% | -0.17% | -0.30% | 101% | 105% |
| Class C | -0.47% | -0.34% | -0.30% | 101% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | -0.28% | -0.26% | 101% | 104% |
| Class D | -1.09% | -0.68% | -1.03% | 101% | 108% |
| Class F (optional) | -0.07% | -0.13% | -0.19% | 101% | 105% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.10% | -0.23% | 100% | 102% |
| Class C | -0.12% | -0.04% | -0.33% | 100% | 103% |
| Class E | -0.13% | -0.59% | 0.49% | 100% | 103% |
| Overall | -0.09% | -0.20% | -0.08% | 100% | 103% |
| Class D | -0.26% | 0.54% | -0.25% | 101% | 106% |
| Class F (optional) | -0.11% | 1.07% | 0.36% | 100% | 104% |

The proposed method is further tested in combination with EE2-2.2a. The performance is shown in table 3:

Table 3. Results of MC boundary padding and EE2-2.2a combination (test 3)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.36% | -0.19% |  |  |
| Class A2 | -0.28% | -0.35% | -0.29% |  |  |
| Class B | -0.39% | -0.22% | -0.22% | 101% | 104% |
| Class C | -0.45% | -0.42% | -0.32% | 102% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | -0.33% | -0.26% |  |  |
| Class D | -1.07% | -0.58% | -0.66% | 101% | 107% |
| Class F (optional) | -0.04% | -0.16% | -0.07% | 101% | 105% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.09% | -0.24% |  |  |
| Class C | -0.14% | 0.01% | -0.31% | 101% | 102% |
| Class E | -0.17% | -0.51% | 0.41% | 100% | 101% |
| Overall | -0.11% | -0.16% | -0.10% |  |  |
| Class D | -0.34% | 0.22% | 0.30% | 102% | 107% |
| Class F (optional) |  |  |  |  |  |

# Conclusion

In this contribution, motion compensation boundary padding is proposed. The proposed method combined with EE2-2.2 “Enhanced bi-directional motion compensation method” achieves a coding efficiency of { -0.35%, -0.28%, -0.26%} for YUV in RA configuration and { -0.09%, -0.20%, -0.08%} for YUV in LDB configuration. It is suggested to adopt the proposed method into ECM.

# Reference

[1] ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM

[2] V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-Y2024, Jan. 2022.

[3] H. Yang, S. Liu, K. Zhang, “CE4: Summary report on inter prediction and motion vector coding”, JVET-K0024, July. 2018

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).