| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0131-v2 |
| --- | --- |

| Title: | EE2-related: Block Vector Difference Binarization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keming Cao Vadim Seregin Marta Karczewicz | Tel: Email: | kemicao@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a binarization method for IBC block vector difference using Exp-Golomb code with the context coded prefix. The proposed method was implemented on top of ECM-4.0 software, it reportedly provides Y, U, V-BD rate reduction with encoder and decoder runtime as follows:

AI: Class F -0.73% 98% 94%; Class TGM -1.53% 96% 93%

RA: Class F -0.32% 98% 98%; Class TGM -0.98% 101% 100%

# Introduction

In ECM-4.0, Block Vector Difference (BVD) shares the same binarization method with Motion Vector Difference (MVD). For each component, greater than 0, greater than 1 flags are signalled followed by the bypass coded remaining magnitude binarized with EG1 code. The whole flowchart for the binarization method is shown in Fig.1.

Figure 1 Flowchart for BVD binarization method in ECM-4.0.

# Proposed method

In the contribution, it is proposed to remove greater than 1 flag and context code the first 5 bins of the EG1 prefix, all other bins are kept bypass coded.

# Results

The proposed method is implemented on top of ECM-4.0. The test simulations were conducted using the Common Test Conditions as defined in [3]. Test results for ClassF and ClassTGM under AI and RA configurations are summarized in the following tables. More detailed results can be found in the attached spreadsheets.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.73% | -0.49% | -0.54% | 98% | 94% |
| Class TGM | -1.53% | -1.36% | -1.33% | 96% | 93% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.32% | -0.55% | -0.47% | 98% | 98% |
| Class TGM | -0.98% | -0.82% | -0.78% | 101% | 100% |

When greater than 1 flag is kept and the first 5 bins of the EG1 prefix is context coded, the results are shown below.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.69% | -0.31% | -0.49% | 104% | 100% |
| Class TGM | -1.49% | -1.31% | -1.30% | 102% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.38% | -0.48% | -0.46% | 101% | 100% |
| Class TGM | -0.89% | -0.81% | -0.78% | 103% | 102% |

The proposed method is also implemented on top of EE2-3.6, the combined test results are summarized below.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -5.11% | -4.65% | -4.75% | 130% | 101% |
| Class TGM | -10.82% | -9.97% | -9.96% | 113% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -3.35% | -3.34% | -3.41% | 110% | 101% |
| Class TGM | -7.43% | -6.99% | -7.24% | 106% | 99% |

# Conclusion

This contribution proposes a binarization method for block vector difference in IBC using EG1 code with the context coded prefix, improving efficiency of IBC in ECM. We recommend adopting the proposed method.

# References

- ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-4.0.

- Vadim Seregin, Jie Chen, Ling Li, Karam Naser, Jacob Ström, Martin Winken, Xiaoyu Xiu, Kai Zhang “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2).” JVET-Y2024, Jan. 2022.

- Marta Karczewicz, Yan Ye, 	"Common test conditions and evaluation procedures for enhanced compression tool testing," JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).