| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0136-v1_r1 |
| --- | --- |

| Title: | EE2: Enhanced bi-directional motion compensation (test 2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, C.-W. Kuo, Ning Yan, Xianglin Wang (Kwai) Han Huang, Yao-Jen Chang, Chun-Chi Chen, Marta Karczewicz, Vadim Seregin, Yan Zhang, Zhi Zhang (Qualcomm) | Email: | xiaoyuxiu@kwai.com zhizhang@qti.qualcomm.com |
| Source: | Kwai Inc. Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of Exploration Experiment 2 Test-2.2 on enhanced bi-directional motion compensation.

In Test-2.2a, one uni-directional MC sample is regarded as out-of-boundary (OOB) when one of its reference samples is located outside the reference picture beyond half sample. For each prediction sample in a bi-directional block, when one of its uni-directional prediction samples is OOB and the other one is non-OOB, the corresponding bi-directional sample is set equal to the non-OOB sample instead of weighted average of the two uni-directional prediction samples.

In Test-2.2b, the OOB checking is conducted prior to the BDOF process, where the OOB sample in one uni-directional prediction block is replaced by its collocated prediction sample from the other direction if the collocated sample is non-OOB. In such design, the existing weighted average is kept unchanged.

Simulation results are summarized as follows:

- Test-2.2a:

- RA: -0.22% (Y), -0.16% (U), -0.12% (V), 102% (EncT), 104% (DecT)

- LB: -0.03% (Y),  -0.10% (U), -0.04% (V), 101% (EncT), 102% (DecT)

- Test-2.2b:

- RA: -0.27% (Y), -0.20% (U), -0.17% (V), 101% (EncT), 104% (DecT)

- LB: -0.07% (Y),  -0.19% (U), -0.15% (V), 101% (EncT), 102% (DecT)

In the revision, the following decoding run-time results are provided for Test-2.2a with additional software optimizations which produce bit-exact BD-rate performance.

- Test-2.2a (software optimization):

- RA: -0.22% (Y), -0.16% (U), -0.12% (V), 102% (EncT), 101% (DecT)

- LB: -0.03% (Y),  -0.10% (U), -0.04% (V), 101% (EncT), 101% (DecT)

# Introduction

In ECM-4.0, due to the reference samples padding of the reference picture, it is possible for an inter CU to have a reference block located outside the reference picture partially or totally as illustrated in Figure 1. In the Figure 1, bi-directional motion compensation is performed to generate the inter prediction block of the current block. In this example, list 0 reference block is partially out-of-boundary (OOB) while list 1 reference block is fully inside the reference picture. However, the OOB part of the motion compensated blocks usually provides less prediction efficiency because the OOB part is simply repetitive samples derived from the boundary samples within the reference picture.

Figure 1: Bi-directional prediction with the OOB

In the JVET-Y meeting, JVET-Y0125 and JVET-Y0219 were proposed to handle OOB of bi-directional motion compensation. The methods in two contributions were included in Exploration Experiment 2 (EE2) [4] for further study. This contribution reports the simulation results of the EE test.

# Technical aspects

## Test-2.2a

In Test-2.2a, when combining more than one prediction blocks, the OOB prediction samples are discarded and only the non-OOB predictors are used to generate the final predictor. Specifically, let Pos_xi,j and Pos_yi,j denote the position of one prediction sample in one current block, Mv_xi,jLx and Mv_yi,jLx (x = 0,1) denote the MV of the current block; PosLeftBdry, PosRightBdry, PosTopBdry and PosBottomBdry are the positions of four boundaries of the picture. One prediction sample is regarded as OOB when at least one of the following conditions is satisfied:

(Pos_xi,j+Mv_xi,jLx)>(PosRightBdry+half_pixel),

(Pos_xi,j+Mv_xi,jLx)<(PosLeftBdry- half_pixel),

(Pos_yi,j+Mv_yi,jLx)>(PosBottomBdry+ half_pixel),

(Pos_yi,j+Mv_yi,jLx)<(PosTopBdry- half_pixel)

where half_pixel is equal to 8 that represents the half-pel sample distance in the 1/16-pel sample precision.

After examining the OOB condition for each sample, the final prediction samples of one bi-directional block is generated as follows:

If Pi,jL0 is OOB and Pi,jL1 is non-OOB

Pi,jfinal=Pi,jL1

else if Pi,jL0 is non-OOB and Pi,jL1 is OOB

Pi,jfinal=Pi,jL0

else

Pi,jfinal=(Pi,jL0+Pi,jL1+1)>>1

Please be noted that the above OOB checking process is also applicable when BCW is enabled.

## Test-2.2b

In Test-2.2b, the OOB checking is conducted prior to the BDOF process, where the OOB samples in one uni-directional prediction block is replaced by its collocated prediction sample from the other direction if the collocated sample is non-OOB, as depicted as

If Pi,jL0 is OOB and Pi,jL1 is non-OOB

Pi,jL0=Pi,jL1

else if Pi,jL0 is non-OOB and Pi,jL1 is OOB

Pi,jL1=Pi,jL0

Then, the existing BDOF or weighted averaging is applied without any modifications.

# Simulation results

The proposed scheme was implemented on top of the ECM4.0 [2] and tested using the common test conditions as defined in [5].

Table 1 Results of Test-2.2a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.20% | -0.02% | 102% | 104% |
| Class A2 | -0.15% | -0.20% | -0.12% | 103% | 105% |
| Class B | -0.24% | -0.12% | -0.18% | 102% | 104% |
| Class C | -0.29% | -0.16% | -0.13% | 102% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.16% | -0.12% | 102% | 104% |
| Class D | -0.79% | -0.41% | -0.54% | 101% | 104% |
| Class F | -0.04% | -0.04% | -0.17% | 102% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.13% | -0.16% | 101% | 102% |
| Class C | -0.07% | 0.04% | -0.10% | 101% | 102% |
| Class E | -0.01% | -0.25% | 0.24% | 102% | 102% |
| Overall | -0.03% | -0.10% | -0.04% | 101% | 102% |
| Class D | -0.23% | 0.62% | -0.06% | 101% | 103% |
| Class F | -0.07% | 0.57% | 0.06% | 101% | 101% |

Table 2 Results of Test-2.2b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.30% | -0.02% | 102% | 104% |
| Class A2 | -0.18% | -0.25% | -0.21% | 102% | 105% |
| Class B | -0.29% | -0.10% | -0.24% | 101% | 104% |
| Class C | -0.35% | -0.20% | -0.19% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.27% | -0.20% | -0.17% | 101% | 104% |
| Class D | -0.87% | -0.40% | -0.57% | 100% | 104% |
| Class F | -0.05% | -0.13% | -0.14% | 102% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.21% | -0.18% | 100% | 101% |
| Class C | -0.06% | 0.00% | -0.02% | 101% | 102% |
| Class E | -0.10% | -0.39% | -0.26% | 101% | 102% |
| Overall | -0.07% | -0.19% | -0.15% | 101% | 102% |
| Class D | -0.23% | 0.27% | -0.14% | 100% | 103% |
| Class F | -0.08% | 0.60% | 0.08% | 101% | 101% |

# In the following, additional results are provided for Test2.2a where two pure software optimizations (as described below) are applied which produces bit-exact BD-rate performance while reducing the average decoding run-time for RA and LDB configurations:

- Some unnecessary memory set and copy operations in original software implementation of Test2.2a are removed.

- SIMD implementation is provided for the sample-wise OOB checking, i.e., determining whether one prediction sample regarded as out-of-boundary.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.20% | -0.02% | 102% | 102% |
| Class A2 | -0.15% | -0.20% | -0.12% | 103% | 100% |
| Class B | -0.24% | -0.12% | -0.18% | 102% | 101% |
| Class C | -0.29% | -0.16% | -0.13% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.16% | -0.12% | 102% | 101% |
| Class D | -0.79% | -0.41% | -0.54% | 101% | 104% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.13% | -0.16% | 101% | 102% |
| Class C | -0.07% | 0.04% | -0.10% | 101% | 100% |
| Class E | -0.01% | -0.25% | 0.24% | 102% | 102% |
| Overall | -0.03% | -0.10% | -0.04% | 101% | 101% |
| Class D | -0.23% | 0.62% | -0.06% | 101% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# References

- ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- Y.-W. Chen, C.-W. Kuo, N. Yan, W. Chen, X. Xiu, X. Wang, “AHG12: Enhanced bi-directional motion compensation”, JVET-Y0125, Feb. 2022.

- H. Huang, Y-J. Chang, C.-C. Chen, M. Karczewicz, V. Seregin, Y. Zhang, Z. Zhang, “Cross-check of JVET-Y0125 (AHG12: Enhanced bi-directional motion compensation)”, JVET-Y0219, Feb. 2022.

- V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-Y2024, Apr. 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Apr. 2022.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).