| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0140-v2 |
| --- | --- |

| Title: | AHG12: Enhanced CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Che-Wei Kuo, Xiaoyu Xiu, Ning Yan, Hong-Jheng Jhu, W. Chen, H. Gao, Xianglin Wang | Email: | {cheweikuo, xiaoyuxiu, yanning03, jhuhong-jheng, chenwei06, hangao, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

In VVC/ECM, CCLM achieves significant coding performance improvement by exploiting strong correlation between luma/chroma components. However, only single downsampled luma sample is considered when deriving the linear model, neglecting the possible correlation among neighbouring luma samples. In this contribution, it is observed that video sequences containing dramatic luma intensity change may lead to corresponding chroma value change, known as the purple fringing problem. Two techniques, namely Filter-based Linear Model (FLM) and Gradient Linear Model (GLM), are proposed to tackle the situation when luma gradients are highly correlated to chroma values. The FLM extends the Simple Linear Regression (SLR) in the CCLM to Multiple Linear Regression (MLR), while the GLM keeps SLR and uses luma gradients to predict chroma samples.

Compared to ECM-4.0 anchors, the BD-rate impact is summarized as below.

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| FLM | -0.85% | -0.49% | -0.68% | -0.36% | -0.38% | -0.35% |
| GLM | -0.53% | -0.42% | -0.55% | -0.21% | -0.28% | -0.19% |

Introduction

Purple fringing is one well-known color artefact in modern cameras [1]. Figure 1 shows examples observed in common sequences. The fringing phenomenon may result from multiple reasons, including 1) high luminance blooming which incurs saturated photodiode quantum-well and charge leakage in a charge coupled device (CCD); 2) demosaicing built in cameras to interpolate missing colors due to Bayer pattern Color Filter Array (CFA) arrangement, which may introduce aliasing when abrupt color changes; 3) chromatic aberration caused by different refractive indices for different RGB colors, leading to different focal lengths that affect color accuracy.

In VVC/ECM, CCLM achieves significant coding performance by exploiting of strong correlation between luma/chroma components. However, only single collocated, downsampled luma sample is considered when deriving the linear model, neglecting the possible correlation among neighbouring luma samples. As observed in Figure 1, video sequences containing dramatic luma intensity change may lead to corresponding chroma value change, known as the purple fringing problem. Two techniques, namely Filter-based Linear Model (FLM) and Gradient Linear Model (GLM), are proposed to tackle the situation when luma gradients are highly correlated to chroma values.

Figure 1 ParkRunning3, POC228. DaylightRoad2, POC131. BQTerrace, POC60. CatRobot, POC290.

Proposal

Filter-based Linear Model

The FLM extends the Simple Linear Regression (SLR) in the CCLM to Multiple Linear Regression (MLR), which can be formulated as

C=i=0N-1αi∙Li+β

where C is the to-be-predicted chroma sample; Li is the i-th reconstructed luma sample surround the chroma sample, αi is the i-th coefficient, β is the offset, and N is the number of the involved luma samples. As illustrated in Figure 2, the same number of neighbouring luma/chroma template samples as CCLM, i.e., top 2 rows/left 3 columns luma samples and top 1 row/left 1 column chroma samples, are used to derive the MLR parameters through the Cholesky decomposition. In the current FLM design, multiple filter shapes are supported with N ranging from 2 to 6. The selection of filter shape is switched at CU level.

Figure 2 Reconstructed neighbouring luma/chroma samples are used to derive the MLR parameters.

Gradient Linear Model

Compared with CCLM, instead of downsampled luma values, the GLM utilizes luma sample gradients to derive the linear model. In other words, rather than the low-pass filtered luma sample, a gradient G is replaced in the CCLM process. The other designs of CCLM (e.g., parameter derivation, prediction sample linear transform) are kept unchanged.

C=α∙G+β

Figure 3 shows that G can be calculated by one of four Sobel based gradient patterns.

For signaling, when the CCLM mode is enabled to the current CU, two flags are signaled separately for Cb/Cr component to indicate whether GLM is enabled to the component; if the GLM is enabled for one component, one syntax element is further signaled to select one of four gradient patterns in Figure 3 for gradient calculation.

Figure 3 Four Sobel based gradient patterns for GLM.

Simulation results

The proposed techniques were implemented on ECM-4.0 reference software [2] and tested by following the common test condition (CTC) as specified in [3]. Significant luma/chroma coding performance improvement are observed on colorful sequences containing dramatic luma intensity change, as well as screen content sequences.

FLM

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.14% | -0.08% | 100% | 101% |
| Class A2 | -4.72% | -1.54% | -3.85% | 99% | 100% |
| Class B | -0.13% | -0.40% | 0.19% | 100% | 100% |
| Class C | 0.02% | -0.13% | -0.07% | 101% | 101% |
| Class E | -0.05% | -0.44% | -0.38% | 99% | 98% |
| Overall | -0.85% | -0.49% | -0.68% | 100% | 100% |
| Class D | 0.05% | 0.13% | -0.03% | 100% | 99% |
| Class F | -0.67% | -2.13% | -1.76% | 99% | 99% |
| Class TGM | -3.11% | -6.22% | -5.83% | #VALUE! | #VALUE! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.24% | 0.01% | 100% | 100% |
| Class A2 | -1.56% | -0.73% | -2.04% | 100% | 100% |
| Class B | -0.07% | -0.50% | 0.01% | 100% | 100% |
| Class C | 0.00% | -0.07% | 0.18% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | -0.38% | -0.35% | 100% | 100% |
| Class D | 0.02% | 0.06% | -0.05% | 99% | 99% |
| Class F | -1.10% | -2.48% | -2.22% | 99% | 99% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

GLM

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.24% | 0.00% | 104% | 98% |
| Class A2 | -3.12% | -1.63% | -3.36% | 109% | 98% |
| Class B | -0.08% | -0.35% | -0.10% | 104% | 98% |
| Class C | 0.03% | -0.01% | 0.04% | 105% | 99% |
| Class E | -0.02% | -0.07% | 0.17% | 104% | 100% |
| Overall | -0.53% | -0.42% | -0.55% | 105% | 99% |
| Class D | 0.04% | 0.24% | -0.06% | 105% | 100% |
| Class F | -0.46% | -2.53% | -1.57% | 103% | 99% |
| Class TGM | -3.09% | -8.63% | -8.00% | #VALUE! | #VALUE! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.28% | 0.19% | 102% | 100% |
| Class A2 | -1.05% | -0.85% | -1.44% | 102% | 100% |
| Class B | -0.03% | -0.16% | -0.02% | 102% | 99% |
| Class C | 0.02% | 0.01% | 0.25% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.21% | -0.28% | -0.19% | 102% | 99% |
| Class D | 0.02% | 0.21% | -0.04% | 100% | 98% |
| Class F | -1.05% | -2.60% | -1.92% | 101% | 99% |
| Class TGM | -2.12% | -4.25% | -4.12% | #VALUE! | #VALUE! |

References

- Baek-Kyu Kim and Rae-Hong Park, "Automatic detection and correction of purple fringing using the gradient information and desaturation," 2008 16th European Signal Processing Conference, 2008, pp. 1-5.

- ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Apr. 2022.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).