| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0142-v2 |
| --- | --- |

| Title: | EE2-related: On MMVD and Affine MMVD Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar, Yuwen He, Kai Zhang, Na Zhang, Li Zhang (Bytedance) 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | mehdi.salehifar@bytedance.com yuwen.he@bytedance.com zhangkai.video@bytedance.com zhangna.cynthia@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

A template matching based reordering method for extended MMVD (TMR-EMMVD) was proposed (JVET-Y0067) and adopted into ECM-4.0 during last JVET meeting.

In this proposal, a simple extension of JVET-Y0067is proposed with the following aspects:

- One sided MMVD position is added for the bi-prediction cases.

- Extra base positions are added depending on the neighboring blocks.

On top of the ECM-4.0, simulation results of the proposed method are reported as below:

RA: -0.18%, -0.27%, -0.21%, 102%, 102%;                  LDB: -0.22%, -0.39%, -0.58%, 104%, 103%.

# Introduction

A template matching based reordering method for extended MMVD (TMR-EMMVD) was proposed (JVET-X0085 [1], JVET-Y0067 [2]) and adopted into ECM4 at the last JVET meeting.

TMR-EMMVD related part includes the following aspects:

- MMVD refinement positions are reordered based on template costs.

- Additional positions are added along new angles.

- After the reordering process, selected positions are kept as available positions, consequently for MMVD index coding.

- Similar concept is also extended to Affine MMVD.

# Proposed method

In this document we propose an extension of TMR-EMMVD with the following aspects:

Aspect #1:

For a bi-prediction MMVD base in VVC or ECM, the MVD is added to one list, but derived from the first list MVD with a possible scaling and mirroring depending on the POC differences for the other list, known as two-sided MMVD.

On top of the current design, we propose one-sided MMVD (or affine MMVD), where individual MVD is used for each list independently, where a non-zero MVD is applied for list X, but a zero MVD is applied for list (1-X). We also propose to remove the scaling process for the two-sided MMVD.

It needs to be noted that prediction calculated for one list, could be reused for the same list with similar MVD, thus this aspect does not add any extra prediction calculation.

Aspect #2:

We propose to increase the number of the bases for MMVD from 2 to 3.

For affine MMVD, we also propose to increase the bases from 1, to up to 3, depending on the neighboring block affine flags.

Some encoder speed up is also applied.

# Simulation results

The proposed methods have been implemented on top of the ECM-4.0 [3]. Testing results under RA/LDB configurations are summarized below. The simulations were performed following the common test conditions [4]. More detailed results can be found in the attached spreadsheet.

Table 1: Performance of the proposed method

|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.29% | -0.24% | 102% | 101% |  |  |  |  |  |
| Class A2 | -0.22% | -0.36% | -0.29% | 102% | 101% |  |  |  |  |  |
| Class B | -0.19% | -0.24% | -0.19% | 103% | 102% | -0.26% | -0.55% | -0.42% | 103% | 102% |
| Class C | -0.12% | -0.22% | -0.14% | 103% | 102% | -0.18% | 0.05% | -0.61% | 104% | 105% |
| Class E |  |  |  |  |  | -0.22% | -0.69% | -0.81% | 104% | 102% |
| Overall | -0.18% | -0.27% | -0.21% | 102% | 102% | -0.22% | -0.39% | -0.58% | 104% | 103% |
| Class D | -0.09% | 0.00% | -0.14% | 103% | 99% | -0.23% | 0.22% | -0.24% | 102% | 103% |
| Class F | -0.12% | -0.20% | -0.31% | 102% | 101% | -0.10% | 0.66% | 0.09% | 101% | 101% |

# Conclusions

In this contribution, an extension of TMR-EMMVD is proposed. With negligible impact on complexity, it brings reasonable gain.  It is recommended to adopt the proposed method in ECM.

# Reference

[1] M. Salehifar, Y. He, K. Zhang, N. Zhang, L. Zhang, “Non-EE2: Template Matching-based Reordering for Extended MMVD Design,” JVET-X0085, Oct. 2021.

[2] M. Salehifar et al., “EE2-3.9 and EE2-3.10: TM based reordering for MMVD and affine MMVD and MVD sign prediction”, JVET-Y0067, Jan. 2022.

[3] ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-4.0.

[4] M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).