| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0143-v2 |
| --- | --- |

| Title: | Non-EE2: Chroma intra modes derived from collocated luma blocks and neighboring chroma blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Keming Cao, Bappaditya Ray, Vadim Seregin, Marta Karczewicz | Email: | yjchang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to add multiple direct modes, neighbouring chroma modes, and secondary chroma modes into the chroma intra mode list. The multiple direct modes are derived from the intra prediction modes of collocated luma blocks. The neighbouring chroma modes are derived from the intra prediction modes of neighbouring chroma blocks. The secondary chroma modes are derived from the chroma intra angular modes currently existed in the chroma intra mode list. On top of ECM-4.0, the experimental results are reported as follows:

AI: -0.08% (Y), -0.75% (U), -0.67 % (V), 101% (EncT), 99% (DecT)

RA: -0.04% (Y), -0.36% (U), -0.23% (V), 100% (EncT), 100% (DecT)

# Introduction

(a) Luma partition structure               (b) Chroma partition structure

Figure 1. Separate block partitioning structure for luma and chroma components in I slices, where the luma CU covering the position c is used to derive DM mode.

In ECM [1], there are 11 intra prediction modes for chroma intra coding, which are listed as follows:

- 6 cross-component linear model (CCLM) modes, i.e., LM, LM_L, LM_A, MMLM, MMLM_L, and MMLM_A.

- 5 traditional intra modes, i.e., direct mode (DM) and 4 default modes, where DM mode directly depends on the intra prediction mode of the corresponding luma block. Since separate block partitioning structure for luma and chroma components is enabled in I slices as shown in Figure 1, one chroma block may correspond to multiple luma blocks. Chroma DM mode is directly set to be the intra prediction mode of the corresponding luma block covering the center position of the current chroma block.

A CCLM flag is first parsed to indicate if it is CCLM mode or non-CCLM mode. If CCLM flag is true, LM flag is parsed, and MMLM flag is further parsed if LM flag is false. If LM flag and MMLM flag are all false, a fix-length index coding is parsed to indicate which mode of the remaining CCLM modes is used. A DM flag is parsed if the CCLM flag is false. If DM flag is true, DM mode is used for current chroma CU. If DM flag is false, a 2-bit fix-length coding is parsed to indicate which default mode is used for current chroma CU.

(a) Luma partition structure                         (b) Chroma partition structure

Figure 2. MDMs derived from the co-located luma CUs corresponding to different positions in current chroma CU

# Proposed chroma intra modes

This contribution proposes to replace 5 non-CCLM chroma intra modes (i.e., DM and 4 default modes) with 9 chroma intra modes derived from the intra prediction modes of collocated luma blocks and neighbouring chroma blocks. More specifically, four groups of chroma intra modes are used to derive 9 chroma intra modes, and the first 9 unique modes are added into the non-CCLM chroma mode list in the following order:

- Multiple direct modes (MDM): the chroma intra mode is set to be the intra prediction mode of collocated luma block corresponding to a position in current chroma CU. Figure 2(a) shows the luma blocks in the positions of C, TL, TR, BL and BR that are used to derive the MDMs.

- The neighbouring chroma modes: the chroma intra mode is set to be the intra prediction mode of neighbouring chroma blocks. Figure 2(b) shows the locations of chroma neighbouring blocks in the positions of 0, 1, 2, 3, and 4 that are used to derive the neighbouring chroma modes.

- The secondary chroma modes: the chroma intra mode is derived by offsetting +1 or -1 to the first two chroma intra modes in the non-CCLM chroma mode list.

- Default modes: PLANAR_IDX, VER_IDX, HOR_IDX, DC_IDX, VDIA_IDX, VER_IDX - 4, VER_IDX + 4, HOR_IDX - 4, HOR_IDX + 4.

A truncated unary coding (0, 10, 110, 1110, …, 11111110, 11111111) is used to code the index of the nine chroma intra modes.

# Simulation results

The proposed chroma modes are tested on top of ECM-4.0 under the common test conditions [2]. The results are summarized in Table 1.

Table 1. Performance of the proposed chroma modes on top of ECM-4.0

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.56% | -0.40% | 101% | 100% | -0.03% | -0.31% | -0.08% | 101% | 101% |
| Class A2 | -0.07% | -0.76% | -0.78% | 102% | 100% | -0.04% | -0.36% | -0.23% | 100% | 100% |
| Class B | -0.07% | -0.66% | -0.74% | 100% | 98% | -0.04% | -0.44% | -0.46% | 99% | 99% |
| Class C | -0.08% | -0.51% | -0.51% | 101% | 100% | -0.04% | -0.29% | -0.07% | 99% | 99% |
| Class E | -0.16% | -1.39% | -0.90% | 100% | 97% |  |  |  |  |  |
| Overall | -0.08% | -0.75% | -0.67% | 101% | 99% | -0.04% | -0.36% | -0.23% | 100% | 100% |
| Class D | -0.07% | -0.34% | -0.54% | 102% | 100% | 0.00% | -0.42% | -0.37% | 99% | 99% |
| Class F | -0.06% | -0.09% | -0.54% | 101% | 99% | 0.00% | -0.26% | -0.42% | 100% | 100% |

The proposed chroma modes are also tested on top of ECM-4.0 + EE2-1.1 + EE2-1.2b under the common test conditions [2]. The results are summarized in Table 2.

Table 2. Performance of the proposed chroma modes on top of ECM-4.0 + EE2-1.1 + EE2-1.2b

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -2.51% | -2.50% | 101% | 94% | -0.03% | -1.02% | -0.89% | 98% | 96% |
| Class A2 | -0.13% | -2.41% | -1.70% | 106% | 97% | -0.06% | -1.17% | -0.64% | 99% | 99% |
| Class B | -0.13% | -2.56% | -2.75% | 101% | 96% | -0.05% | -1.35% | -1.50% | 100% | 98% |
| Class C | -0.15% | -1.94% | -2.02% | 102% | 94% | -0.04% | -0.62% | -0.65% | 102% | 102% |
| Class E | -0.15% | -2.44% | -1.87% | 101% | 93% |  |  |  |  |  |
| Overall | -0.13% | -2.37% | -2.22% | 102% | 95% | -0.05% | -1.05% | -0.98% | 100% | 99% |
| Class D | -0.12% | -1.56% | -1.51% | 103% | 99% | -0.05% | -0.64% | -0.93% | 104% | 103% |
| Class F | -0.23% | -1.50% | -1.88% | 100% | 95% | -0.12% | -0.73% | -0.75% | 101% | 101% |

# Conclusion

This contribution proposes to add multiple direct modes, neighbouring chroma modes, and secondary chroma modes into the chroma intra mode list. The proposed chroma modes provide promising coding gains with negligible encoding and decoding time change.

# References

- M. Coban, F. Le Léannec, K. Naser, and J. Ström, “Algorithm description of Enhanced Compression Model 4 (ECM 4),” JVET-Y2025, Apr. 2022.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Feb. 2022.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).