| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0145-v2 |
| --- | --- |

| Title: | EE2-2.5 related: More test results for ARMC with refined motion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Kai Zhang2, Na Zhang1, Zhipin Deng1, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports more test results for EE2-2.5. Adaptive reordering of merge candidates (ARMC) with refined motion is tested in EE2-2.5. Based on Test #2.5b, more simplified changes are proposed in this contribution (i.e., Test #2.5c). First, only the above or left template is used during the motion refinement of TM when the block is flat or narrow. Second, only the first four merge candidates are reordered in TM merge mode instead of the first eight merge candidates in Test #2.5b. Third, when constructing the AMVP list, an MVP candidate with a TM cost larger than a threshold is skipped. Fourth, TM is extended to perform 1/16-pel MVD precision in TM merge mode. Compared to ECM-4.0, simulation results of Test #2.5c are reported as below:

RA: {-0.10%, -0.13%, -0.10%; 101%, 101%};

LDB: {-0.06%, -0.03%, -0.06%; 102%, 101%}.

Introduction

Test results for EE2-2.5 on ARMC with refined motion are reported in JVET-Z0058 [1]. In ARMC with refined motion, each merge candidate in the merge candidate list is refined using TM/multi-pass DMVR first and the refined motion will be used in ARMC to reorder the merge candidate list. When multi-pass DMVR is used to derive the refined motion, only the first pass (i.e., PU level) of multi-pass DMVR is applied in reordering. ARMC with refined motion as proposed in JVET-Y0151 is tested in Test #2.5a. while a simplified version is tested in Test #2.5b. In Test #2.5b, when template matching is used to derive the refined motion, the template size is set equal to 1. And only the first eight merge candidates are reordered with the refined motion in TM merge mode.

Proposed method

To get a better trade-off for ARMC with refined motion, this contribution presents more simplified changes on top of Test #2.5b. First, only the above or left template is used during the motion refinement of TM when the block is flat (w > 2×h) or narrow (h > 2× w). Second, only the first four merge candidates are reordered in TM merge mode instead of the first eight merge candidates in Test #2.5b. Third, when constructing the AMVP list, an MVP candidate with a TM cost larger than a threshold, which is equal to five times of the cost of the first MVP candidate, is skipped. Fourth, TM is extended to perform 1/16-pel MVD precision in TM merge mode.

Simulation results

The proposed simplification of ARMC with refined motion is implemented on top of ECM-4.0 [2]. Simulations are performed following the common test conditions [3]. The results of Test #2.5c compared to ECM-4.0 are shown in Table 1.

Table 1: Test results of Test 2.5c

|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.27% | -0.03% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.11% | -0.16% | -0.17% | 101% | 101% |  |  |  |  |  |
| Class B | -0.10% | 0.00% | -0.10% | 102% | 101% | -0.04% | -0.11% | 0.27% | 102% | 101% |
| Class C | -0.14% | -0.18% | -0.09% | 102% | 102% | -0.03% | 0.16% | -0.45% | 102% | 102% |
| Class E |  |  |  |  |  | -0.12% | -0.17% | -0.10% | 102% | 100% |
| Overall | -0.10% | -0.13% | -0.10% | 101% | 101% | -0.06% | -0.03% | -0.06% | 102% | 101% |
| Class D | -0.10% | -0.04% | -0.20% | 103% | 102% | -0.03% | 1.01% | 0.43% | 102% | 102% |
| Class F | -0.06% | -0.12% | -0.12% | 101% | 101% | -0.18% | -0.07% | -0.47% | 101% | 101% |

Conclusions

In this contribution, ARMC with refined motion is further simplified. It is recommended to adopt the proposed simplification of ARMC with refined motion in ECM.

Reference

- Y. Wang, K. Zhang, N. Zhang, Z. Deng, and L. Zhang, “EE2-2.5: Adaptive reordering of merge candidates with refined motion,” Document of Joint Video Experts Team, JVET-Z0058, by teleconference, April 2022.

- ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-4.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).