| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0146-v1 |
| --- | --- |

| Title: | AHG12: Using samples before deblocking filter for adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In ECM-4.0, in-loop filters are applied in the following order: deblocking filter, operating in parallel bilateral filter, sample adaptive offset, cross-component sample adaptive offset, and adaptive loop filter. ALF only uses neighbouring samples processed by SAO. In this contribution, for luma component, it is proposed that neighbouring reconstructed samples before deblocking filter are also utilized in ALF. For those additional samples, two filter shapes, 3x3 and 5x5 diamond shapes, are tested. Test results for the proposed method implemented on top of ECM-4.0 following the common test conditions are reported as follows.

|  | All intra |  |  | Random access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Y | EncT | DecT | BD-rate Y | EncT | DecT | BD-rate Y | EncT | DecT |
| 3x3 | -0.16% | 101% | 102% | -0.21% | 100% | 101% | -0.15% | 102% | 102% |
| 5x5 | -0.19% | 103% | 104% | -0.23% | 102% | 102% | -0.18% | 104% | 103% |

# Introduction

In ECM-4.0, for luma component, two 13x13 diamond shape fixed filters are applied to derive two intermediate samples. After that, a signalled filter is applied to these two intermediate samples and neighboring samples to derive a filtered sample as

Rx,y=Rx, y+i=019cifi,0+fi,1+i=2021cigi,

(1)

where fi,j  is the clipped difference between a neighboring sample and current sample Rx, y, and gi is the clipped difference between an intermediate sample and current sample Rx, y. The filter coefficients ci, i=0,…21, are signalled. The input samples to adaptive loop filter (ALF) are the neighboring samples processed by sample adaptive offset (SAO).

# Proposed methods

In this contribution, for luma component, it is proposed that samples before deblocking filter (DBF) are used for ALF as well. Specifically, a filtered sample is derived as

Rx,y=Rx, y+i=019cifi,0+fi,1+i=2021cigi+i=22Ncihi,0+hi,1

(2)

where hi,j  is the clipped difference between a neighboring sample before DBF and current sample Rx, y. Two filter shapes are proposed, which are 3x3 and 5x5 diamond shapes as follows.

Figure 1. Proposed 3x3 and 5x5 filter shapes which are applied to samples before DBF with N=24 and N=28 in Eq. (2) respectively.

In an adaptation parameter set, a flag is signalled to indicate whether samples before DBF are used for ALF. In this proposal, this flag is always set as true at encoder.

# Results

The proposed method was implemented on top EMC-4.0 and tests were performed following ECM CTC. The test results of the proposed method are summarized in the next tables.

Table 1. Proposed method using 3x3 filter shape for DBF samples.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | 0.02% | 0.02% | 101% | 103% |
| Class A2 | -0.20% | 0.02% | 0.02% | 100% | 103% |
| Class B | -0.21% | 0.02% | 0.02% | 101% | 102% |
| Class C | -0.11% | 0.00% | 0.00% | 101% | 101% |
| Class E | -0.09% | 0.02% | 0.02% | 101% | 102% |
| Overall | -0.16% | 0.02% | 0.02% | 101% | 102% |
| Class D | -0.06% | 0.02% | 0.02% | 101% | 99% |
| Class F | -0.16% | 0.03% | 0.04% | 99% | 99% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.21% | 0.08% | 99% | 100% |
| Class A2 | -0.24% | 0.02% | -0.02% | 99% | 101% |
| Class B | -0.27% | 0.13% | 0.07% | 100% | 102% |
| Class C | -0.15% | 0.09% | 0.17% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.21% | 0.03% | 0.08% | 100% | 101% |
| Class D | -0.10% | 0.05% | 0.11% | 102% | 102% |
| Class F | -0.14% | -0.11% | -0.04% | 103% | 102% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | 0.20% | -0.05% | 101% | 102% |
| Class C | -0.07% | 0.26% | -0.32% | 103% | 101% |
| Class E | -0.31% | -0.42% | -0.16% | 104% | 103% |
| Overall | -0.15% | 0.07% | -0.17% | 102% | 102% |
| Class D | -0.11% | 0.30% | 0.38% | 101% | 102% |
| Class F | -0.37% | 0.85% | 0.26% | 101% | 100% |

Table 2. Proposed method using 5x5 filter shape for DBF samples.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | 0.02% | 0.02% | 104% | 105% |
| Class A2 | -0.26% | 0.03% | 0.03% | 102% | 104% |
| Class B | -0.23% | 0.03% | 0.03% | 103% | 104% |
| Class C | -0.14% | 0.01% | 0.01% | 102% | 104% |
| Class E | -0.10% | 0.03% | 0.03% | 103% | 103% |
| Overall | -0.19% | 0.02% | 0.02% | 103% | 104% |
| Class D | -0.09% | 0.07% | 0.07% | 102% | 100% |
| Class F | -0.23% | 0.06% | 0.06% | 102% | 101% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.19% | 0.14% | 102% | 102% |
| Class A2 | -0.30% | 0.08% | 0.02% | 102% | 102% |
| Class B | -0.30% | 0.10% | 0.04% | 103% | 102% |
| Class C | -0.14% | -0.02% | -0.02% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.23% | 0.01% | 0.04% | 102% | 102% |
| Class D | -0.05% | 0.03% | 0.16% | 105% | 101% |
| Class F | -0.26% | 0.01% | -0.09% | 106% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | 0.07% | -0.18% | 102% | 102% |
| Class C | -0.18% | 0.09% | -0.32% | 104% | 103% |
| Class E | -0.25% | -0.41% | -0.16% | 109% | 103% |
| Overall | -0.18% | -0.04% | -0.22% | 104% | 103% |
| Class D | -0.26% | 0.30% | 0.41% | 105% | 101% |
| Class F | -0.10% | 1.11% | 0.41% | 106% | 99% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).