| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0147-v2 |
| --- | --- |

| Title: | Non-EE2: Improvement of Inter-MTS in ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray Vadim Seregin Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121, USA | Email: | bray@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

The current Inter-MTS is disabled in CTC due to a poor trade-off. In this contribution, certain modification of Inter-MTS is proposed to improve this trade-off. The performance on top of ECM-4.0 of this modified Inter-MTS is shown below.

RA: -0.26% (Y)/ 0.29% (U)/ 0.32%(V), EncT: 104%, DecT: 101%

LDB: -0.29% (Y)/0.29%(U)/0.23% (V), EncT: 104%, DecT: 101%

In the updated version, as an additional result, some further encoder speedup (non-normative) is proposed. The results on top of ECM-4.0 is:

RA: -0.22% (Y)/ 0.25% (U)/ 0.27%(V), EncT: 101%, DecT: 100%

LDB: -0.xx% (Y)/0.xx%(U)/0.xx% (V), EncT: 1xx%, DecT: xx%

# Introduction

Currently, in ECM-4.0, for the MTS of inter-coded CUs, four candidates: {(DST7, DST7), (DST7, DCT8), (DCT8, DST7), (DCT8, DCT8)} are used for every CU. As reported in JVET-Y0159, Inter-MTS provides compression benefit (~ 0.35% in RA) but at the expense of higher encoding runtime (~20% increase in RA) [2].

# Proposed method

In this contribution, the following modification of Inter-MTS is done.

- For the larger resolution sequences (width > 1080) maximum CU size for Inter-MTS usage is set to 32 (i.e., Inter-MTS is used for CU with width <=32 and height <=32), and for the remaining sequences (smaller resolution) it is set to 16.

- As proposed in JVET-D0126, for 4-pt, 8-pt and 16-pt transforms, the current AMT transform cores, i.e., DST-7 and DCT-8, is replaced with separable KLTs.

# Results

The proposed method was implemented on top ECM-4.0. The anchor is run with CTC condition [1] and for the test, Inter-MTS is enabled (MTS: 3). The results of the proposed method are summarized below.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | 0.20% | 0.46% | 104% | 101% |
| Class A2 | -0.42% | 0.30% | 0.33% | 105% | 100% |
| Class B | -0.24% | 0.39% | 0.27% | 102% | 101% |
| Class C | -0.23% | 0.23% | 0.28% | 104% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.26% | 0.29% | 0.32% | 104% | 101% |
| Class D | -0.29% | 0.44% | 0.45% | 105% | 101% |
| Class F | -0.07% | -0.01% | 0.00% | 102% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.24% | 0.26% | 0.27% | 102% | 101% |
| Class C | -0.41% | 0.61% | 0.27% | 107% | 101% |
| Class E | -0.20% | -0.07% | 0.10% | 101% | 101% |
| Overall | -0.29% | 0.29% | 0.23% | 104% | 101% |
| Class D | -0.37% | 0.89% | 0.47% | 107% | 101% |
| Class F | -0.05% | 0.77% | 0.79% | 103% | 100% |

Additionally, further encoder speedup is applied, and the results shown below.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.01% | 0.25% | 101% | 102% |
| Class A2 | -0.34% | 0.24% | 0.31% | 101% | 98% |
| Class B | -0.19% | 0.35% | 0.22% | 101% | 100% |
| Class C | -0.22% | 0.34% | 0.32% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | 0.25% | 0.27% | 101% | 100% |
| Class D | -0.27% | 0.53% | 0.44% | 103% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.39% | 0.61% | 0.36% | 105% | 101% |
| Class E | -0.20% | -0.23% | 0.44% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.42% | 1.51% | 0.72% | 103% | 100% |
| Class F | -0.05% | 0.73% | 0.30% | 100% | 97% |

# References

[1] M. Karczewicz, Y. Yen, "Common Test Conditions and evaluation procedures for enhanced compression tool testing", JVET-X2017.

[2] T. Hashimoto, T. Ikai, “EE2-related: inter MTS refinement on adaptive intra MTS (EE2-4.4)”, JVET-Y0159.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).