| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0156-v1 |
| --- | --- |

| Title: | AHG4: New test sequences for JVET exploration |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen Yan Ye Rui Li Wenfei Jiang | Email: | jiechen.cj@alibaba-inc.com, yan.ye@alibaba-inc.com, yichen.lr@alibaba-inc.com, wenfei.jwf@taobao.com |
| Source: | Alibaba group, Youku |  |  |

_____________________________

- Abstract

This contribution proposes 2 new 4K test contents (4 sequences in total in 8- and 10-bit) for future video coding exploration. The proposed sequences are from TV drama content on Youku [1]. The coding results of VTM-11.0-anc and ECM-4.0 on these sequences and the copyright information are also provided in this contribution.

Introduction

The ECM common test conditions[2] currently include a total of six 4K sequences. This contribution proposes two new 4K test contents (4 sequences in total as both 8-bit and 10-bit versions are provided) for future video coding exploration. Both of them are TV content provided by Youku.

Proposed sequences

The following Table 1 summarizes the information of the proposed content. StarAndSea and PostalService are 4K sequences at 25fps, and we have provided both 8-bit version and 10-bit version. The 540p demo version of the proposed contents in mp4 files is also attached to the contribution.

Table 1 The proposed sequences information

| Sequence | Thumbnail and brief description | File name | resolution @fps | Bitdepth |
| --- | --- | --- | --- | --- |
| StarAndSea | Three people with motion, the background is changed sometimes | StarAndSea_3840x2160_25_8bit.yuv | 3840x2160@25fps | 8bit |
|  |  | StarAndSea_3840x2160_25_10bit.yuv |  | 10bt |
| PostalService | Two people walking downstairs, the background keep changing | PostalService_3840x2160_25_8bit.yuv | 3840x2160@25fps | 8bit |
|  |  | PostalService_3840x2160_25_10bit.yuv |  | 10bt |

Coding results

The proposed sequences are coded with VTM11.0-anc and ECM-4.0. VTM-11.0-anc is the VTM-11.0 with JVET-V0056, JVET-Y0085 and JVET-Y0152 integrated to align with ECM software and it is used as the anchor for ECM in AHG6. To compare coding gain of the proposed sequences with current CTC sequences, VTM11.0-anc is also used as the anchor in this contribution.

The tests follow the common test conditions of ECM [2] and the first 10 seconds of the sequences are coded. Both 8-bit sequences and 10-bit sequences are coded with internal coding bit-depth of 10. The results are shown in Table 2 to Table 4.

Table 2 Test results in AI

|  |  | VTM-11.0-anc |  |  |  | ECM-4.0 |  |  |  | BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP | kbps | Y psnr | U psnr | V psnr | kbps | Y psnr | U psnr | V psnr | Y | U | V |
| StarAndSea | 22 | 2367.30 | 51.26 | 51.18 | 50.58 | 2259.81 | 51.36 | 51.33 | 50.74 | -8.73% | -15.59% | -14.19% |
| 10bit | 27 | 1352.07 | 49.40 | 49.49 | 48.32 | 1284.31 | 49.55 | 49.78 | 48.66 |  |  |  |
|  | 32 | 823.36 | 47.16 | 47.76 | 46.06 | 781.37 | 47.38 | 48.24 | 46.57 |  |  |  |
|  | 37 | 511.56 | 44.45 | 46.43 | 44.36 | 491.00 | 44.75 | 47.04 | 45.10 |  |  |  |
| StarAndSea | 22 | 2381.40 | 50.13 | 49.94 | 49.51 | 2270.92 | 50.20 | 50.05 | 49.63 | -8.77% | -15.57% | -14.39% |
| 8bit | 27 | 1353.08 | 48.62 | 48.61 | 47.64 | 1285.90 | 48.75 | 48.85 | 47.94 |  |  |  |
|  | 32 | 823.47 | 46.67 | 47.17 | 45.65 | 782.34 | 46.87 | 47.56 | 46.12 |  |  |  |
|  | 37 | 511.60 | 44.18 | 45.99 | 44.08 | 490.62 | 44.46 | 46.50 | 44.76 |  |  |  |
| PostalSerive | 22 | 1081.06 | 57.05 | 57.98 | 58.74 | 1017.98 | 57.34 | 58.51 | 59.31 | -10.49% | -20.75% | -20.88% |
| 10bit | 27 | 682.16 | 54.11 | 54.53 | 55.20 | 643.09 | 54.47 | 55.49 | 56.24 |  |  |  |
|  | 32 | 436.98 | 50.87 | 51.41 | 52.01 | 414.52 | 51.31 | 52.86 | 53.46 |  |  |  |
|  | 37 | 280.67 | 47.43 | 48.94 | 49.72 | 268.85 | 47.94 | 50.84 | 51.65 |  |  |  |
| PostalSerive | 22 | 1085.60 | 53.58 | 53.62 | 53.97 | 1085.63 | 54.01 | 54.20 | 54.54 | -10.75% | -21.49% | -21.89% |
| 8bit | 27 | 682.69 | 52.00 | 52.03 | 52.46 | 643.48 | 52.22 | 52.51 | 52.94 |  |  |  |
|  | 32 | 437.04 | 49.73 | 50.01 | 50.46 | 413.78 | 50.06 | 50.95 | 51.41 |  |  |  |
|  | 37 | 280.38 | 46.88 | 48.08 | 48.74 | 268.80 | 47.32 | 49.58 | 50.16 |  |  |  |

Table 3 Test results in RA

|  |  | VTM-11.0-anc |  |  |  | ECM-4.0 |  |  |  | BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP | kbps | Y psnr | U psnr | V psnr | kbps | Y psnr | U psnr | V psnr | Y | U | V |
| StarAndSea | 22 | 3914.66 | 49.64 | 50.67 | 49.76 | 3508.52 | 49.78 | 50.86 | 49.94 | -17.88% | -24.44% | -23.11% |
| 10bit | 27 | 2074.93 | 47.60 | 49.26 | 48.07 | 1836.77 | 47.83 | 49.59 | 48.40 |  |  |  |
|  | 32 | 1142.42 | 45.03 | 47.78 | 46.24 | 1000.50 | 45.36 | 48.21 | 46.72 |  |  |  |
|  | 37 | 658.77 | 42.29 | 46.30 | 44.38 | 571.30 | 42.73 | 46.78 | 44.94 |  |  |  |
| StarAndSea | 22 | 3906.02 | 48.81 | 49.54 | 48.84 | 3500.75 | 48.93 | 49.67 | 48.98 | -18.00% | -24.36% | -23.31% |
| 8bit | 27 | 2070.18 | 47.05 | 48.41 | 47.42 | 1834.23 | 47.26 | 48.67 | 47.71 |  |  |  |
|  | 32 | 1140.94 | 44.72 | 47.17 | 45.82 | 998.43 | 45.03 | 47.53 | 46.25 |  |  |  |
|  | 37 | 658.90 | 42.13 | 45.85 | 44.08 | 570.09 | 42.54 | 46.28 | 44.62 |  |  |  |
| PostalSerive | 22 | 1673.54 | 54.89 | 57.49 | 58.17 | 1454.29 | 55.27 | 57.94 | 58.64 | -22.96% | -29.99% | -30.77% |
| 10bit | 27 | 921.99 | 51.95 | 54.86 | 55.48 | 792.05 | 52.46 | 55.62 | 56.34 |  |  |  |
|  | 32 | 533.27 | 48.70 | 52.34 | 52.99 | 451.14 | 49.33 | 53.35 | 54.02 |  |  |  |
|  | 37 | 323.76 | 45.52 | 49.74 | 50.50 | 270.93 | 46.24 | 51.22 | 51.96 |  |  |  |
| PostalSerive | 22 | 1729.07 | 52.59 | 53.38 | 53.72 | 1540.17 | 53.17 | 53.54 | 53.87 | -24.08% | -30.97% | -31.53% |
| 8bit | 27 | 929.60 | 50.51 | 52.16 | 52.56 | 793.17 | 50.89 | 52.54 | 52.95 |  |  |  |
|  | 32 | 537.53 | 47.95 | 50.61 | 51.08 | 450.67 | 48.47 | 51.27 | 51.72 |  |  |  |
|  | 37 | 327.77 | 45.14 | 48.73 | 49.32 | 271.26 | 45.79 | 49.82 | 50.38 |  |  |  |

Table 4 Test results in LDB

|  |  | VTM-11.0-anc |  |  |  | ECM-4.0 |  |  |  | BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP | kbps | Y psnr | U psnr | V psnr | kbps | Y psnr | U psnr | V psnr | Y | U | V |
| StarAndSea | 22 | 4667.64 | 49.64 | 50.07 | 49.00 | 4147.57 | 49.76 | 50.29 | 49.22 | -16.94% | -26.91% | -25.84% |
| 10bit | 27 | 2261.67 | 47.04 | 48.28 | 46.81 | 1986.94 | 47.25 | 48.67 | 47.26 |  |  |  |
|  | 32 | 1237.95 | 44.16 | 46.73 | 44.97 | 1081.25 | 44.45 | 47.28 | 45.58 |  |  |  |
|  | 37 | 722.14 | 41.27 | 45.32 | 43.26 | 630.62 | 41.63 | 46.01 | 44.07 |  |  |  |
| StarAndSea | 22 | 4640.03 | 48.79 | 49.06 | 48.21 | 4123.98 | 48.90 | 49.24 | 48.39 | -17.01% | -26.71% | -26.07% |
| 8bit | 27 | 2244.49 | 46.53 | 47.58 | 46.31 | 1973.29 | 46.73 | 47.91 | 46.71 |  |  |  |
|  | 32 | 1229.84 | 43.86 | 46.22 | 44.61 | 1078.22 | 44.14 | 46.70 | 45.20 |  |  |  |
|  | 37 | 717.27 | 41.11 | 44.97 | 43.03 | 626.70 | 41.47 | 45.61 | 43.76 |  |  |  |
| PostalSerive | 22 | 2179.97 | 53.96 | 55.38 | 56.22 | 1897.68 | 54.24 | 56.05 | 56.74 | -20.32% | -33.21% | -32.05% |
| 10bit | 27 | 1073.51 | 50.48 | 52.61 | 53.36 | 925.52 | 50.88 | 53.44 | 54.14 |  |  |  |
|  | 32 | 603.29 | 47.05 | 50.13 | 50.76 | 517.23 | 47.54 | 51.40 | 52.02 |  |  |  |
|  | 37 | 362.70 | 43.84 | 47.81 | 48.63 | 310.86 | 44.39 | 49.47 | 50.32 |  |  |  |
| PostalSerive | 22 | 2134.17 | 51.86 | 52.43 | 52.94 | 1905.60 | 52.38 | 52.99 | 53.47 | -21.04% | -33.87% | -31.33% |
| 8bit | 27 | 1087.54 | 49.49 | 50.79 | 51.36 | 936.67 | 49.81 | 51.31 | 51.75 |  |  |  |
|  | 32 | 609.38 | 46.61 | 48.97 | 49.61 | 524.00 | 47.06 | 49.94 | 50.44 |  |  |  |
|  | 37 | 365.98 | 43.64 | 47.12 | 47.79 | 314.41 | 44.19 | 48.52 | 49.14 |  |  |  |

Copyright statement

The proposed sequences are produced by Alibaba Group（licensed by one of its subsidiary Youku） and all intellectual property rights remain with Alibaba Group.

The following uses are allowed for the contributed sequences:

1.  Sequences may be published in technical papers, played at technology research and development events.

2.  Sequences may be used by Standards committees. (e.g., ITU, MPEG, VQEG).

The following uses are NOT allowed for the contributed sequences:

1.  Do not publish snapshots in product brochures.

2.  Do not use video for marketing purposes.

3.  Do not redistribute video with a commercial product.

4.  Do not use in television shows, commercials, or movies.

5.  Do not sublicense or transfer the sequences to anyone who has not received this license.

Reference

- Youku, https://www.youku.com/

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).