| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0157-v3 |
| --- | --- |

| Title: | EE2-3.2-related: IBC Adaptation for Camera-Captured Contents |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu 8910 University Center Lane San Diego, CA92122, USA | Tel: Email: | +1-619-436-7216 xujizheng@bytedane.com |
| Source: | ByteDance Inc. |  |  |

_____________________________

# Abstract

This contrubution describes an IBC adaption scheme based on EE2-3.2 and presents the IBC-on results. The overall BD-rate(Y)/EncTime results for AI are -0.55%/108% respectively.

# Introduction

IBC is an effective tool for screen content coding. It also shows coding efficiency improvement for some camera-capture contents at the cost of significant amount of encoding time increase. This contribution describes an IBC adaption scheme based on EE2-3.2 [1] software and shows that IBC can obtain good coding gains at a controllable increase of encoding time.

# Proposed scheme

The proposed scheme is based on EE2-3.2 software. The decoder is exactly the same as EE2-3.2 when CTU size is 128x128. It means that the the reference area for IBC is extended to two CTU-rows above the current CTU, as shown in Figure 1. Specifically, for CTU (m,n) to be coded, the reference area includes CTUs with index (m–2,n–2)…(W,n–2),(0,n–1)…(W,n–1),(0,n)…(m,n), where W denotes the maximum horizontal index within the current picture.

However, when CTU size is 256x256, which is the default setting in coding of classA sequences, two additional rows of CTU above may require extra memory. To keep IBC from using extra memory, when CTU size is 256x256, the reference area is shown in Figure 2. Specifically, assume that the current CTU index is (m, n), the reference area includes CTUs with index (0,n)…(m,n) and (m–1,n–1)…(W,n–1).

Besides the change of EE2-3.2 decoder when CTU size is 256x256, the encoder of EE2-3.2 is modified to limit the per-sample block vector search (or called local search) range to be [–12,12] horizontally and [–12,12] vertically centered at the first block vector predictor for each IBC block.

Figure 1. Reference area for IBC when CTU size is 128x128. The green and blue blocks denote the reference area while the white blocks denote invalid reference area.

Figure 2. Reference area for IBC when CTU size is 256x256.

# Experimental results

Table 1. The summarized result of the modified software with IBC on compared with ECM-4.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.03% | -0.08% | 110% | 100% |
| Class A2 | -0.94% | -1.40% | -1.31% | 111% | 101% |
| Class B | -0.67% | -0.72% | -0.78% | 107% | 100% |
| Class C | -0.18% | -0.20% | -0.06% | 107% | 101% |
| Class E | -0.96% | -1.01% | -1.02% | 107% | 100% |
| Overall | -0.55% | -0.65% | -0.63% | 108% | 100% |
| Class D | -0.49% | -0.49% | -0.74% | 104% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.09% | 0.20% | 99% | 101% |
| Class A2 | -0.22% | -0.40% | -0.21% | 98% | 97% |
| Class B | -0.20% | -0.32% | -0.34% | 99% | 100% |
| Class C | 0.23% | -0.05% | 0.18% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.22% | -0.07% | 99% | 100% |
| Class D | 0.19% | -0.27% | -0.48% | 97% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.06% | -0.45% | 106% | 98% |
| Class C | -0.05% | 0.22% | -0.23% | 104% | 99% |
| Class E | -0.25% | -0.24% | -0.50% | 103% | 99% |
| Overall | -0.09% | -0.01% | -0.39% | 105% | 99% |
| Class D | -0.02% | -0.15% | 0.01% | 105% | 101% |

The detailed results can be found in the accompanying excel file. By adapting the IBC tool, it shows that IBC can achieve good gains for ECM-4.0 considering that for AI coding, every 1% bit-saving needs about 22% encoding time increase compared with VTM-11.0ecm [2]. Thus, it is proposed to enable the modified IBC tool for the camera-captured sequences from classA to classE.

The result with only the encoder change to the EE2-3.2 software is summarized in Table 2. The detailed results can be found in the accompanying excel file JVET-Z0157-enc-change.xlsm.

Table 2. The summarized result of the EE2-3.2 software with only the encoder change and with IBC on compared with ECM-4.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.03% | -0.06% | 113% | 103% |
| Class A2 | -0.94% | -1.44% | -1.32% | 114% | 101% |
| Class B | -0.67% | -0.72% | -0.78% | 107% | 100% |
| Class C | -0.18% | -0.20% | -0.06% | 107% | 101% |
| Class E | -0.96% | -1.01% | -1.02% | 107% | 100% |
| Overall | -0.55% | -0.66% | -0.63% | 109% | 101% |
| Class D | -0.49% | -0.49% | -0.74% | 104% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.10% | 0.21% | 101% | 101% |
| Class A2 | -0.19% | -0.38% | -0.18% | 99% | 100% |
| Class B | -0.20% | -0.32% | -0.34% | 99% | 100% |
| Class C | 0.23% | -0.05% | 0.18% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.22% | -0.06% | 99% | 100% |
| Class D | 0.19% | -0.27% | -0.48% | 97% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.06% | -0.45% | 106% | 98% |
| Class C | -0.05% | 0.22% | -0.23% | 104% | 99% |
| Class E | -0.25% | -0.24% | -0.50% | 103% | 99% |
| Overall | -0.09% | -0.01% | -0.39% | 105% | 99% |
| Class D | -0.02% | -0.15% | 0.01% | 105% | 101% |

# Reference

[1] J. Xu,  N. Zhang, “EE2-3.2: IBC Reference Area Extension,” JVET-Z0153

[2] V. Seregin, J. Chen, F. Le Léannec, K. Zhang, “JVET AHG report: ECM software development (AHG6),” JVET-Z0006

# Patent rights declaration(s)

ByteDance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).