| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0164 |
| --- | --- |

| Title: | YCgCo-R: Request for new code points in CICP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Derek Buitenhuis Vimeo Inc. 555 West 18th Street New York, New York, 10011 United States Alexis Michael Tourapis Apple Inc. One Apple Park Way Cupertino, California, 95014 United States | Tel: Email: | derekb@vimeo.com atourapis@apple.com |
| Source: | Vimeo Inc, Apple Inc. |  |  |

_____________________________

# Abstract

In this contribution it is requested that additional code points are introduced in the CICP specification to better handle the YCgCo-R representation in applications that require both the luma and chroma information to be signalled using the same bit depth. Such representations were originaly presented in contributions JVET-T0111 and JVET-U0093, where coding gains for both lossless and lossy applications were documented.

# Introduction

During the 20th and 21st JVET meetings it was proposed that two new YCgCo-R representations should be included in the CICP specification. It was asserted that there may be an issue with many video coding specifications as well as encoder implementations that can only allow for the same bit depth to be used for both luma and chroma components. This tends to restrict the support of the YCgCo-R representation, which can allow improved coding performance for especially lossless applications, since it assumes that the chroma components should be assigned one more bit compared to the luma component. The proposed representations allowed for more efficient packing of the YCgCo-R data, essentially allowing for similar coding benefits as intended with the original YCgCo-R representation method.

In particular, the YCgCo-R representation is defined as a lossless transformation between the original RGB representation and YCgCo using a lifting scheme transformation method defined as follows:

Co = Round( R ) − Round( B ) + ( 1 << ( BitDepthC − 1 ) )

t = Round( B ) + ( ( Co − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

Cg = Round( G ) − t + ( 1 << ( BitDepthC − 1 ) )

Y = t + ( ( Cg − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

where R, G, and B are the red, green, and blue input colour component values, Co, Cg, and Y are the chroma orange-difference, chroma green-difference, and luma output colour component values, and BitDepthC corresponds to the output bit depth of the two chroma components. BitDepthC is required to be equal to the bit depth of the luma component, and consequently the red, green, and blue components, plus 1.

Unfortunately, when one cannot support different luma and chroma bit depths the actual transformation supported is as follows:

Y = Round( 0.5 * G + 0.25 * ( R + B ) )

Cg = Round( 0.5 * G − 0.25 * ( R + B ) ) + ( 1 << ( BitDepthC − 1 ) )

Co = Round( 0.5 * (R − B ) ) + ( 1 << ( BitDepthC − 1 ) )

In order for this transformation to be lossless, BitDepthC and, essentially, the bit depth of all colour components, need to be higher of that of the original input R, G, and B signals. This, in effect, results in the luma component being MSB aligned in the resulting representation. Experiments provided in JVET-T0111 and JVET-U0093 have shown that this particular representation is not as efficient.

# Proposed improvement to the YCgCo-R representation

To avoid the above stated problem, JVET-T0111 and JVET-U0093 proposed adding additional YCgCo based code points, i.e. new matrix coefficients entries, which would enforce an LSB alignment of the luma values in the luma component according to the YCgCo transformation process. In particular, it was suggested introducing at least one more matrix coefficients code point in the CICP specification, which would permit the same transformation as the current YCgCo matrix coefficients entry (value 8) specified for when BitDepthC != BitDepthY, i.e.:

Co = Round( R ) − Round( B ) + ( 1 << ( BitDepthC − 1 ) )

t = Round( B ) + ( ( Co − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

Cg = Round( G ) − t + ( 1 << ( BitDepthC − 1 ) )

Y = t + ( ( Cg − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

with, however, the inverse process, being performed as follows:

maxValue = ( 1 << (BitDepthY – bitOffset) ) – 1

t = Y − ( ( Cg − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )

G = Clip3( 0, maxValue, t + ( Cg − ( 1 << ( BitDepthC − 1 ) ) ) )

B = Clip3( 0, maxValue, t − ( ( Co − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )

R = Clip3(0, maxValue, B + ( Co − ( 1 << ( BitDepthC − 1 ) ) ) )

where the value of bitOffset is set equal to 2 for the first code point, and 1 for the second.

The intent of the value of bitOffset being set equal to 2 is to allow for an "even" bit depth RGB representation, e.g. 8 bits, to be efficiently represented in a 10 bit input representation to the video encoder, or in general for applications where both the original RGB and intended input to the encoder representations are of odd or even precision.

The second code point (bitOffset set to 1) is intended for applications where the RGB and input representations are of different bit depth parity, i.e. even vs odd representations. However, since odd bit depth representations are relatively uncommon, this second code value may be omitted.

It was also highlighted that, with these representations, the bit depth of the RGB components is also reduced according to the value of bitOffset. In particular, the bit depth of the red, green, and blue components is equal to a new variable bitDepth, which is computed as follows:

bitDepth = BitDepthY – bitOffset

Consequently, a video range representation RGB signal would be generated as:

R = Clip1Y( ( 1 << ( bitDepth − 8 ) ) * ( 219 * E′R + 16 ) )

G = Clip1Y( ( 1 << ( bitDepth − 8 ) ) * ( 219 * E′G + 16 ) )

B = Clip1Y( ( 1 << ( bitDepth − 8 ) ) * ( 219 * E′B + 16 ) )

while a full range representation RGB signal would be generated as:

R = Clip1Y( ( ( 1 << bitDepth) − 1 ) * E′R )

G = Clip1Y( ( ( 1 << bitDepth) − 1 ) * E′G )

B = Clip1Y( ( ( 1 << bitDepth) − 1 ) * E′B )

# Experiments from JVET-T0111 and JVET-U0093

In this section we again repeat the experiments presented in JVET-T0111 and JVET-U0093.

Table 1 through Table 4 include the results using the VTM software that were presented in JVET-T0111.

|  | RGB |  |  | YCoCg-R |  |
| --- | --- | --- | --- | --- | --- |
|  | no DualItree + ACT 8 bit coded | DualItree 8-bit coded | no DualItree 8 bit coded | MSB aligned 10-bit coded | LSB aligned 10-bit coded |
| Mean | 100.00% | 100.98% | 101.52% | 123.27% | 97.20% |
| StdDev | 0.00% | 0.85% | 0.81% | 3.64% | 2.86% |
| Best | 100.00% | 99.63% | 100.04% | 111.51% | 87.58% |
| Worst | 100.00% | 103.69% | 104.18% | 131.11% | 103.19% |

Table 1: Lossless coding performance for subset1 using the VTM

|  | RGB |  |  | YCoCg-R |  |
| --- | --- | --- | --- | --- | --- |
|  | no DualItree + ACT 8 bit coded | DualItree 8-bit coded | no DualItree 8 bit coded | MSB aligned 10-bit coded | LSB aligned 10-bit coded |
| Mean | 100.00% | 100.94% | 101.49% | 123.26% | 97.19% |
| StdDev | 0.00% | 0.88% | 0.86% | 2.95% | 2.13% |
| Best | 100.00% | 99.08% | 99.93% | 117.01% | 91.07% |
| Worst | 100.00% | 103.17% | 103.70% | 131.53% | 101.11% |

Table 2: Lossless coding performance for subset2 using the VTM

|  | no DualItree + ACT 8 bit coded | DualItree 8-bit coded | no DualItree 8 bit coded | MSB aligned 10-bit coded | LSB aligned 10-bit coded |
| --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 101.12% | 101.40% | 119.75% | 95.69% |
| StdDev | 0.00% | 1.02% | 0.94% | 6.27% | 4.17% |
| Best | 100.00% | 95.18% | 96.22% | 90.85% | 74.00% |
| Worst | 100.00% | 108.66% | 105.58% | 153.45% | 115.25% |

Table 3: Lossless coding performance for subset3 using the VTM

|  | no DualItree + ACT 8 bit coded | DualItree 8-bit coded | no DualItree 8 bit coded | MSB aligned 10-bit coded | LSB aligned 10-bit coded |
| --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 101.23% | 101.53% | 119.82% | 95.67% |
| StdDev | 0.00% | 1.14% | 1.00% | 6.23% | 4.00% |
| Best | 100.00% | 92.66% | 91.67% | 84.26% | 67.66% |
| Worst | 100.00% | 111.75% | 105.72% | 146.40% | 114.84% |

Table 4: Lossless coding performance for subset4 using the VTM

Table 5 through Table 8 showcase the performance of the proposed representations using the JM reference SW of MPEG-4 AVC, while Table 9 through Table 12 provide additional information using the x264 encoder implementation of the same standard. It should be noted that x264 is not currently capable of encoding data with different bit depths between luma and chroma or 9 bit representations.

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R LSB 9-bit | YCoCg-R 9-bit Co/Cg, 8-bit Y | YCoCg-R MSB 9-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 91.64% | 91.94% | 91.97% | 100.33% | 119.31% |
| StdDev | 0.00% | 9.24% | 9.32% | 9.35% | 9.88% | 13.23% |
| Best | 100.00% | 73.94% | 74.46% | 74.63% | 67.44% | 81.13% |
| Worst | 100.00% | 109.35% | 109.96% | 110.01% | 124.23% | 145.81% |

Table 5: Lossless coding performance for subset1 using the JM

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R LSB 9-bit | YCoCg-R 9-bit Co/Cg, 8-bit Y | YCoCg-R MSB 9-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 90.99% | 90.92% | 90.96% | 100.85% | 119.84% |
| StdDev | 0.00% | 10.64% | 10.58% | 10.46% | 10.06% | 14.96% |
| Best | 100.00% | 64.14% | 64.14% | 64.14% | 73.06% | 79.69% |
| Worst | 100.00% | 114.34% | 114.85% | 111.57% | 120.80% | 152.42% |

Table 6: Lossless coding performance for subset2 using the JM

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R LSB 9-bit | YCoCg-R 9-bit Co/Cg, 8-bit Y | YCoCg-R MSB 9-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 88.16% | 88.35% | 88.41% | 98.36% | 120.42% |
| StdDev | 0.00% | 11.01% | 11.00% | 11.00% | 10.25% | 15.99% |
| Best | 100.00% | 32.11% | 32.11% | 32.10% | 47.66% | 69.36% |
| Worst | 100.00% | 135.79% | 136.98% | 137.20% | 150.46% | 208.96% |

Table 7: Lossless coding performance for subset3 using the JM

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R LSB 9-bit | YCoCg-R 9-bit Co/Cg, 8-bit Y | YCoCg-R MSB 9-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 87.56% | 87.75% | 87.79% | 97.41% | 120.44% |
| StdDev | 0.00% | 10.13% | 10.14% | 10.14% | 9.46% | 14.69% |
| Best | 100.00% | 40.98% | 41.61% | 41.81% | 56.74% | 73.10% |
| Worst | 100.00% | 138.54% | 138.55% | 138.53% | 147.33% | 177.08% |

Table 8: Lossless coding performance for subset4 using the JM

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- |
| Mean | 100.00% | 85.73% | 116.30% |
| StdDev | 0.00% | 8.45% | 10.98% |
| Best | 100.00% | 58.94% | 81.47% |
| Worst | 100.00% | 103.35% | 141.55% |

Table 9: Lossless coding performance for subset1 using x264

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- |
| Mean | 100.00% | 84.84% | 115.24% |
| StdDev | 0.00% | 8.21% | 11.06% |
| Best | 100.00% | 55.34% | 75.42% |
| Worst | 100.00% | 100.59% | 136.37% |

Table 10: Lossless coding performance for subset2 using x264

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- |
| Mean | 100.00% | 85.10% | 114.77% |
| StdDev | 0.00% | 8.44% | 10.79% |
| Best | 100.00% | 33.78% | 51.98% |
| Worst | 100.00% | 115.60% | 154.56% |

Table 11: Lossless coding performance for subset3 using x264

|  | RGB (GBR) | YCoCg-R LSB 10-bit | YCoCg-R MSB 10-bit |
| --- | --- | --- | --- |
| Mean | 100.00% | 84.84% | 114.52% |
| StdDev | 0.00% | 7.66% | 9.89% |
| Best | 100.00% | 34.26% | 51.93% |
| Worst | 100.00% | 120.34% | 156.60% |

Table 12: Lossless coding performance for subset4 using x264

It can be seen from the JM results that the proposed representations allow for similar coding performance as that of the original YCgCo-R representation. Coding benefits seem to be achieved almost always compard to an MSB aligned representation.

Additional results are presented in Table 13 through Table 24 for the JPEG 2K, JPEG XR, and JPEG XL standards. Even though the first two standards support an internal residual color transform, and therefore, on average, there is no gain in using the proposed transform, we can see some individual cases where some coding gain is observed. In the JPEG XL case the transform is already supported in two modes, and the results showcase the coding advantage this transform can provide.

|  | RGB (with RCT) | YCoCg-R LSB 10-bit Coded | YCoCg-R LSB 9-bit Coded | YCoCg-R MSB 9-bit Coded | YCoCg-R MSB 10-bit Coded |
| --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 100.42% | 100.41% | 109.56% | 137.03% |
| StdDev | 0.00% | 0.92% | 0.92% | 1.67% | 5.60% |
| Best | 100.00% | 98.75% | 98.74% | 105.74% | 126.35% |
| Worst | 100.00% | 103.17% | 103.17% | 113.48% | 150.39% |

Table 13: Lossless coding performance for subset1 using JPEG 2K

|  | RGB (with RCT) | YCoCg-R LSB 10-bit Coded | YCoCg-R LSB 9-bit Coded | YCoCg-R MSB 9-bit Coded | YCoCg-R MSB 10-bit Coded |
| --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 100.92% | 100.91% | 110.03% | 137.16% |
| StdDev | 0.00% | 1.63% | 1.63% | 2.35% | 5.67% |
| Best | 100.00% | 99.44% | 99.43% | 106.34% | 124.99% |
| Worst | 100.00% | 111.17% | 111.16% | 122.06% | 154.73% |

Table 14: Lossless coding performance for subset2 using JPEG 2K

|  | RGB (with RCT) | YCoCg-R LSB 10-bit Coded | YCoCg-R LSB 9-bit Coded | YCoCg-R MSB 9-bit Coded | YCoCg-R MSB 10-bit Coded |
| --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 100.60% | 100.59% | 110.52% | 140.14% |
| StdDev | 0.00% | 1.14% | 1.14% | 2.55% | 7.29% |
| Best | 100.00% | 95.77% | 95.76% | 105.01% | 123.95% |
| Worst | 100.00% | 112.53% | 112.53% | 129.40% | 182.10% |

Table 15: Lossless coding performance for subset3 using JPEG 2K

|  | RGB (with RCT) | YCoCg-R LSB 10-bit Coded | YCoCg-R LSB 9-bit Coded | YCoCg-R MSB 9-bit Coded | YCoCg-R MSB 10-bit Coded |
| --- | --- | --- | --- | --- | --- |
| Mean | 100.00% | 100.52% | 100.51% | 110.39% | 140.02% |
| StdDev | 0.00% | 1.11% | 1.11% | 2.42% | 7.39% |
| Best | 100.00% | 94.64% | 94.64% | 100.83% | 118.68% |
| Worst | 100.00% | 109.19% | 109.17% | 127.67% | 171.31% |

Table 16: Lossless coding performance for subset4 using JPEG 2K

|  | RGB (with RCT) | YCoCg-R LSB 10-bit Coded | YCoCg-R MSB 10-bit Coded |
| --- | --- | --- | --- |
| Mean | 100.00% | 100.04% | 131.12% |
| StdDev | 0.00% | 0.22% | 2.94% |
| Best | 100.00% | 99.21% | 125.96% |
| Worst | 100.00% | 100.73% | 138.18% |

Table 17: Lossless coding performance for subset1 using JPEG XR

|  | RGB (with RCT) | YCoCg-R LSB 10-bit Coded | YCoCg-R MSB 10-bit Coded |
| --- | --- | --- | --- |
| Mean | 100.00% | 100.05% | 130.90% |
| StdDev | 0.00% | 0.29% | 2.96% |
| Best | 100.00% | 99.29% | 123.48% |
| Worst | 100.00% | 101.43% | 138.26% |

Table 18: Lossless coding performance for subset2 using JPEG XR

|  | RGB (with RCT) | YCoCg-R LSB 10-bit Coded | YCoCg-R MSB 10-bit Coded |
| --- | --- | --- | --- |
| Mean | 100.00% | 100.01% | 132.47% |
| StdDev | 0.00% | 1.04% | 3.83% |
| Best | 100.00% | 82.31% | 115.52% |
| Worst | 100.00% | 113.95% | 180.36% |

Table 19: Lossless coding performance for subset3 using JPEG XR

|  | RGB (with RCT) | YCoCg-R LSB 10-bit Coded | YCoCg-R MSB 10-bit Coded |
| --- | --- | --- | --- |
| Mean | 100.00% | 99.99% | 132.32% |
| StdDev | 0.00% | 0.94% | 3.61% |
| Best | 100.00% | 86.48% | 117.32% |
| Worst | 100.00% | 106.34% | 152.11% |

Table 20: Lossless coding performance for subset4 using JPEG XR

|  | RGB (no RCT) | YCoCg-R whole image | Mixed RGB/YCoCg-R Regions |
| --- | --- | --- | --- |
| Mean | 100.00% | 85.54% | 83.89% |
| StdDev | 0.00% | 9.15% | 8.19% |
| Best | 100.00% | 56.42% | 56.35% |
| Worst | 100.00% | 106.43% | 97.82% |

Table 21: Lossless coding performance for subset1 using JPEG XL

|  | RGB (no RCT) | YCoCg-R whole image | Mixed RGB/YCoCg-R Regions |
| --- | --- | --- | --- |
| Mean | 100.00% | 84.42% | 82.80% |
| StdDev | 0.00% | 8.69% | 8.57% |
| Best | 100.00% | 54.60% | 54.32% |
| Worst | 100.00% | 101.28% | 99.36% |

Table 22: Lossless coding performance for subset2 using JPEG XL

|  | RGB (no RCT) | YCoCg-R whole image | Mixed RGB/YCoCg-R Regions |
| --- | --- | --- | --- |
| Mean | 100.00% | 84.70% | 83.14% |
| StdDev | 0.00% | 8.63% | 8.33% |
| Best | 100.00% | 53.49% | 52.37% |
| Worst | 100.00% | 126.16% | 100.08% |

Table 23: Lossless coding performance for subset3 using JPEG XL

|  | RGB (no RCT) | YCoCg-R whole image | Mixed RGB/YCoCg-R Regions |
| --- | --- | --- | --- |
| Mean | 100.00% | 84.18% | 82.66% |
| StdDev | 0.00% | 8.41% | 8.10% |
| Best | 100.00% | 54.14% | 52.67% |
| Worst | 100.00% | 131.56% | 100.00% |

Table 24: Lossless coding performance for subset4 using JPEG XL

Finally, we experimented with lossy performance of this transform using HEVC and using a single image from the FoodMarket2 sequence. It can be observed from Figure 1 through Figure 3 that some gain can be seen in the higher bitrates using this transform vs. using the lossy YCoCg transform.

Figure 1. RD performance using B-PSNR for FoodMarket2

Figure 2. RD performance using G-PSNR for FoodMarket2

Figure 3. RD performance using R-PSNR for FoodMarket2

# Proposed Change to CICP Text

## Matrix coefficients

Type: Unsigned integer, enumeration
Range: 0–255, plus associated flag

MatrixCoefficients describes the matrix coefficients used in deriving luma and chroma signals from the green, blue and red or X, Y and Z primaries, as specified in Table 4 and equations 11 to 77.

A flag, VideoFullRangeFlag, may be supplied with this code point.

VideoFullRangeFlag specifies the scaling and offset values applied in association with the MatrixCoefficients. When not present or not specified, the value 0 for VideoFullRangeFlag would ordinarily be inferred as the default value for video imagery.

An 8-bit field should be adequate for representation of the MatrixCoefficients code point.

NOTE 1 – Certain values of MatrixCoefficients may be disallowed depending on the application and the characteristics and format of the signal, e.g., regarding combinations of the chroma format sampling structure and the values of BitDepthY and BitDepthC.

The interpretation of MatrixCoefficients is specified by equations 11 to 77. ER, EG and EB are defined as "linear-domain" real-valued signals based on the indicated colour primaries (see ‎8.1) before applying the transfer characteristics (see ‎8.2).

NOTE 2 – For purposes of the YZX representation when MatrixCoefficients is equal to 0, the symbols R, G and B are substituted for X, Y and Z, respectively, in the following descriptions of equations 11 to 13, equations 14 to 16, equations 20 to 22, and equations 26 to 28.

Nominal peak white is specified as having ER equal to 1, EG equal to 1 and EB equal to 1.

Nominal black is specified as having ER equal to 0, EG equal to 0 and EB equal to 0.

The application of the transfer characteristics function is denoted by ( x )′ for an argument x.

–	If MatrixCoefficients is not equal to 14, the signals E′R, E′G and E′B are determined by application of the transfer characteristics function as follows:

E′R = ( ER )′			(11)

E′G = ( EG )′			(12)

E′B = ( EB )′			(13)

In this case, the range of E′R, E′G and E′B is specified as follows:

–	If TransferCharacteristics is equal to 11 or 12, or TransferCharacteristics is equal to 13 and MatrixCoefficients is not equal to 0, E′R, E′G and E′B are real numbers with values that have a larger range than the range of 0 to 1, and their range is not specified in this Specification.

–	Otherwise, E′R, E′G and E′B are real numbers with values in the range of 0 to 1.

–	Otherwise (MatrixCoefficients is equal to 14), the "linear-domain" real-valued signals EL, EM and ES are determined as follows:

EL = ( 1688 * ER + 2146 * EG + 262 * EB ) ÷ 4096		(14)

EM = ( 683 * ER + 2951 * EG + 462 * EB ) ÷ 4096		(15)

ES = ( 99 * ER + 309 * EG + 3688 * EB ) ÷ 4096		(16)

In this case, the signals E′L, E′M and E′S are determined by application of the transfer characteristics function as follows:

E′L = ( EL )′			(17)

E′M = ( EM )′			(18)

E′S = ( ES )′			(19)

The interpretation of MatrixCoefficients is specified as follows.

–	If VideoFullRangeFlag is equal to 0, the following applies:

–	If MatrixCoefficients is equal to 0 or 8, equations 20 to 22 apply:

R = Clip1Y( ( 1 << ( BitDepthY − 8 ) ) * ( 219 * E′R + 16 ) )	(20)

G = Clip1Y( ( 1 << ( BitDepthY − 8 ) ) * ( 219 * E′G + 16 ) )	(21)

B = Clip1Y( ( 1 << ( BitDepthY − 8 ) ) * ( 219 * E′B + 16 ) )	(22)

–	Otherwise, if MatrixCoefficients is equal to 15 or 16, equations 20 to 22 apply:

bitOffset = MatrixCoefficients == 15 ? 2 : 1		(20)

bitDepth = BitDepthY – bitOffset		(20)

R = Clip1Y( ( 1 << ( bitDepth − 8 ) ) * ( 219 * E′R + 16 ) )	(20)

G = Clip1Y( ( 1 << ( bitDepth − 8 ) ) * ( 219 * E′G + 16 ) )	(21)

B = Clip1Y( ( 1 << ( bitDepth − 8 ) ) * ( 219 * E′B + 16 ) )	(22)

–	Otherwise, if MatrixCoefficients is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, 13 or 14, equations 23 to 25 apply:

Y = Clip1Y( Round( ( 1 << ( BitDepthY − 8 ) ) * ( 219 * E′Y + 16 ) ) )	(23)

Cb = Clip1C( Round( ( 1 << ( BitDepthC − 8 ) ) * ( 224 * E′PB + 128 ) ) )	(24)

Cr = Clip1C( Round( ( 1 << ( BitDepthC − 8 ) ) * ( 224 * E′PR + 128 ) ) )	(25)

–	Otherwise, if MatrixCoefficients is equal to 2, the interpretation of the MatrixCoefficients code point is unknown or is determined by the application.

–	Otherwise (MatrixCoefficients is not equal to 0, 1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13 or 14), the interpretation of the MatrixCoefficients code point is reserved for future definition by ITU-T | ISO/IEC.

–	Otherwise (VideoFullRangeFlag is equal to 1), the following equations apply:

–	If MatrixCoefficients is equal to 0 or 8, equations 26 to 28 apply:

R = Clip1Y( ( ( 1 << BitDepthY ) − 1 ) * E′R )	(26)

G = Clip1Y( ( ( 1 << BitDepthY ) − 1 ) * E′G )	(27)

B = Clip1Y( ( ( 1 << BitDepthY ) − 1 ) * E′B )	(28)

–	Otherwise, if MatrixCoefficients is equal to 15 or 16, equations 20 to 22 apply:

bitOffset = MatrixCoefficients == 15 ? 2 : 1		(20)

bitDepth = BitDepthY – bitOffset		(20)

R = Clip1Y( ( ( 1 << bitDepth ) − 1 ) * E′R )	(26)

G = Clip1Y( ( ( 1 << bitDepth ) − 1 ) * E′G )	(27)

B = Clip1Y( ( ( 1 << bitDepth ) − 1 ) * E′B )	(28)

–	Otherwise, if MatrixCoefficients is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12, 13 or 14, equations 29 to 31 apply:

Y = Clip1Y( Round( ( ( 1 << BitDepthY ) − 1 ) * E′Y ) )	(29)

Cb = Clip1C( Round( ( ( 1 << BitDepthC ) − 1 )  * E′PB ) + ( 1 << ( BitDepthC − 1 ) ) )	(30)

Cr = Clip1C( Round( ( ( 1 << BitDepthC ) − 1 )  * E′PR ) + ( 1 << ( BitDepthC − 1 ) ) )	(31)

–	Otherwise, if MatrixCoefficients is equal to 2, the interpretation of the MatrixCoefficients code point is unknown or is determined by the application.

–	Otherwise (MatrixCoefficients is not equal to 0, 1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15 or 16), the interpretation of the MatrixCoefficients code point is reserved for future definition by ITU-T | ISO/IEC.

When MatrixCoefficients is equal to 1, 4, 5, 6, 7, 9, 10, 11, 12 or 13, the constants KB and KR are specified as follows:

–	If MatrixCoefficients is not equal to 12 or 13, the constants KB and KR are specified in Table 4.

–	Otherwise (MatrixCoefficients is equal to 12 or 13), the constants KR and KB are computed as follows, using the chromaticity coordinates (xR, yR), (xG, yG), (xB, yB) and (xW, yW) specified in Table 2 for the ColourPrimaries code point for the red, green, blue and white colour primaries, respectively:

KR = 	(32)

KB = 	(33)

where the values of zR, zG, zB and zW, are given by:

zR = 1 − ( xR + yR )		(34)

zG = 1 − ( xG + yG )		(35)

zB = 1 − ( xB + yB )		(36)

zW = 1 − ( xW + yW )		(37)

The variables E′Y, E′PB and E′PR (for MatrixCoefficients not equal to 0, 8, 15, or 16) or Y, Cb and Cr (for MatrixCoefficients equal to 0 or 8) are specified as follows.

–	If MatrixCoefficients is not equal to 0, 8, 10, 11, 13, 14, 15 or 16, equations 38 to 40 apply:

E′Y = KR * E′R + ( 1 − KR − KB ) * E′G + KB * E′B		(38)

E′PB = 0.5 * ( E′B − E′Y ) ÷ ( 1 − KB )		(39)

E′PR = 0.5 * ( E′R − E′Y ) ÷ ( 1 − KR )		(40)

NOTE 3 – E′Y is a real number with the value 0 associated with nominal black and the value 1 associated with nominal white. E′PB and E′PR are real numbers with the value 0 associated with both nominal black and nominal white. When TransferCharacteristics is not equal to 11 or 12, E′Y is a real number with values in the range of 0 to 1. When TransferCharacteristics is not equal to 11 or 12, E′PB and E′PR are real numbers with values in the range of −0.5 to 0.5. When TransferCharacteristics is equal to 11 (IEC 61966-2-4) or 12 (Rec. ITU-R BT.1361-0 extended colour gamut system), E′Y, E′PB and E′PR are real numbers with a larger range not specified in this Specification.

–	Otherwise, if MatrixCoefficients is equal to 0, equations 41 to 43 apply:

Y   = Round( G )		(41)

Cb = Round( B )		(42)

Cr = Round( R )		(43)

–	Otherwise, if MatrixCoefficients is equal to 8, the following applies:

–	If BitDepthC is equal to BitDepthY, equations 44 to 46 apply:

Y   = Round( 0.5 * G + 0.25 * ( R + B ) )		(44)

Cb = Round( 0.5 * G − 0.25 * ( R + B ) ) + ( 1 << ( BitDepthC − 1 ) )	(45)

Cr = Round( 0.5 * (R − B ) ) + ( 1 << ( BitDepthC − 1 ) )	(46)

NOTE 4 – For purposes of the YCgCo nomenclature used in Table 4, Cb and Cr of equations 45 and 46 may be referred to as Cg and Co, respectively. The inverse conversion for equations 44 to 46 should be computed as:

t   = Y − ( Cb − ( 1 << ( BitDepthC − 1 ) ) )	(47)

G = Clip1Y( Y + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(48)

B = Clip1Y( t − ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(49)

R = Clip1Y( t + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(50)

–	Otherwise (BitDepthC is not equal to BitDepthY), equations 51 to 54 apply:

Cr = Round( R ) − Round( B ) + ( 1 << ( BitDepthC − 1 ) )	(51)

t = Round( B ) + ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(52)

Cb = Round( G ) − t + ( 1 << ( BitDepthC − 1 ) )	(53)

Y = t + ( ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(54)

NOTE 5 – For purposes of the YCgCo nomenclature used in Table 4, Cb and Cr of equations 53 and 51 may be referred to as Cg and Co, respectively. The inverse conversion for equations 51 to 54 should be computed as:

t   = Y − ( ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(55)

G = Clip1Y( t + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(56)

B = Clip1Y( t − ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )	(57)

R = Clip1Y( B + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(58)

–	Otherwise, if MatrixCoefficients is equal to 15 or 16, the following applies:

Cr = Round( R ) − Round( B ) + ( 1 << ( BitDepthC − 1 ) )	(51)

t = Round( B ) + ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(52)

Cb = Round( G ) − t + ( 1 << ( BitDepthC − 1 ) )	(53)

Y = t + ( ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(54)

NOTE 5 – For purposes of the YCgCo nomenclature used in Table 4, Cb and Cr of equations 53 and 51 may be referred to as Cg and Co, respectively. The inverse conversion for equations 51 to 54 should be computed as:

bitOffset = MatrixCoefficients == 15 ? 2 : 1		(XX)

maxValue = ( 1 << (BitDepthY – bitOffset) ) – 1 (XX)

t = Y − ( ( Cg − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) (XX)

G = Clip3( 0, maxValue, t + ( Cg − ( 1 << ( BitDepthC − 1 ) ) ) )  (XX)

B = Clip3( 0, maxValue, t − ( ( Co − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )  (XX)

R = Clip3(0, maxValue, B + ( Co − ( 1 << ( BitDepthC − 1 ) ) ) )  (XX)

–	Otherwise, if MatrixCoefficients is equal to 10 or 13, the signal E′Y is determined by application of the transfer characteristics function as follows and equations 61 to 68 apply for specification of the signals E′PB and E′PR:

EY = KR * ER + ( 1 − KR − KB ) * EG + KB * EB		(59)

E′Y = ( EY )′			(60)

NOTE 6 – In this case, EY is defined from the "linear-domain" signals for ER, EG and EB, prior to application of the transfer characteristics function, which is then applied to produce the signal E′Y. EY and E′Y are real values with the value 0 associated with nominal black and the value 1 associated with nominal white.

E′PB = ( E′B − E′Y ) ÷ ( 2 * NB )	for −NB  <=  E′B − E′Y  <=  0	(61)

E′PB = ( E′B − E′Y ) ÷ ( 2 * PB )	for 0  <  E′B − E′Y  <=  PB	(62)

E′PR = ( E′R − E′Y ) ÷ ( 2 * NR )	for −NR  <=  E′R − E′Y  <=  0	(63)

E′PR = ( E′R − E′Y ) ÷ ( 2 * PR )	for 0  <  E′R − E′Y  <=  PR	(64)

where the constants NB, PB, NR and PR are determined by application of the transfer characteristics function to expressions involving the constants KB and KR as follows:

NB = ( 1 − KB )′			(65)

PB = 1 − ( KB )′ 			(66)

NR = ( 1 − KR )′		(67)

PR = 1 − ( KR )′			(68)

–	Otherwise, if MatrixCoefficients is equal to 11, equations 69 to 71 apply:

E′Y = E′G			(69)

E′PB = ( 0.986566 * E′B − E′Y ) ÷ 2.0	 	(70)

E′PR = ( E′R − 0.991902 * E′Y ) ÷ 2.0		(71)

NOTE 7 – In this case, for purposes of the Y′D′ZD′X nomenclature used in Table 4, E′PB may be referred to as D′Z and E′PR may be referred to as D′X.

–	Otherwise (MatrixCoefficients is equal to 14), the following applies:

–	If TransferCharacteristics is not equal to 18, equations 72 to 74 apply:

E′Y = 0.5 * ( E′L + E′M )		(72)

E′PB = ( 6610 * E′L − 13613 * E′M + 7003 * E′S ) ÷ 4096	(73)

E′PR = ( 17933 * E′L − 17390 * E′M − 543 * E′S ) ÷ 4096	(74)

–	If TransferCharacteristics is not equal to 18, equations 75 to 77 apply:

E′Y = 0.5 * ( E′L + E′M )		(75)

E′PB = ( 3625 * E′L − 7465 * E′M + 3840 * E′S ) ÷ 4096	(76)

E′PR = ( 9500 * E′L − 9212 * E′M − 288 * E′S ) ÷ 4096	(77)

In these cases, for purposes of the ICTCP nomenclature used in Table 4, E′Y, E′PB and E′PR of equations 72, 73 and 74 or equations 75, 76, and 77 may be referred to as I, CT and CP, respectively. Equations 72, 73, and 74 were designed specifically for use with TransferCharacteristics equal to 16 (PQ), and equations 75, 76, and 77 were designed specifically for use with TransferCharacteristics equal to 18 (HLG).

| Table 4 – Interpretation of matrix coefficients (MatrixCoefficients) value |  |  |
| --- | --- | --- |
| Value | Matrix coefficients | Informative remarks |
| 0 | Identity | The identity matrix. Typically used for GBR (often referred to as RGB); however, may also be used for YZX (often referred to as XYZ); IEC 61966-2-1 sRGB SMPTE ST 428-1 (2019) See equations 41 to 43 |
| 1 | KR = 0.2126; KB = 0.0722 | Rec. ITU-R BT.709-6 Rec. ITU-R BT.1361-0 conventional colour gamut system and extended colour gamut system (historical) IEC 61966-2-4 xvYCC709 SMPTE RP 177 (1993) Annex B See equations 38 to 40 |
| 2 | Unspecified | Image characteristics are unknown or are determined by the application |
| 3 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 4 | KR = 0.30; KB = 0.11 | United States Federal Communications Commission (2003) Title 47 Code of Federal Regulations 73.682 (a) (20) See equations 38 to 40 |
| 5 | KR = 0.299; KB = 0.114 | Rec. ITU-R BT.470-6 System B, G (historical) Rec. ITU-R BT.601-7 625 Rec. ITU-R BT.1358-0 625 (historical) Rec. ITU-R BT.1700-0 625 PAL and 625 SECAM IEC 61966-2-1 sYCC IEC 61966-2-4 xvYCC601 (functionally the same as the value 6) See equations 38 to 40 |
| 6 | KR = 0.299; KB = 0.114 | Rec. ITU-R BT.601-7 525 Rec. ITU-R BT.1358-1 525 or 625 (historical) Rec. ITU-R BT.1700-0 NTSC SMPTE ST 170 (2004) (functionally the same as the value 5) See equations 38 to 40 |
| 7 | KR = 0.212; KB = 0.087 | SMPTE ST 240 (1999) See equations 38 to 40 |
| 8 | YCgCo | See equations 44 to 58 |
| 9 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020-2 (non-constant luminance) Rec. ITU-R BT.2100-2 Y′CbCr See equations 38 to 40 |
| 10 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020-2 (constant luminance) See equations 59 to 68 |
| 11 | Y′D′ZD′X | SMPTE ST 2085 (2015) See equations 69 to 71 |
| 12 | See equations 32 to 37 | Chromaticity-derived non-constant luminance system See equations 38 to 40 |
| 13 | See equations 32 to 37 | Chromaticity-derived constant luminance system See equations 59 to 68 |
| 14 | ICTCP | Rec. ITU-R BT.2100-2 ICTCP See equations 72 to 74 for TransferCharacteristics value 16 (PQ) See equations 75 to 77 for TransferCharacteristics value 18 (HLG) |
| 15 | YCgCo-Ro | See equations XX to YY |
| 16 | YCgCo-Re | See equations XX to YY |
| 17–255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

NOTE 8 – In a previous version of this Specification, the IEC 61966-2-1 sYCC representation was identified as corresponding to MatrixCoefficients equal to 1. Closer study later determined that this representation should correspond to MatrixCoefficients equal to 5 instead (which is functionally the same as the value 6). This Specification was therefore revised to correct the error.

# Conclusion

It is proposed that one or more new matrix coefficients values, to indicate an LSB aligned YCoCg-R representation, are introduced in the CICP specification and consequently any video specification that utilize such code points.

# Patent rights declaration(s)

Vimeo may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).