| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0165-v3 |
| --- | --- |

| Title: | EE2-3.6: Combination tests of EE2-3.2+EE2-3.3+EE2-3.4+EE2-3.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1, Jizheng Xu2, Kai Zhang2, Li Zhang2 (ByteDance Inc.) 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA A. Robert, K. Naser, T. Poirier, Y. Chen, F. Galpin (InterDigital) D. Ruiz Coll, A. Filippov,V. Rufitskiy (Ofinno) | Email: | zhangna.cynthia@bytedance.com xujizheng@bytedance.com antoine.robert@interdigital.com karam.naser@interdigital.com duizcoll@ofinno.com afilippov@ofinno.com vrufitskiy@ofinno.com |
| Source: | ByteDance Inc. InterDigital Ofinno, LLC |  |  |

_____________________________

# Abstract

This document reports results for EE2-3.6 on combination tests of EE2-3.2+EE2-3.3+EE2-3.4+EE2-3.5.

On top of the ECM-4.0, simulation results of the combination tests are reported as below:

EE2-3.6a (i.e. Test 3.2+Test3.3)

AI: 	Class F -4.13%,107%,96%; 	Class TGM -9.06%,104%,83%

RA: 	Class F -2.56%,103%,99%; 	Class TGM -6.17%,101%,95%

LDB: 	Class F -1.59%,101%,100%; 	Class TGM -5.00%,100%,93%

EE2-3.6b (i.e. Test 3.2+Test3.4)

AI: 	Class F -4.51%,108%,103%; 	Class TGM -8.53%,107%,103%

RA: 	Class F -2.83%,105%,103%; 	Class TGM -5.60%,106%,103%

LDB: 	Class F -1.92%%,105%,103%; 	Class TGM -4.45%,105%,103%

EE2-3.6c (i.e. Test 3.2+Test3.5c)

AI: 	Class F -3.92%,109%,99%; 	Class TGM -7.93%,102%,97%

RA: 	Class F -2.46%,105%,100%; 	Class TGM -5.29%,102%,101%

LDB: 	Class F -1.92%%,105%,103%; 	Class TGM -4.45%,105%,103%

EE2-3.6e (i.e. Test 3.3+Test3.4)

AI: 	Class F -0.69%,106%,103%; 	Class TGM -1.68%,106%,103%

RA: 	Class F -0.62%,104%,101%; 	Class TGM -1.50%,105%,103%

LDB: 	Class F -0.76%, 104%,101%; 	Class TGM -1.21%, 103%,102%

EE2-3.6f (i.e. Test 3.3+Test3.5c)

AI: 	Class F -0.42%,104%,101%; 	Class TGM -1.22%,102%,101%;

RA: 	Class F -0.37%,104%,100%; 	Class TGM -1.24%,103%,101%

LDB: 	Class F -0.28%, 106%,99%; 	Class TGM -0.99%, 100%,98%

EE2-3.6g (i.e. Test 3.4+Test3.5c)

AI: 	Class F -0.74%,104%,102%; 	Class TGM -0.56%,103%,102%;

RA: 	Class F -0.42%,104%,101%; 	Class TGM -0.46%,104%,102%

LDB: 	Class F -0.34%, 103%,101%; 	Class TGM -0.33%, 102%,101%

EE2-3.6h (i.e. Test 3.2+Test3.3+Test3.4)

AI: 	Class F -4.63%,116%,98%; 	Class TGM -9.60%,110%,92%

RA: 	Class F -3.01%, 108%,98%; 	Class TGM -6.62%, 103%,96%

LDB: 	Class F -2.08%%, 106%,98%; 	Class TGM -5.35%, 106%,97%

EE2-3.6i (i.e. Test 3.2+Test3.3+Test3.5c)

AI: 	Class F -4.14%,111%,102%; 	Class TGM -9.03%,102%,102%

RA: 	Class F -2.67%,106%,101%; 	Class TGM -6.35%,102%,101%

LDB: 	Class F -1.74%,107%,94%; 	Class TGM -5.18%,101%,89%

EE2-3.6j (i.e. Test 3.2+Test3.4+Test3.5c)

AI: 	Class F -4.43%,117%,106%; 	Class TGM -8.43%,107%,110%

RA: 	Class F -2.80%,108%,101%; 	Class TGM -5.53%,104%,102%

LDB: 	Class F -1.79%,110%,91%; 	Class TGM -4.37%,105%,91%

EE2-3.6k (i.e. Test 3.3+Test3.4+Test3.5c)

AI: 	Class F -0.72%,109%,105%; 	Class TGM -1.61%,107%,113%

RA: 	Class F -0.56%,107%,100%; 	Class TGM -1.45%,105%,103%

LDB: 	Class F -0.42%,109%,101%; 	Class TGM -1.17%,104%,98%

EE2-3.6l (i.e. Test 3.2+Test3.3+Test3.4+Test3.5c)

AI: 	Class F -4.54%,118%,98%; 	Class TGM -9.56%,108%,93%

RA: 	Class F -2.99%,110%,99%; 	Class TGM -6.57%,106%,97%

LDB: 	Class F -1.94%,113%,93%; 	Class TGM -5.30%,104%,91%

# Introduction

In JVET-Y0124 [1], IBC with an extended reference area by covering more CTUs that have been used by other coding modes is proposed.

In JVET-Y0160 [2], the number of HMVP candidates for IBC is increased to improve the coding efficiency.

In JVET-Y0088 [3], Template Matching with IBC for both IBC merge mode and IBC AMVP mode is proposed.

In JVET-Y0133 [4], two techniques are proposed in the IBC Merge/AMVP list construction. (1) the clipping of the invalid candidates BVP pointing outside of the IBC reference region; (2) the replacement of the null candidates including completing the list when there are no sufficient valid candidates, by a set of valid candidates distributed in the IBC reference region.

# Proposed methods

In this contribution, different combination tests of EE2-3.2+EE2-3.3+EE2-3.4+EE2-3.5 are tested per the description of EE2.

# Experimental Results

The combination tests have been implemented on top of the ECM-4.0 [5]. Testing results under AI/RA/LDB configurations are summarized in Table 1~11. The simulations were performed following the common test conditions [6] with the SCC configuration. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of EE2-3.6a compared with the ECM-4.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -4.13% | -3.71% | -3.80% | 107% | 96% |
| Class TGM | -9.06% | -8.33% | -8.30% | 104% | 83% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -2.56% | -2.67% | -2.74% | 103% | 99% |
| Class TGM | -6.17% | -5.89% | -6.09% | 101% | 95% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.59% | -1.09% | -2.14% | 101% | 100% |
| Class TGM | -5.00% | -4.78% | -4.94% | 100% | 93% |

Table 2: Coding performance of EE2-3.6b compared with the ECM-4.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -4.51% | -4.29% | -4.28% | 108% | 103% |  |
| Class TGM | -8.53% | -7.87% | -7.85% | 107% | 103% |  |
| Overall | -6.52% | -6.08% | -6.06% | 107% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -2.83% | -2.84% | -2.91% | 105% |  | 103% |  |
| Class TGM | -5.60% | -5.30% | -5.45% | 106% |  | 103% |  |
| Overall | -4.21% | -4.07% | -4.18% | 105% | 103% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.92% | -1.05% | -2.82% | 105% |  | 103% |  |
| Class TGM | -4.45% | -3.98% | -4.18% | 105% |  | 103% |  |
| Overall | -3.18% | -2.51% | -3.50% | 105% | 103% |

Table 3: Coding performance of EE2-3.6c compared with the ECM-4.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -3.92% | -3.59% | -3.74% | 109% | 99% |
| Class TGM | -7.93% | -7.37% | -7.34% | 102% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -2.46% | -2.63% | -2.54% | 105% | 100% |
| Class TGM | -5.29% | -5.01% | -5.23% | 102% | 101% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.64% | -1.27% | -2.64% | 104% | 97% |
| Class TGM | -4.20% | -3.92% | -4.22% | 100% | 89% |

Table 4: Coding performance of EE2-3.6e compared with the ECM-4.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.69% | -0.44% | -0.61% | 106% | 103% |
| Class TGM | -1.68% | -1.42% | -1.41% | 106% | 103% |
| Overall | -1.19% | -0.93% | -1.01% | 106% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.62% | -0.76% | -0.63% | 104% | 101% |
| Class TGM | -1.50% | -1.44% | -1.39% | 105% | 103% |
| Overall | -1.06% | -1.10% | -1.01% | 104% | 102% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.76% | 0.67% | -0.42% | 104% | 101% |
| Class TGM | -1.21% | -1.08% | -1.12% | 103% | 102% |
| Overall | -0.98% | -0.20% | -0.77% | 104% | 102% |

Table 5: Coding performance of EE2-3.6f compared with the ECM-4.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.42% | -0.10% | -0.19% | 104% | 101% |
| Class TGM | -1.22% | -1.05% | -1.00% | 102% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.37% | -0.72% | -0.59% | 104% | 100% |
| Class TGM | -1.24% | -1.22% | -1.21% | 103% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.28% | -0.01% | -0.90% | 106% | 99% |
| Class TGM | -0.99% | -1.04% | -1.08% | 100% | 98% |

Table 6: Coding performance of EE2-3.6g compared with the ECM-4.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.74% | -0.35% | -0.39% | 104% | 102% |
| Class TGM | -0.56% | -0.47% | -0.43% | 105% | 103% |
| Overall | -0.65% | -0.41% | -0.41% | 105% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.42% | -0.51% | -0.44% | 104% | 101% |
| Class TGM | -0.46% | -0.37% | -0.36% | 104% | 102% |
| Overall | -0.44% | -0.44% | -0.40% | 104% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.34% | 1.22% | -0.45% | 103% | 101% |
| Class TGM | -0.33% | -0.31% | -0.02% | 102% | 101% |
| Overall | -0.34% | 0.45% | -0.24% | 103% | 101% |

Table 7: Coding performance of EE2-3.6h compared with the ECM-4.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -4.63% | -4.18% | -4.40% | 116% | 98% |
| Class TGM | -9.60% | -8.80% | -8.77% | 110% | 92% |
| Overall | -7.11% | -6.49% | -6.59% | 114% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -3.01% | -3.27% | -2.96% | 108% | 98% |
| Class TGM | -6.62% | -6.33% | -6.47% | 103% | 96% |
| Overall | -4.81% | -4.80% | -4.72% | 105% | 97% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -2.08% | -1.30% | -2.55% | 108% | 98% |
| Class TGM | -5.35% | -5.10% | -5.44% | 103% | 96% |
| Overall | -3.71% | -3.20% | -4.00% | 105% | 97% |

Table 8: Coding performance of EE2-3.6i compared with the ECM-4.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -4.14% | -3.89% | -3.86% | 111% | 102% |
| Class TGM | -9.03% | -8.27% | -8.27% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -2.67% | -2.91% | -2.81% | 106% | 101% |
| Class TGM | -6.35% | -6.08% | -6.27% | 102% | 101% |

|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class F | -1.74% | -1.20% |  | -1.98% |  | 107% |  | 94% |  |
| Class TGM | -5.18% | -4.86% |  | -5.29% |  | 101% |  | 89% |  |

Table 9: Coding performance of EE2-3.6j compared with the ECM-4.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -4.43% | -4.07% | -4.29% | 117% | 106% |
| Class TGM | -8.43% | -7.85% | -7.82% | 107% | 110% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -2.80% | -3.06% | -3.10% | 108% | 101% |
| Class TGM | -5.53% | -5.23% | -5.38% | 104% | 102% |

|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class F | -1.79% | -1.21% |  | -2.25% |  | 110% |  | 91% |  |
| Class TGM | -4.37% | -4.06% |  | -4.19% |  | 105% |  | 91% |  |

Table 10: Coding performance of EE2-3.6k compared with the ECM-4.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.72% | -0.56% | -0.52% | 109% | 105% |
| Class TGM | -1.61% | -1.40% | -1.37% | 107% | 113% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.56% | -0.66% | -0.65% | 107% | 100% |
| Class TGM | -1.45% | -1.37% | -1.35% | 105% | 103% |

|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class F | -0.42% | 0.05% |  | -1.21% |  | 109% |  | 101% |  |
| Class TGM | -1.17% | -1.05% |  | -1.06% |  | 104% |  | 98% |  |

Table 11: Coding performance of EE2-3.6l compared with the ECM-4.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -4.54% | -4.08% | -4.33% | 118% | 98% |
| Class TGM | -9.56% | -8.74% | -8.71% | 108% | 93% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -2.99% | -3.10% | -3.18% | 110% | 99% |
| Class TGM | -6.57% | -6.20% | -6.48% | 106% | 97% |

|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class F | -1.94% | -1.45% |  | -2.36% |  | 113% |  | 93% |  |
| Class TGM | -5.30% | -5.01% |  | -5.35% |  | 104% |  | 91% |  |

- Conclusions

In this contribution, different combination tests of EE2-3.2+EE2-3.3+EE2-3.4+EE2-3.5 are tested. It is recommended to adopt one of them into ECM.

# References

[1] J. Xu, “Non-EE2: Intra Block Copy with An Extended Reference Area,” JVET-Y0124, Jan. 2022.

[2] N. Zhang, K. Zhang, L. Zhang, “EE2-3.13-related: Enlarged HMVP table for IBC,” JVET-Y0160, Jan. 2022.

[3] A. Robert, K. Naser, T. Poirier, Y. Chen, F. Galpin, “EE2-related: IBC with Template Matching,” JVET-Y0088, Jan. 2022.

[4] D. Ruiz Coll, A. Filippov, V. Rufitskiy, T.M. Bae, “EE2-related: BVP candidate adjustment based on IBC reference region implemented on top of test EE2-3.13,” JVET-Y0133, Jan. 2022.

[5] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-4.0

[6] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

ByteDance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).