| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0169-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0048 (EE2-related: Modification of LFNST for MIP coded block) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xinwei Li | Tel: Email: | sid.lxw@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-Z0048. The simulation results confirm that BD-rate results match with the proponent’s results under all intra and random access test conditions.

Introduction

JVET-Z0048 [1] proposes to use DIMD to derive a traditional intra prediction mode based on the prediction samples of MIP modes, then the LFNST transform set and LFNST transpose flag are both determined by the derived intra mode. Moreover, LFNST is enable for MIP coded blocks of width and height greater than or equal to 4.

Simulation results

The simulation results of JVET-Z0048 are verified. The tests were conducted under the common test conditions (CTC) of ECM-4.0 [2].

Table 1. Simulation results

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.19% | 0.09% | 105% | 100% |
| Class A2 | -0.04% | 0.00% | 0.07% | 105% | 100% |
| Class B | -0.06% | -0.11% | -0.09% | 105% | 101% |
| Class C | -0.11% | 0.01% | -0.06% | 104% | 101% |
| Class E | -0.12% | 0.01% | 0.01% | 105% | 101% |
| Overall | -0.07% | -0.06% | -0.01% | 105% | 101% |
| Class D | -0.11% | 0.01% | -0.20% | 105% | 102% |
| Class F | -0.15% | 0.09% | 0.00% | 104% | 101% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.17% | 0.08% | 101% | 99% |
| Class A2 | -0.04% | -0.05% | -0.01% | 101% | 99% |
| Class B | -0.07% | -0.04% | -0.03% | 102% | 100% |
| Class C | -0.11% | -0.37% | 0.10% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.16% | 0.03% | 101% | 100% |
| Class D | -0.03% | -0.61% | 0.05% | 102% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

References

- J.-Y. Huo, et. al., “EE2-related: Modification of LFNST for MIP coded block,” JVET-Z0048, April 2022.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

Patent rights declaration(s)

Alibaba Group does not have any current or pending patent rights relating to the technology described in this contribution.