| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0178-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0091 (EE1-1.2: Neural network based in-loop filter with a single model) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Zhenyu Dai | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for EE1-1.2 reported in JVET-Z0091 [1], which uses neural network-based models for in-loop filtering on top of the VTM software.

The crosscheck conducted all the test conditions, including All Intra, Random access and Low delay B.

The results exactly match for AI and LDB conditions in terms of bitrate and PSNR, while mentioning some cases in RA may have 0.0001 difference which are probably due to the rounding difference.

# Introduction

EE1-1.2 proposed a neural network based in-loop filter, which is a further development for NN-based filter studied in EE1-1.1. Compared with EE1-1.1, the number of models is decreased into only one for further saving the parameter memory.

The crosscheck was conducted by cloning the EE1-1.2 branch (SHA: ac06943a) from VTM repository. There are 3 test items in all as following,

Test 1.2.1: Optimize the filter in both the network and the implementation based on JVET-Y0080.

Test 1.2.2: Float SADL implementation of Test 1.2.1.

Test 1.2.3: Integer SADL implementation of Test 1.2.1.

# Results

The source code was compiled using gcc 8.1.0 and tested according to the NN-CTC[2] on Ubuntu 16.04, CPU only. The results were compared with the test results provided by the proponent for AI, RA and LDB.

## Test 1.2.1

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -6.18% | -13.48% | -16.09% | -7.42% | -16.25% | -17.03% | 141% | 19794% |
| Class A2 | -5.80% | -14.77% | -12.04% | -6.64% | -15.41% | -11.01% | 118% | 17186% |
| Class B | -5.86% | -14.79% | -16.04% | -6.03% | -16.03% | -17.22% | 112% | 16237% |
| Class C | -6.44% | -15.94% | -17.95% | -6.54% | -17.06% | -18.80% | 106% | 12074% |
| Class E | -8.69% | -15.12% | -17.00% | -8.98% | -14.87% | -17.81% | 118% | 18237% |
| Overall | -6.50% | -14.88% | -15.96% | -6.97% | -16.00% | -16.60% | 117% | 16172% |
| Class D | -6.47% | -15.00% | -17.77% | -5.92% | -16.81% | -18.81% | 104% | 13267% |
| Class F | -4.06% | -11.21% | -11.44% | -4.00% | -12.60% | -13.13% | 94% | 13882% |

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.34% | -15.99% | -18.74% | -10.45% | -18.01% | -20.87% | 125% | 39106% |
| Class A2 | -9.30% | -17.82% | -14.84% | -9.08% | -17.24% | -13.15% | 120% | 36596% |
| Class B | -8.29% | -18.99% | -19.70% | -8.22% | -19.13% | -19.58% | 126% | 40799% |
| Class C | -8.20% | -20.74% | -21.33% | -8.34% | -18.15% | -19.07% | 116% | 36262% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -8.68% | -18.63% | -18.97% | -8.87% | -18.27% | -18.42% | 122% | 38359% |
| Class D | -9.02% | -19.10% | -21.32% | -7.41% | -16.82% | -18.24% | 117% | 33920% |
| Class F | -3.77% | -12.08% | -11.64% | -4.22% | -12.64% | -11.91% | 141% | 19260% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -7.27% | -14.59% | -16.42% | -7.22% | -14.23% | -17.21% | 108% | 30912% |
| Class C | -7.83% | -17.03% | -17.91% | -8.02% | -10.05% | -14.12% | 96% | 23968% |
| Class E | -7.71% | -13.81% | -15.08% | -7.17% | -12.87% | -15.67% | 136% | 19702% |
| Overall | -7.57% | -15.68% | -17.08% | -7.47% | -12.37% | -15.84% | 110% | 25374% |
| Class D | -8.85% | -17.16% | -18.37% | -7.54% | -12.63% | -14.86% | 105% | 28309% |
| Class F | -4.16% | -11.18% | -10.06% | -4.85% | -10.24% | -11.39% | 122% | 11913% |

## Test 1.2.2

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -6.18% | -13.48% | -16.09% | -7.42% | -16.25% | -17.03% | 376% | 121239% |
| Class A2 | -5.80% | -14.78% | -12.06% | -6.64% | -15.41% | -11.01% | 245% | 100323% |
| Class B | -5.86% | -14.79% | -16.03% | -6.03% | -16.03% | -17.22% | 230% | 105350% |
| Class C | -6.44% | -15.94% | -17.95% | -6.54% | -17.06% | -18.80% | 181% | 82866% |
| Class E | -8.69% | -15.12% | -17.00% | -8.97% | -14.87% | -17.81% | 242% | 112849% |
| Overall | -6.50% | -14.88% | -15.97% | -6.97% | -16.00% | -16.60% | 241% | 102582% |
| Class D | -6.47% | -15.00% | -17.77% | -5.92% | -16.81% | -18.81% | 163% | 81082% |
| Class F | -4.06% | -11.21% | -11.44% | -4.00% | -12.60% | -13.13% | 142% | 86109% |

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.31% | -15.90% | -18.79% | -10.33% | -17.94% | -21.12% | 282% | 288155% |
| Class A2 | -9.30% | -17.80% | -14.87% | -9.14% | -16.92% | -13.16% | 260% | 258489% |
| Class B | -8.30% | -19.03% | -19.66% | -8.23% | -19.22% | -19.57% | 275% | 272114% |
| Class C | -8.20% | -20.81% | -21.32% | -8.37% | -18.23% | -19.13% | 227% | 235513% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -8.67% | -18.63% | -18.97% | -8.87% | -18.24% | -18.48% | 260% | 262141% |
| Class D | -9.02% | -19.11% | -21.36% | -7.39% | -16.82% | -18.21% | 244% | 219164% |
| Class F | -3.76% | -12.04% | -11.62% | -4.19% | -12.56% | -12.05% | 374% | 124948% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -7.26% | -14.54% | -16.14% | -7.18% | -14.45% | -16.65% | 210% | 179075% |
| Class C | -7.86% | -17.20% | -17.80% | -8.07% | -10.52% | -14.00% | 207% | 162147% |
| Class E | -7.81% | -13.84% | -15.28% | -7.37% | -13.00% | -16.00% | 442% | 129369% |
| Overall | -7.60% | -15.72% | -16.87% | -7.52% | -12.70% | -15.47% | 252% | 159719% |
| Class D | -8.86% | -16.35% | -18.33% | -7.51% | -12.21% | -15.79% | 236% | 218880% |
| Class F | -4.12% | -10.74% | -9.90% | -4.68% | -10.19% | -11.03% | 336% | 77550% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -6.18% | -13.48% | -16.09% | -7.42% | -16.25% | -17.03% | 376% | 121239% |
| Class A2 | -5.80% | -14.78% | -12.06% | -6.64% | -15.41% | -11.01% | 245% | 100323% |
| Class B | -5.86% | -14.79% | -16.03% | -6.03% | -16.03% | -17.22% | 230% | 105350% |
| Class C | -6.44% | -15.94% | -17.95% | -6.54% | -17.06% | -18.80% | 181% | 82866% |
| Class E | -8.69% | -15.12% | -17.00% | -8.97% | -14.87% | -17.81% | 242% | 112849% |
| Overall | -6.50% | -14.88% | -15.97% | -6.97% | -16.00% | -16.60% | 241% | 102582% |
| Class D | -6.47% | -15.00% | -17.77% | -5.92% | -16.81% | -18.81% | 163% | 81082% |
| Class F | -4.06% | -11.21% | -11.44% | -4.00% | -12.60% | -13.13% | 142% | 86109% |

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.31% | -15.90% | -18.79% | -10.33% | -17.94% | -21.12% | 282% | 288155% |
| Class A2 | -9.30% | -17.80% | -14.87% | -9.14% | -16.92% | -13.16% | 260% | 258489% |
| Class B | -8.30% | -19.03% | -19.66% | -8.23% | -19.22% | -19.57% | 275% | 272114% |
| Class C | -8.20% | -20.81% | -21.32% | -8.37% | -18.23% | -19.13% | 227% | 235513% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -8.67% | -18.63% | -18.97% | -8.87% | -18.24% | -18.48% | 260% | 262141% |
| Class D | -9.02% | -19.11% | -21.36% | -7.39% | -16.82% | -18.21% | 244% | 219164% |
| Class F | -3.76% | -12.04% | -11.62% | -4.19% | -12.56% | -12.05% | 374% | 124948% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -7.26% | -14.54% | -16.14% | -7.18% | -14.45% | -16.65% | 210% | 179075% |
| Class C | -7.86% | -17.20% | -17.80% | -8.07% | -10.52% | -14.00% | 207% | 162147% |
| Class E | -7.81% | -13.84% | -15.28% | -7.37% | -13.00% | -16.00% | 442% | 129369% |
| Overall | -7.60% | -15.72% | -16.87% | -7.52% | -12.70% | -15.47% | 252% | 159719% |
| Class D | -8.86% | -16.35% | -18.33% | -7.51% | -12.21% | -15.79% | 236% | 218880% |
| Class F | -4.12% | -10.74% | -9.90% | -4.68% | -10.19% | -11.03% | 336% | 77550% |

# Reference

[1] “EE1-1.2: Neural network based in-loop filter with a single model”, JVET-Z0091, L. Wang, X. Xu, S. Liu

[2] Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Y2016, S. Liu, A. Segall, E. Alshina, R.-L. Liao