| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0187-v1 |
| --- | --- |

| Title: | Cross-check of JVET-Z0123 (EE2-related: On chroma interpolation filters for motion compensation) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | K. Andersson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

This document verifies reported PSNR and bitrate numbers in JVET-Z0123, for all bitstreams completed so far.

# Introduction

In JVET-Z0123 they suggest to increase the chroma motion compensation filters from 4-tap to 6-taps with the following set of coefficients for phases 1 to 31:

{{  1,  -6, 255,   8,  -2,   0},

{  3, -12, 254,  14,  -4,   1},

{  5, -18, 252,  23,  -7,   1},

{  7, -23, 248,  32,  -9,   1},

{  7, -27, 245,  41, -11,   1},

{  8, -30, 240,  50, -14,   2},

{ 10, -34, 234,  60, -17,   3},

{ 10, -36, 228,  71, -20,   3},

{ 10, -38, 221,  81, -22,   4},

{ 10, -39, 213,  92, -25,   5},

{ 11, -40, 205, 103, -28,   5},

{ 11, -41, 196, 114, -30,   6},

{ 10, -41, 187, 125, -32,   7},

{ 10, -40, 177, 135, -34,   8},

{ 10, -39, 167, 146, -36,   8},

{  9, -38, 157, 157, -38,   9},

{  8, -36, 146, 167, -39,  10},

{  8, -34, 135, 177, -40,  10},

{  7, -32, 125, 187, -41,  10},

{  6, -30, 114, 196, -41,  11},

{  5, -28, 103, 205, -40,  11},

{  5, -25,  92, 213, -39,  10},

{  4, -22,  81, 221, -38,  10},

{  3, -20,  71, 228, -36,  10},

{  3, -17,  60, 234, -34,  10},

{  2, -14,  50, 240, -30,   8},

{  1, -11,  41, 245, -27,   7},

{  1,  -9,  32, 248, -23,   7},

{  1,  -7,  23, 252, -18,   5},

{  1,  -4,  14, 254, -12,   3},

{  0,  -2,   8, 255,  -6,   1}}

By inspection of the phase and magnitude responses we noted that the phases of the proposed filters were misaligned at lower frequencies.

Below the phase and magnitude responses of the filters suggested in JVET-Z0123 are shown. The green straight lines in the top part (a) of the figures show the ideal phase delay responses, and the red curves show the phase delay responses for the JVET-Z0123 filters. Likewise, the green line in the bottom part (b) of the figures shows the ideal magnitude responses and the red curves show the magnitude responses of the JVET-Z0123 filters.

Figure 1: Plots of phase delay responses (a) and magnitude of frequency responses (b) of 6-tap chroma interpolation filters in JVET-Z0123.

# Results

We confirm the reported bitrate and PSNR for all bitstreams completed so far. Encoding and decoding times are not accurate.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0,07% | -1,06% | -1,33% | 95% | 101% |
| Class C | -0,05% | -2,17% | -2,14% | 96% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0,01% | -3,40% | -3,95% | 108% | 110% |
| Class F | -0,08% | -1,17% | -1,23% | 96% | 94% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0,11% | -1,99% | -2,38% | 93% | 97% |
| Class E | -0,08% | -0,08% | 0,70% | 94% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0,19% | -2,09% | -3,70% | 96% | 97% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# References

[1] J. Gan, Y. Yu, H. Yu (OPPO), “EE2-related: On chroma interpolation filters for motion compensation”, JVET-Z0123, on-line meeting, April 2022

[2] V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-Y2024, Meeting by Teleconference, January 2022

[3] ECM-4.0 reference software,  https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Meeting by Teleconference, January 2022

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).