| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0199 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0157 (EE2-3.2-related: IBC Adaptation for Camera-Captured Contents) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Damian Ruiz Coll | Tel: Email: | druizcoll@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for JVET-Z0157, which proposes to adopt the intra-block copy (IBC) scheme proposed in EE2-3.2 to camera capture content, including two modifications regarding the IBC reference region and the IBC search range. The cross-checked source code follows the description in the proposal, and the results are matched with the results provided by the proponent.

# Introduction

In JVET-Z0157, the IBC scheme proposed in EE2-3.2 test is used, which proposes an extended IBC reference region up to two CTU rows above in addition to the CTU-row of the current CTU. Figure 1 illustrates the reference area for coding CTU (m,n). Specifically, for CTU (m,n) to be coded, the reference area includes CTUs with index (m–2,n–2)…(W,n–2),(0,n–1)…(W,n–1),(0,n)…(m,n), where W denotes the maximum horizontal index within the current tile, slice or picture two methods to adjust the BVP candidates.

Figure 1. Figure 1. Reference area for IBC when CTU size is 128x128. The green and blue blocks denote the reference area while the white blocks denote the invalid reference area.

However, when CTU size is 256x256, which is the default setting in the coding of classA sequences, two additional rows of CTU above may require extra memory. To keep IBC from using extra memory, when CTU size is 256x256, the reference area is shown in Figure 2. Specifically, assume that the current CTU index is (m, n), the reference area includes CTUs with index (0,n)…(m,n) and (m–1,n–1)…(W,n–1).

Besides the change of EE2-3.2 decoder when CTU size is 256x256, the encoder of EE2-3.2 is modified to limit the per-sample block vector search (or called local search) range to be [–12,12] horizontally and [–12,12] vertically centered at the first block vector predictor for each IBC block.

Figure 2. Reference area for IBC when CTU size is 256x256

Cross-check of the source code and the simulation results have been conducted. Inspected source code follows the description in the proposal. The simulation is performed on top of ECM-4.0. The cross-check results are provided in the tables below and match the results provided by the proponent. Note that coding time is not accurate.

Below is the test result:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.03% | -0.08% | 111% | 102% |
| Class A2 |  |  |  |  |  |
| Class B | -0.67% | -0.72% | -0.78% | 108% | 101% |
| Class C | -0.18% | -0.20% | -0.06% | 107% | 102% |
| Class E | -0.96% | -1.01% | -1.02% | 108% | 101% |
| Overall |  |  |  |  |  |
| Class D | -0.49% | -0.49% | -0.74% | 105% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.09% | 0.20% | 98% | 101% |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | -0.32% | -0.34% | 99% | 98% |
| Class C | 0.23% | -0.05% | 0.18% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.19% | -0.27% | -0.48% | 99% | 98% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.05% | 0.22% | -0.23% | 106% | 98% |
| Class E | -0.25% | -0.24% | -0.50% | 104% | 97% |
| Overall |  |  |  |  |  |
| Class D | -0.02% | -0.15% | 0.01% | 104% | 98% |

It is confirmed that the obtained results perfectly match the BD-rate gains reported in JVET-Z0157. Obtained runtimes relative to anchor are in the same range as those reported in JVET-Z0157.

# Conclusion

Results matched with the results provided by the proponent.

# Reference

[1] Jizheng Xu, “EE2-3.2-related: IBC Adaptation for Camera-Captured Contents”, JVET-Z0157, April 2022