| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0200-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0131 (EE2-related: Block Vector Difference Binarization) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Damian Ruiz Coll | Tel: Email: | druizcoll@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of the JVET-131 proposal, which proposes a binarization method for IBC block vector difference using Exp-Golomb code with the context-coded prefix.

The cross-checked source code follows the description in the proposal, and the results are matched with the results provided by the proponents.

# Introduction

In ECM4.0, Block Vector Difference (BVD) shares the same binarization method as Motion Vector Difference (MVD). For each greater than 0 component, a greater than 1 flag is signaled followed by the bypass coded remaining magnitude binarized with EG1 code. The whole flowchart for the binarization method is shown in Fig.1

JVET-131 propose three test, which crosschecking results are reported as follows:

Below is the result for the Test #1 (removing the “greater than 1 flag” context coded the first 5 bins of the EG1 prefix):

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.73% | -0.49% | -0.54% | 102% | 100% |
| Class TGM | -1.53% | -1.36% | -1.33% | 101% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.32% | -0.55% | -0.47% | 102% | 99% |
| Class TGM | -0.98% | -0.82% | -0.78% | 102% | 100% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.19% | 1.07% | -0.01% | 102% | 101% |
| Class TGM | -0.66% | -0.58% | -0.46% | 99% | 97% |

Below is the result for the Test #2 (keeping the “greater than 1 flag” and the first 5 bins of the EG1 prefix is context coded):

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.69% | -0.31% | -0.49% | 103% | 101% |
| Class TGM | -1.49% | -1.31% | -1.30% | 101% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.38% | -0.48% | -0.46% | 102% | 100% |
| Class TGM | -0.89% | -0.81% | -0.78% | 102% | 100% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.54% | -0.04% | -0.61% | 102% | 100% |
| Class TGM | -0.57% | -0.30% | -0.31% | 99% | 98% |

Below is the result for the Test #3 (proposed method implemented on top of EE2-3.6l):

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -5.11% | -4.65% | -4.75% | 123% | 98% |
| Class TGM | -10.82% | -9.97% | -9.96% | 110% | 93% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -3.35% | -3.34% | -3.41% | 110% | 100% |
| Class TGM | -7.43% | -6.99% | -7.24% | 106% | 97% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -2.28% | -1.81% | -2.41% | 113% | 100% |
| Class TGM | -6.10% | -5.65% | -5.97% | 105% | 91% |

It is confirmed that the obtained results perfectly match the BD-rate gains reported in JVET-0131. Obtained runtimes relative to anchor are in the same range as those reported in JVET-0131.

# Conclusion

Results of JVET-0131 have been confirmed.

# Reference

[1] Keming Cao, Vadim Seregin, Marta Karczewicz, “EE2-related: Block Vector Difference Binarization”, JVET-Z0131, April 2022