| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0204-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0059 (Non-EE2: Adaptive width for GPM blending area) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Zhipin Deng | Tel: Email: | zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-Z0059. The simulation results reportedly match those provided by the proponents.

# Verification

JVET-Z0059 [1] proposes an adaptive width for GPM blending area to further enhance coding performance of ECM. The proposed method enables GPM to select the adaptive blending width by signalling among the three different widths: the same width as that of VVC and ECM, a quarter of that width, and four times of that width. The maximum value of the weighs is changed from 8 to 32 to maximize the effect especially for wider widths. Moreover, an additional signalling that restricts selectable width only to the narrowest width, is proposed to optimize the proposed method for SCC.

We verified that

- The source code implements match with what was described in the proposal.

- Simulation results match with those provided in the proposal.

The following two tests were arranged for the crosscheck.

- Test 1: the proposed method w/o additional signalling restriction

- Test 2: the proposed method w/ additional signalling restriction (only for class F and class TGM)

The coding results of Test 1 and Test 2 under the ECM CTC are summarized in Table 1-2.

Table 1: Performance of method 1 over ECM-4.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.47% | -0.18% | 99% | 98% |
| Class A2 | -0.05% | -0.12% | 0.00% | 99% | 98% |
| Class B | -0.13% | -0.10% | -0.27% | 101% | 99% |
| Class C | -0.24% | -0.13% | -0.05% | 105% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.15% | -0.18% | -0.14% | 101% | 100% |
| Class D | -0.26% | -0.28% | -0.07% | 107% | 109% |
| Class F | -0.62% | -0.67% | -0.67% | 105% | 108% |
| Class TGM | -2.37% | -1.76% | -1.70% | 99% | 94% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | -0.26% | -0.19% | 89% | 99% |
| Class C | -0.36% | 0.16% | -0.52% | 103% | 99% |
| Class E | -0.21% | -0.44% | -0.26% | 102% | 104% |
| Overall | -0.23% | -0.17% | -0.32% | 96% | 100% |
| Class D | -0.42% | 0.18% | -0.68% | 108% | 107% |
| Class F | -0.72% | -0.40% | -1.00% | 103% | 106% |
| Class TGM | -3.27% | -2.62% | -2.51% | 97% | 92% |

Table 2: Performance of method 2 over ECM-4.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.45% | -0.34% | -0.24% | 103% | 110% |
| Class TGM | -2.50% | -1.94% | -1.83% | 98% | 95% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.59% | 0.50% | -0.54% | 101% | 107% |
| Class TGM | -3.54% | -2.89% | -2.71% | 96% | 92% |

The BD rate differences match with the results from the proposal [1]. Note that timing results are not accurate in this report, since the crosscheck tests and anchor test were launched in different time slots with different simulation environments.

# References

Y. Kidani, H. Kato, K. Unno, K. Kawamura, “Non-EE2: Adaptive width for GPM blending area”, JVET-Z0059, April 2022.