| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0206-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0143 (Non-EE2: Chroma intra modes derived from collocated luma blocks and neighboring chroma blocks) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Pekka Astola | Email: | pekka.astola@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for JVET-Z0143. It was verified that the software corresponds to the description in the proposal.

# Introduction

JVET-Z0143 proposes adding multiple direct modes, neighbouring chroma modes, and secondary chroma modes into the chroma intra mode list. The proposed modes are derived from the intra prediction modes of co-located luma blocks and neighbouring chroma blocks. The 5 non-CCLM chroma intra modes are replaced with the newly derived 9 chroma intra modes. Four different approaches are used to derive four groups of chroma intra modes and the first 9 unique modes are added into the non-CCLM chroma mode list. The index of the selected candidate mode is signalled using truncated binary coding.

The available results are summarized in the below tables. Due to limited computational capacity only classes C and D were crosschecked for Random Access. All available results are verified to match those reported in the proposal.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06 % | -0.56 % | -0.40 % | 102 % | 98 % |
| Class A2 | -0.07 % | -0.76 % | -0.78 % | 103 % | 97 % |
| Class B | -0.07 % | -0.66 % | -0.74 % | 102 % | 98 % |
| Class C | -0.08 % | -0.51 % | -0.51 % | 102 % | 97 % |
| Class E | -0.16 % | -1.39 % | -0.90 % | 102 % | 96 % |
| Overall | -0.08 % | -0.75 % | -0.67 % | 102 % | 97 % |
| Class D | -0.07 % | -0.34 % | -0.54 % | 103 % | 95 % |
| Class F | -0.06 % | -0.09 % | -0.54 % | 101 % | 98 % |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.04 % | -0.29 % | -0.07 % | 101 % | 98 % |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00 % | -0.42 % | -0.37 % | 100 % | 100 % |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

The two tables below report partial results on top of ECM-4.0 + EE2-1.1 + EE2-1.2b. Results are verified to match those reported in Table 2 of JVET-Z0143-v2.docx.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.15 % | -1.94 % | -2.02 % | 103 % | #DIV/0! |
| Class E | -0.15 % | -2.44 % | -1.87 % | 103 % | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.12 % | -1.56 % | -1.51 % | 103 % | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.05 % | -0.64 % | -0.93 % | 101 % | 98 % |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |