| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0207-v1 |
| --- | --- |

| Title: | Cross-check of JVET-Z0165 (Combination tests of EE2-3.2+EE2-3.3+EE2-3.4+EE2-3.5): test EE2-3.6i |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Antoine Robert | Email: | antoine.robert@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking of EE2-3.6i as the combination of EE2-3.2: IBC with extended reference area, EE2-3.3: Enlarged HMVP table for IBC and EE2-3.5c: Invalid BVP candidate adjustment based on IBC reference region and replacement of zero-vector candidates in IBC Merge/AMVP list.

It is reported that the cross-check confirms the results.

# Introduction

In EE2, several tools have been proposed on IBC. Three of these tests are:

- EE2-3.2: it extends the IBC reference region to cover more CTUs that have been used by other coding modes in the current and above CTU rows.

- EE2-3.3: the HMVP table size for IBC is increased to a number larger than 5 (such as 30). And the size of IBC merge list could be enlarged correspondingly. All the IBC merge candidates can be involved in the reordering process. After reordering, the first M candidates with the lowest costs can be selected as the final candidates in the IBC merge list.

- EE2-3.5: two modifications to the IBC Merge/AMVP list construction are tested. First, before checking the validity of the BVP candidates, an adjustment of the candidates pointing outside of the reference region to the nearest boundary of the IBC reference region is applied. Second, a method is tested to replace the zero-padding candidates included to complete the candidate list when there are no sufficient valid candidates by a set of valid candidates distributed in the IBC reference region.

# Simulation results

The proposed combination EE2-3.6i of tests 3.2, 3.3 and 3.5c has been tested under ECM-4.0 CTCs and SCC configuration. The simulation results are presented in Table 1.

Table 1: Combined EE2-3.6i results on top of ECM-4.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -4.14% | -3.89% | -3.86% | 104% | 101% |
| Class TGM | -9.03% | -8.27% | -8.27% | 103% | 101% |
| Overall | -6.59% | -6.08% | -6.06% | 103% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -2.67% | -2.91% | -2.81% | 102% | 102% |
| Class TGM | -6.35% | -6.08% | -6.27% | 103% | 102% |
| Overall | -4.51% | -4.50% | -4.54% | 102% | 102% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.74% | -1.20% | -1.98% | 103% | 102% |
| Class TGM | -5.18% | -4.86% | -5.29% | 103% | 103% |
| Overall | -3.46% | -3.03% | -3.64% | 103% | 102% |

It is confirmed that the obtained results perfectly match the BD-rate gains reported in EE2. Obtained runtimes relative to anchor are in the same range to those reported in EE2.

# Conclusion

Results of EE2 test 3.6i have been confirmed.

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.