| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0209-v1 |
| --- | --- |

| Title: | AhG10：Encoding algorithm optimization for HTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Xile Zhou, Yanzhuo Ma, Fuzheng Yang, Ming Li, Yang Liu | Email: | jyhuo@mail.xidian.edu.cn zhouxile@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn myron.li@oppo.com serena@oppo.com |
| Source: | Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

In HTM 16.3[1], in depth coding, the rendering-based synthesized view distortion is used to calculate the RD cost, bringing sufficient coding gain which is time-consuming. In this proposal, a fast Rate-Distortion Optimization (RDO) method for depth maps is proposed, which checks the RD cost during its calculation process. In HTM,  given a coding mode, the RD cost is composed of several cumulative items. If the accumulated RD cost is equal to or exceeds the minimum RD cost of previously coded modes, it will not be necessary to continue the RD cost calculation for the mode. For RA configuration, compared with HTM16.3, -0.04% BD-rate performance is reported by calculating the synthesized view PSNR and total bitrate, with 89.0% EncT, 99.3% DecT. For depth encoding time, 71.9% EncT is reported.

Introduction

The typical test scenario of 3D-HEVC is shown in Figure 1.

Figure 1  Typical test scenario of 3D-HEVC

The texture videos and depth maps in real cameras (with blue border) are encoded and transmitted. At the decoder side, videos of multiple synthesis positions are synthesized using the reconstructed texture videos and depth maps.

In HTM 16.3, to ensure the quality of synthesized view, the RDO criterion in depth coding can be designed as

whereandare the weighting factors of the distortion of depth maps and synthesized views, respectively. is the depth distortion which is easy to be measured between the original depth block and the reconstructed depth block, is measured by synthesized view distortion change (SVDC)[2].

Proposed Methods

In this contribution, it is proposed to terminate the calculation early if the accumulated RD cost of the current mode is greater than or equal to the minimum RD cost of previously coded modes.

The proposed method use the complexity-guided RDO criterion in depth coding as

where the order is sorted according to the computational complexity of each item, is the number of virtual view that need to be synthesized.

Figure 2  Flowchart of the proposed method

The flowchart of the proposed method is depicted in Figure 2.

For the candidate mode, the RD cost calculation can be split into three stages, the coding bits calculation, depth distortion calculation, and SVD calculation. The proposed method introduces a comparison module at each stage, denoted as diamond blocks in Figure 2. Comparisons betweenandare conducted onceis updated. If is always smaller than, which means the early termination condition never satisfies, the whole RD cost calculation process is conducted to obtain. In this case, is set to, and is marked as the optimal mode among, and. Otherwise, i.e., ≥,  can be asserted as a non-optimal mode in advance andis set to.

Experimental results

The proposed method was implemented based on HTM 16.3 under common test condition. Table 1 presents a summary of the results with Random Access configurations.

Table 1. Performance of the proposed method in CTC

Conclusions

By integrating the proposed method on the latest reference software, it can reduce the encoding complexity by 11% with a small coding gain. It is recommended to investigate in EE.

- 5 References

- HTM-16.3, https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-16.3

- K. Muller, H. Schwarz, D. Marpe,et al., “3D high-efficiency video coding for multi-view video and depth data,” IEEE Transactions on Circuits and Systems for Video Technology.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).