| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0211-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0139 (EE2-2.7, 2.8, 2.9: History-parameter-based affine model inheritance and non-adjacent spatial neighbors for affine merge mode): Tests EE2-2.7c and EE2-2.9b |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Lien-Fei Chen | Email: | lienfeichen@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking of EE2-2.7c and EE2-2.9b of JVET-Z0139. EE2-2.7c combines EE2-2.7a, which is history-parameter-based affine model inheritance (HAMI) without tables stored in the line buffer, and the constructed NA-AFF candidates from non-adjacent affine mode (NA-AFF). The required memory in EE2-2.7c is about 0.8 KB memory. In EE2-2.9b, history-parameter-based affine model inheritance (HAMI) without tables stored in the line buffer is combined with the constructed NA-AFF candidates from non-adjacent affine mode and inherited NA-AFF candidates. The total memory requirement in EE2-2.9b is about 5KB memory. The results are obtained and match the coding results reported in JVET-Z0139. Runtime differ because of the server congestion and the runtime in this report may not be reliable.

# Crosscheck results

## Test Result of EE2-2.7c

In EE2-2.7c, the test result is shown in the following table

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.29% | -0.15% | 101% | 100% |
| Class A2 | -0.57% | -0.49% | -0.42% | 101% | 100% |
| Class B | -0.32% | -0.10% | -0.17% | 100% | 100% |
| Class C | -0.16% | -0.12% | -0.07% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.22% | -0.19% | 100% | 99% |
| Class D | -0.19% | -0.08% | -0.03% | 99% | 97% |
| Class F | -0.22% | -0.25% | -0.27% | 98% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.44% | -0.47% | -0.58% | 102% | 103% |
| Class C | -0.29% | -0.16% | -0.59% | 101% | 101% |
| Class E | -0.29% | -0.03% | -0.17% | 100% | 97% |
| Overall | -0.35% | -0.26% | -0.48% | 101% | 101% |
| Class D | -0.77% | -0.01% | 0.01% | 100% | 99% |
| Class F | -0.57% | 0.27% | -0.23% | 99% | 99% |

## Test Result of EE2-2.9b

In EE2-2.9b, the test result is shown in the following table

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -0.35% | -0.18% | 103% | 103% |
| Class A2 | -0.78% | -0.56% | -0.55% | 103% | 106% |
| Class B | -0.42% | -0.21% | -0.32% | 102% | 104% |
| Class C | -0.24% | -0.15% | -0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.45% | -0.29% | -0.28% | 102% | 103% |
| Class D | -0.28% | 0.06% | -0.15% | 100% | 100% |
| Class F | -0.30% | -0.15% | -0.44% | 100% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over ECM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.48% | -0.34% | -0.54% | 104% | 104% |
| Class C | -0.39% | -0.40% | -0.59% | 103% | 100% |
| Class E | -0.27% | -0.67% | -0.21% | 103% | 97% |
| Overall | -0.40% | -0.44% | -0.47% | 103% | 101% |
| Class D | -0.85% | -0.51% | -0.04% | 100% | 100% |
| Class F | -0.53% | 1.06% | -0.06% | 101% | 99% |

# Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.