| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0222 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0056 (EE2-2.4: Template matching based reordering for GPM split modes) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 70 3508 5829 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-Z0056. The proposal introduces the template matching based reordering for GPM splitting modes to improve the coding performance of ECM, which is being investigated in EE2 in this meeting.

Summary

In JVET-Z0056 [1], it is asserted that a fixed-length binary coded is used for signalling of the 64 GPM splitting modes in the current ECM design as equal-probable events, while the equal-assumption may not hold when the respective template matching (TM) cost values of all the GPM splitting modes are considered. In other words, GPM has room to improve the coding performance if the fixed-length binary code is replaced with variable-length code for the signalling of GPM splitting modes by considering TM costs.

Therefore, JVET-Z0056 proposes to introduce the TM based reordering for the GPM splitting modes in the decoding process of GPM, in order to replace the fixed-length binary codes with the variable-length code for the signalling of GPM splitting modes.

The source code uploaded in the JVET GitHub for EE2 of this meeting [2] was carefully analysed and the cross checker confirmed that the implementation matches the description in JVET-Z0056 but encountered the decoding errors in some video sequences by the decoder-only affected bug. The cross checker had informed the bug to the proponent in JVET-Z0056, and confirmed the decoding errors are resolved with the fixed source code [3] provided by the proponent and uploaded in the JVET GitLab on April 20, 2022.

Simulations were conducted under the RA and LB configuration of the CTC for enhanced compression tool testing [4]. The cross-check results are provided in the accompanying excel documents. After the above decoding errors have been fixed, the cross checker could exactly reproduce the results provided by the proponent and reported in JVET-Z0056. It is noted that the provided software was compiled on 64-bit Ubuntu environment. The experiments were performed on the cluster PCs.

Conclusion

We had verified results reported in JVET-Z0056. The results of the cross-check matched with proponent’s report.

References

- C.-C. Chen, H. Huang, Y. Zhang, Z. Zhang, Y.-J. Chang, V. Seregin, and M. Karczewicz, “EE2-2.4: Template matching based reordering for GPM split modes,” JVET-Z0056, by teleconference, 20–29 April 2022.

- https://vcgit.hhi.fraunhofer.de/ecm/jvet-y-ee2/ECM/-/commit/91ec4a7576a424218c3ac3b4f935854cbdca49eb

- https://vcgit.hhi.fraunhofer.de/ecm/jvet-y-ee2/ECM/-/commit/26b53418da4be5491feccd5fa392e0e4fc536a33

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, by teleconference, 12-21, January. 2022.

Patent rights declaration(s)

KDDI Corporation does not have any current or pending patent rights relating to the technology described in this contribution.