| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0223 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0137 (Non-EE2: Adaptive Blending for GPM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 70 3508 5829 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-Z0137, which proposes the adaptive blending for GPM. The cross checker had verified results reported in JVET-Z0137. The results of the cross-check matched with proponent’s report.

Summary

In JVET-Z0137 [1], it is asserted that the fixed value is utilized for the GPM blending width in the VVC spec and the current ECM design, but it is not always optimal for all video content.

To solve this problem, JVET-Z0137 proposes the adaptive width for GPM blending area by CU-level signalling. The selectable width is organized by total of 5 different values, which are 0 (non-blending), 1 (half of ECM/VCM), 2 (same as ECM/VTM), 4 (double of ECM/VTM), and 8 (4 times of ECM/VTM), respectively. The weighting precision of the GPM blending is extended from 8 to 32 in JVET-Z0137.

The source code provided by the proponents was carefully analysed and it is confirmed that the implementation matches the description in JVET-Y0137.

Simulations were conducted in the RA and LB configuration of the CTC for enhanced compression tool testing [2]. The cross-check results are provided in the accompanying excel documents. The cross checker could exactly reproduce the results provided by the proponent and reported in JVET-Z0137. It is noted that the provided software was compiled on 64-bit Ubuntu environment. The experiments were performed on the cluster PCs.

References

- H. Gao, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, and X. Wang, “Non-EE2: Adaptive Blending for GPM,” JVET-Z0137, by teleconference, 20–29 April 2022.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, by teleconference, 12-21, January. 2022.

Patent rights declaration(s)

KDDI Corporation does not have any current or pending patent rights relating to the technology described in this contribution.