| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0230-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0062 (AHG12: RPR luma filter modifications and RPR enabling fixes in ECM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Z. Zhang | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of crosschecking JVET-Z0062 proposed by Ericsson. JVET-Z0062 proposes to modify the up-sampling and down-sampling RPR luma filter in ECM-4.0. It proposes to use the existing 12-tap up-sampling RPR luma filters also for affine blocks. It proposes to increase the down-sampling RPR luma filter to 12 taps.

The update of the code had been reviewed and confirmed consistent with the proposal description.

The verification simulation was run for RA configuration with RPR enabled. The verification simulation results matched the results in JVET-Z0062 in terms of BD-rate.

# Verification

Ericsson had provided the software and results of their proposed methods in JVET-Z0062[1]. The code was updated according to the description in JVET-Z0062.

The simulation was conducted under the common test conditions as defined in [2] with RPR enabled. The anchor is ECM-4.0 with RPR enabling fix in JVET-Z0062.

Two RPR configuration setting are tested:

| RPR setting 1.5x | RPR setting 2.0x |
| --- | --- |
| (Resolution switch after every 16 frames) | (Resolution switch after every 16 frames) |
| ScalingRatioHor : 1.5 | ScalingRatioHor : 2.0 |
| ScalingRatioVer : 1.5 | ScalingRatioVer : 2.0 |
| FractionNumFrames : 1.0 | FractionNumFrames : 1.0 |
| RPR : 1 | RPR : 1 |
| UpscaledOutput : 2 | UpscaledOutput : 2 |
| SwitchPocPeriod : 16 | SwitchPocPeriod : 16 |

The BD-rate results match the results reported in JVET-Z0062. The simulation results for random access configuration are as follows:

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 with RPR enabled |  |  |  |  |  |  |  |  |  |
|  | RPR setting 1.5x |  |  |  |  | RPR setting 2.0x |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.01% | -0.01% | 99% | 100% | -0.05% | 0.07% | -0.05% | 99% | 100% |
| Class A2 | -0.18% | -0.16% | -0.13% | 99% | 100% | -0.09% | -0.01% | 0.11% | 100% | 100% |
| Class B | -0.30% | -0.25% | -0.20% | 100% | 100% | -0.11% | -0.06% | 0.00% | 99% | 100% |
| Class C | -0.37% | -0.34% | -0.23% | 99% | 100% | -0.18% | -0.02% | -0.12% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.25% | -0.21% | -0.16% | 99% | 100% | -0.11% | -0.01% | -0.02% | 100% | 100% |
| Class D | -0.40% | -0.02% | 0.04% | 99% | 100% | -0.21% | 0.09% | 0.21% | 100% | 100% |

# Reference

[1] R. Yu, K. Andersson (Ericsson), “AHG12: RPR luma filter modifications and RPR enabling fixes in ECM”, JVET-Z0062, Apr, 2022

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).