| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0232-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0067 (Non-EE2: RPR bug fixes and new results of RPR with luma-only re-scaling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Z. Zhang | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of crosschecking JVET-Z0067 proposed by InterDigital. JVET-Z0067 proposes to increase the flexibility of RPR by allowing luma only down-scaling. The luma down-scaling factor is limited to 2 for both horizontal and vertical direction. The chroma format of the reconstructed picture is 4:4:4 when luma only down-scaling is applied. It also proposes to disallow a CU partition with max CU size when luma only down-scaling is applied to avoid the complexity increase comparing to no-RPR picture with chroma format 4:2:0.

The update of the code had been reviewed and confirmed consistent with the proposal description.

The verification simulation was run for LDB configuration with RPR enabled and scaling factor 2.0x. The verification simulation results matched the results in JVET-Z0067 in terms of BD-rate.

# Verification

InterDigital had provided the software and results of their proposed methods in JVET-Z0067[1]. The code was updated according to the description in JVET-Z0067.

The simulation was conducted under the common test conditions as defined in [2] with RPR enabled. The anchor is ECM-4.0 with RPR enabling fix of merge request 105 [3]. This crosscheck reports the results of luma-only RPR with CU size constraint in JVET-Z0067. The RPR configuration setting is as follows:

|  | RPR setting 2.0x |
| --- | --- |
|  | (Resolution switch after every 0.5 second) |
| Existing RPR configuration parameters | ScalingRatioHor : 2.0 |
|  | ScalingRatioVer : 2.0 |
|  | FractionNumFrames : 0.5 |
|  | RPR : 1 |
|  | UpscaledOutput : 2 |
|  | SwitchPocPeriod : 0 |
| Added RPR configuration parameters | rpr444 : 1 |
|  | RprBaseQpOffset : 0 |
|  | RprCbCrQpOffset : 10 |
|  | RprCbCrQpOffsetDualTree : 0 |

The BD-rate results match the results reported in JVET-Z0067. The simulation results for low delay B configuration are as follows:

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 with RPR enabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.36% | -10.44% | -7.25% | 101% | 102% |
| Class C | -0.74% | -8.62% | -3.31% | 100% | 100% |
| Class E | -0.13% | -14.45% | -6.49% | 101% | 102% |
| Overall | -0.43% | -10.84% | -5.75% | 101% | 101% |
| Class D | -0.50% | -5.67% | -7.27% | 100% | 100% |

# Reference

[1] P. Bordes, F. Galpin, H. Guermoud, E. François (InterDigital), “Non-EE2: RPR bug fixes and new results of RPR with luma-only re-scaling”, JVET-Z0067, Apr, 2022.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

[3] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/merge_requests/105

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).