| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0244-v1 |
| --- | --- |

| Title: | AHG9: NN post-filter SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Maria Santamaria, Francesco Cricri, Emre B. Aksu, Hamed. R. Tavakoli Nokia Technologies Tampere, Finland Takeshi Chujoh, Yukinobu Yasugi, Tomohiro Ikai Sharp Corporation Chiba, Japan S. McCarthy, A. Arora, T. Shao, P. Yin, T. Lu, F. Pu, W. Husak Dolby Laboratories, Inc. | Email: | firstname.lastname@nokia.com chujoh.takeshi@sharp.co.jp yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp {sean.mccarthy, pyin} @dolby.com |
| Source: | Nokia, Sharp, Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes neural-network (NN) post-filter supplemental enhancement information into a working draft of a VSEI amendment. This contribution merges aspects of JVET-Z0121 and JVET-Z0151 into JVET-Z0052.

# Introduction

Section 2 contains the proposal for an amendment of VSEI. Section 3 contains the proposed VVC specification text for interfacing with the NN post-filter SEI.

# VSEI proposal

## Overview

The base text without highlighting is the JVET-Z0052 text and the items of JVET-Z0151 that were agreed in the first JVET review, i.e., the following:

- non-square rectangular partitions

- integer input/output

- complexity information

The offline work among the proponents resulted into the following proposals categorized with highlighted specification text as follows:

- The yellow highlighting indicates changes in response to the first JVET review:

  - nnpfc_id was aligned with fp_arrangement_id as follows:

    - ue(v) and value ranges are used like in fp_arrangement_id.

    - The explicit phrasing “determined by the application” was excluded since in the NN post-filtering case, nnpfc_id is determined by the encoder.

  - The following changes were made related to nnpfc_mode_idc:

    - The nnpfc_mode_idc value to indicate a URI was excluded in response to the comments in the first JVET review: "The URI mechanism seems to require some more clarification about its usage and usefulness. The intended purpose could basically be achieved by sending user data via a proprietary SEI message. Referring to a specific URI might be more appropriate (if such a thing would exist). This could be done (if needed) in a future extension."

    - The nnpfc_mode_idc values for standalone NNR bitstream and NNR update were united into one value in response to the comments in the first JVET review: "It was commented that using different mode idc for NNR v1 and v2 may not be necessary and not desirable because this information can be extracted from the NNR stream, and would then also allow using future NNR extensions."

    - Instead of u(2), ue(v) was taken into use in response to the following comment in the JVET review: "It was commented that using more than 2 bits for mode_idc would be preferable for possible extensibility."

- The cyan highlighting indicates a proposal of two SEI messages, one for filter characteristics (with a scope until the end of CLVS) and another for picture-wise filter activation. It is asserted that this design enables to indicate the characteristics for several filters on CLVS basis. The picture-wise filter activation message enables selection of the filter on picture basis, without repeating the filter characteristics. The idea of CLVS-level filter characteristics SEI message and picture-level activation SEI message originates from JVET-Z0121.

If the proposal of two SEI messages is not acceptable, it is proposed to fall back to the Z0052 design on the SEI message with indicated persistence.

- The green highlighting propose the following items originating from JVET-Z0121:

  - Addition of a mode indicator specifying that the post-processing filter is determined by external means not specified in this Specification. This value can be used, for example, when there are systems means to indicate a post-processing filter.

  - Addition of "unknown or unspecified" purpose value.

  - Addition of a figure to illustrate interleaving of luma data channels of nnpfc_inp_order_idc equal to 3 (informative).

  - Addition of padding type syntax element and zero and reflection padding (in addition to the replication padding that was already present in JVET-Z0052).

  - The syntax elements nnpfc_parameter_type_flag and nnpfc_log2_parameter_bit_length_minus3 for the neural network complexity structure were added to merge aspects from both JVET-Z0151 and JVET-Z0121:

nnpfc_parameter_type_flag equal to 0 indicates that the neural network uses only integer parameters. nnpfc_parameter_type_flag equal to 1 indicates that the neural network may use floating point or integer parameters.

nnpfc_log2_parameter_bit_length_minus3 equal to 0, 1, 2, and 3 indicates that the neural network does not use parameters of bit length greater than 8, 16, 32, and 64, respectively.

## Neural-network post-filter characteristics SEI message

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | u(21)ue(v) |
| nnpfc_mode_idc | u(2)ue(v) |
| if( nnpfc_mode_idc > 0 ) { |  |
| nnpfc_persistence_flag | u(1) |
| if( nnpfc_mode_idc = = 1 ¦ ¦ nnpfc_mode_idc = = 2 ) { |  |
| nnpfc_purpose | ue(v) |
| if( nnpfc_purpose = = 12 ¦ ¦ nnpfc_purpose = = 34 ) { |  |
| nnpfc_out_sub_width_c_flag | u(1) |
| nnpfc_out_sub_height_c_flag | u(1) |
| } |  |
| if( nnpfc_purpose = = 23 ¦ ¦ nnpfc_purpose = = 34 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| /* input and output formatting */ |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_sample_idc | ue(v) |
| if( nnpfc_inp_sample_idc = = 4 ) |  |
| nnpfc_inp_tensor_bitdepth_minus8 | ue(v) |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_out_sample_idc | ue(v) |
| if( nnpfc_out_sample_idc = = 4 ) |  |
| nnpfc_out_tensor_bitdepth_minus8 | ue(v) |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_padding_type | ue(v) |
| nnpfc_complexity_idc | ue(v) |
| if( nnpfc_complexity_idc > 0 ) { |  |
| nnpfc_complexity_element( nnpfc_complexity_idc ) |  |
| } |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit | u(1) |
| } |  |
| if( nnpfc_mode_idc = = 1 ) { |  |
| i = 0 |  |
| do |  |
| nnpfc_uri[ i ] | b(8) |
| while( nnpfc_uri[ i++ ] != 0 ) |  |
| } |  |
| /* filter specified or udpated by 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 12 ¦ ¦ nnpfc_mode_idc = = 3 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |
| } |  |

| nnpfc_complexity_element( nnpfc_complexity_idc ) { |  |
| --- | --- |
| if( nnpfc_complexity_idc = = 1 ) { |  |
| nnpfc_parameter_type_flag | u(1) |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(8) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| } |  |
| } |  |

This SEI message either specifies a neural network that may be used as a post-processing filter or cancels the persistence of the previous post-processing filter with the same identifier value (i.e., the same value of nnpfc_id).

This SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

Use of this SEI message requires the definition of the following variables:

–	Cropped decoded output picture width and height in units of luma samples, denoted herein by InpPicWidthInLumaSamples and InpPicHeightInLumaSamples, respectively.

–	Luma sample array CroppedYPic[ y ][ x ] and chroma sample arrays CroppedCbPic[ y ][ x ] and CroppedCrPic[ y ][ x ], when present, of the cropped decoded output picture for vertical coordinates y and horizontal coordinates x, where the top-left corner of the sample array has coordinates y equal to 0 and x equal to 0.

–	Bit depth BitDepthY for the luma sample array of the cropped decoded output picture.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the cropped decoded output picture.

–	Chroma subsampling ratio relative to luma denoted as InpSubWidthC and InpSubHeightC.

–	When nnpfc_inp_order_idc is equal to 3, the initial luma quantization parameter value SliceQPY.

When this SEI message specifies a neural network that may be used as a post-processing filter, the semantics specify the derivation of the luma sample array FilteredYPic[ y ][ x ] and chroma sample arrays FilteredCbPic[ y ][ x ] and FilteredCrPic[ y ][ x ], as indicated by the value of nnpfc_out_order_idc, that contain the output of the post-processing filter.

nnpfc_id contains an identifying number that may be used to identify a post-processing filter. The value of nnpfc_id shall be in the range of 0 to 232 − 2, inclusive.

Values of nnpfc_id from 0 to 220 − 1, inclusive, may be used as determined by the application. Values of nnpfc_id from 220 to 221 − 1, inclusive, are reserved for future use by ITU-T | ISO/IEC.

Values of nnpfc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

nnpfc_mode_idc equal to 0 indicates that no post-processing filter associated with the nnpfc_id value is specified in this SEI message and that the persistence of the previous post-processing filter with the same value of nnpfc_id is cancelled. nnpfc_mode_idc equal to 1 specifies that the post-processing filter associated with the nnpfc_id value is a neural network identified by a given Uniform Resource Identifier (URI), with syntax and semantics as specified in IETF Internet Standard 66. nnpfc_mode_idc equal to 2 specifies that the post-processing filter associated with the nnpfc_id value is a neural network represented by the ISO/IEC 15938-17 bitstream with general_profile_idc equal to 0 contained in this SEI message. nnpfc_mode_idc equal to 3 specifies that the post-processing filter is the neural network identified by the previous NNR post-filter SEI message, in decoding order, with nnpfc_mode_idc equal to 1 or 2 and updated by the ISO/IEC 15938-17 bitstream with general_profile_idc equal to 1 contained in this SEI message.

nnpfc_mode_idc equal to 0 specifies that the post-processing filter associated with the nnpfc_id value is determined by external means not specified in this Specification.

nnpfc_mode_idc equal to 1 specifies that the post-processing filter associated with the nnpfc_id value is a neural network represented by the ISO/IEC 15938-17 bitstream contained in this SEI message.

The value of nnpfc_mode_idc shall be in the range of 0 to 255, inclusive. Values of nnpfc_mode_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_mode_idc.

When the current CLVS contains a preceding neural-network post-filter characteristics SEI message, in decoding order, that has the same value of nnpfc_id equal to the value of nnpfc_id in this SEI message, at least one of the following conditions shall apply:

–	This SEI message has nnpfc_mode_idc equal to 1 and the same content as the preceding neural-network post-filter characteristics SEI message except that the values of nnpfc_mode_idc and nnpfc_payload_byte[ i ] may differ in order to provide a neural network update.

–	This SEI message has the same content as the preceding neural-network post-filter characteristics SEI message.

nnpfc_persistence_flag specifies the persistence of this SEI message for the current layer.

nnpfc_persistence_flag equal to 0 specifies that this SEI message applies to the current decoded picture only.

nnpfc_persistence_flag equal to 1 specifies that this SEI message applies to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	This SEI message has nnpfc_mode_idc equal to 1 or 2 and a picture that follows the current picture in output order and is associated with a NNR post-filter SEI message with the same nnpfc_id value as that in this SEI message and nnpfc_mode_idc value equal to 0, 1, or 2 is output.

–	This SEI message has nnpfc_mode_idc equal to 3 and a picture that follows the current picture in output order and is associated with a NNR post-filter SEI message with the same nnpfc_id value as that in this SEI message and any nnpfc_mode_idc value is output.

When this SEI message is the first neural-network post-filter characteristics SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, it specifies a base post-processing filter that pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS. When this SEI message is not the first neural-network post-filter SEI message, in decoding order, that has a a particular nnpfc_id value within the current CLVS, this SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or the next neural-network post-filter characteristics SEI message having that particular nnpfc_id value, in output order, within the current CLVS.

nnpfc_purpose indicates the purpose of post-processing filter as specified in Table XYZ1. The value of nnpfc_purpose shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_purpose that do not appear in Table XYZ1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_purpose.

Table XYZ1 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Unknown or unspecified |
| 01 | Visual quality improvement |
| 12 | Chroma upsampling from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format |
| 23 | Increasing the width or height of the cropped decoded output picture without changing the chroma format |
| 34 | Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |

NOTE 1 – When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

nnpfc_out_sub_width_c_flag and nnpfc_out_sub_height_c_flag are used to derive the variables outSubWidthC and outSubHeightC, respectively, which specify the chroma subsampling ratio relative to luma in the picture resulting from the post-processing filtering. When not present, nnpfc_out_sub_width_c_flag and nnpfc_out_sub_height_c_flag are both inferred to be equal to 0. When nnpfc_out_sub_width_c_flag and nnpfc_out_sub_height_c_flag are present, the sum of nnpfc_out_sub_width_c_flag and nnpfc_out_sub_height_c_flag shall be greater than 0.

outSubWidthC = InpSubWidthC − nnpfc_out_sub_width_c_flag	(1)

outSubHeightC = InpSubHeightC − nnpfc_out_sub_height_c_flag	(2)

It is a requirement of bitstream conformance that outSubWidthC and outSubHeightC are both greater than 0.

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting by applying the post-processing filter identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to InpPicWidthInLumaSamples and InpPicHeightInLumaSamples, respectively.

nnpfc_component_last_flag equal to 0 specifies that the second dimension in the input tensor inputTensor to the post-processing filter and the output tensor outputTensor resulting from the post-processing filter is used for the channel. nnpfc_component_last_flag equal to 1 specifies that the last dimension in the input tensor inputTensor to the post-processing filter and the output tensor outputTensor resulting from the post-processing filter is used for the channel.

NOTE 2 – The specified values of nnpfc_inp_sample_idc and nnpfc_out_sample_idc specify that the first dimension in the input tensor and in the output tensor, respectively, is used for the batch index, which is a practice in some neural network frameworks. While the semantics of this SEI message use batch size equal to 1, it is up to the post-processing implementation to determine the batch size used as input to the neural network inference.

NOTE 3 – A colour component is an example of a channel.

nnpfc_inp_sample_idc indicates the method of converting a sample value of the cropped decoded output picture to an input value to the post-processing filter. When nnpfc_inp_sample_idc is equal to 0, 1, 2, or 3, the input values to the post-processing filter are binary16, binary32, binary64, or binary128 floating point values, respectively, as specified in IEEE 754-2019, and the functions InpY and InpC are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthY ) − 1 )		(3)

InpC( x )= x ÷ ( ( 1  <<  BitDepthC ) − 1 )		(4)

When nnpfc_inp_sample_idc is equal to 4, the input values to the post-processing filter are unsigned integer and the functions InpY and InpC are specified as follows:

if inpTensorBitDepth >= BitDepthY,

InpY( x ) = x  <<  ( inpTensorBitDepth − BitDepthY )	(5)

Otherwise,

InpY( x ) = Clip3(0, ( 1  <<  inpTensorBitDepth ) − 1, ( x + ( 1  <<  ( shift − 1 ) ) )  >>  shift )	(6)
where shift = BitDepthY − inpTensorBitDepth

if inpTensorBitDepth >= BitDepthC,

InpC( x ) = x  <<  ( inpTensorBitDepth − BitDepthC )	(7)

Otherwise,

InpC( x ) = Clip3(0, ( 1  <<  inpTensorBitDepth ) − 1, ( x + ( 1  <<  ( shift − 1 ) ) )  >>  shift )	(8)
where shift = BitDepthC − inpTensorBitDepth

The variables inpTensorBitDepth is derived from the syntax elements nnpfc_inp_tensor_bitdepth_minus8 as specified below.

The value of nnpfc_inp_sample_idc shall be in the range of 0 to 255, inclusive. Values of nnpfc_inp_sample_idc greater than 4 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_inp_sample_idc.

nnpfc_inp_tensor_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the input integer tensor. The value of inpTensorBitDepth is derived as follows:

inpTensorBitDepth = nnpfc_inp_tensor_bitdepth_minus8 + 8	(9)

It is a requirement of bitstream conformance that the value of nnpfc_inp_tensor_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_inp_order_idc indicates the method of ordering the sample arrays of a cropped decoded output picture as the input to the post-processing filter. Table XYZ2A contains an informative description of nnpfc_inp_order_idc values. The semantics of nnpfc_inp_order_idc in the range of 0 to 3, inclusive, are specified in Table XYZ2B, which specifies a process for deriving the input tensors inputTensor for different values of nnpfc_inp_order_idc and a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensors. When the chroma format of the cropped decoded output picture is not 4:2:0, nnpfc_inp_order_idc shall not be equal to 3. The value of nnpfc_inp_order_idc shall be in the range of 0 to 255, inclusive. Values of nnpfc_inp_order_idc greater than 3 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_inp_order_idc.

Table XYZ2A – Informative description of nnpfc_inp_order_idc values

| nnpfc_inp_ order_idc | Description |
| --- | --- |
| 0 | Only the luma matrix is present in the input tensor, thus the number of channels is 1. |
| 1 | Only the chroma matrices are present in the input tensor, thus the number of channels is 2. |
| 2 | The luma and chroma matrices are present in the input tensor, thus the number of channels is 3. |
| 3 | Four luma matrices, two chroma matrices, and a quantization parameter matrix are present in the input tensor, thus the number of channels is 7. The luma channels are derived in an interleaved manner as illustrated in Figure XYF1. This nnpfc_inp_order_idc can only be used when the chroma format is 4:2:0. |
| 4..255 | reserved |

Figure XYF1 – Illustration of luma data channels of nnpfc_inp_order_idc equal to 3 (informative)

A patch is a rectangular array of samples from a component (e.g., a luma or chroma component) of a picture.

nnpfc_constant_patch_size_flag equal to 0 specifies that the post-processing filter accepts any patch size that is a positive integer multiple of the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. nnpfc_constant_patch_size_flag equal to 1 specifies that the post-processing filter accepts exactly the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input.

nnpfc_patch_width_minus1 + 1, when nnpfc_constant_patch_size_flag equal to 1, specifies the horizontal sample counts of the patch size required for the input to the post-processing filter. When nnpfc_constant_patch_size_flag is equal to 0, any positive integer multiple of ( nnpfc_patch_width_minus1 + 1 ) may be used as the horizontal sample counts of the patch size used for the input to the post-processing filter. The value of nnpfc_patch_width_minus1 shall be in the range of 0 to 32766, inclusive.

nnpfc_patch_height_minus1 + 1, when nnpfc_constant_patch_size_flag equal to 1, specifies the vertical sample counts of the patch size required for the input to the post-processing filter. When nnpfc_constant_patch_size_flag is equal to 0, any positive integer multiple of ( nnpfc_patch_height_minus1 + 1 ) may be used as the vertical sample counts of the patch size used for the input to the post-processing filter. The value of nnpfc_patch_height_minus1 shall be in the range of 0 to 32766, inclusive.

nnpfc_overlap specifies the overlapping horizontal and vertical sample counts of adjacent input tensors of the post-processing filter. The value of nnpfc_overlap shall be in the range of 0 to 16383, inclusive.

The variables inpPatchWidth, inpPatchHeight, outPatchWidth, outPatchHeight, horCScaling, verCScaling, outPatchCWidth, outPatchCHeight, and overlapSize are derived as follows:

inpPatchWidth = nnpfc_patch_width_minus1 + 1
inpPatchHeight = nnpfc_patch_height_minus1 + 1
outPatchWidth = ( nnpfc_pic_width_in_luma_samples * inpPatchWidth ) / InpPicWidthInLumaSamples
outPatchHeight = ( nnpfc_pic_height_in_luma_samples * inpPatchHeight ) / InpPicHeightInLumaSamples
horCScaling = InpSubWidthC / outSubWidthC
verCScaling = InpSubHeightC / outSubHeightC						(5)
outPatchCWidth = outPatchWidth * horCScaling
outPatchCHeight = outPatchHeight * verCScaling
overlapSize = nnpfc_overlap

It is a requirement of bitstream conformance that outPatchWidth * InpPicWidthInLumaSamples shall be equal to nnpfc_pic_width_in_luma_samples * inpPatchWidth and outPatchHeight * InpPicHeightInLumaSamples shall be equal to nnpfc_pic_height_in_luma_samples * inpPatchHeight.

nnpfc_padding_type specifies the process of padding when referencing sample locations outside the boundaries of the cropped decoded output picture as described in Table XYZ5. The value of nnpfc_padding_type shall be in the range of 0 to 15, inclusive.

Table XYZ5 – Informative description of nnpfc_padding_type values

| nnpfc_padding_type | Description |
| --- | --- |
| 0 | zero padding |
| 1 | replication padding |
| 2 | reflection padding |
| 3..15 | reserved |

The function Reflect( y, z ) is specified as follows:

Reflect( y, z ) =  Min( -z, y );z < 0Max( y-(z-y), 0 );z > yz;otherwise	(6)

The function InpSampleVal( y, x, picHeight, picWidth, croppedPic ) with inputs being a vertical sample location y, a horizontal sample location x, a picture height picHeight, a picture width picWidth, and sample array croppedPic returns the value of sampleVal derived as follows:

if( nnpfc_padding_type  = =  0 )
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x >= picWidth )
		sampleVal = 0
	else
		sampleVal = croppedPic[ y ][ x ] 		(7)
else if( nnpfc_padding_type  = =  1 )
	sampleVal = croppedPic[ Clip3( 0, picHeight − 1, y ) ][ Clip3( 0, picWidth − 1, x ) ]
else /* nnpfc_padding_type  = =  2 */
	sampleVal = croppedPic[ Reflect( picHeight − 1, y ) ][ Reflect( picWidth − 1, x ) ]

Table XYZ2B – Process for deriving the input tensors inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensors

| nnpfc_inp_ order_idc | Process DeriveInputTensors( ) for deriving input tensors |
| --- | --- |
| 0 | for ( yP = –overlapSize; yP < inpPatchHeight + overlapSize; yP++) for ( xP = –overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { y = Clip3( 0, InpPicHeightInLumaSamples – 1, cTop + yP ) x = Clip3( 0, InpPicWidthtInLumaSamples – 1, cLeft + xP ) if( nnpfc_component_last_flag = = 0 ) inputTensor[ 0 ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = InpY( CroppedYPic[ y ][ x ] ) else inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = InpY( CroppedYPic[ y ][ x ] ) } |
| 0 | for ( yP = –overlapSize; yP < inpPatchHeight + overlapSize; yP++) for ( xP = –overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { inpVal = InpY( InpSampleVal( cTop + yP, cLeft + xP, InpPicHeightInLumaSamples, InpPicWidthtInLumaSamples, CroppedYPic ) ) if( nnpfc_component_last_flag = = 0 ) inputTensor[ 0 ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = inpVal ) else inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = inpVal ) } |
| 1 | for ( yP = –overlapSize; yP < inpPatchHeight + overlapSize; yP++) for ( xP = –overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { y = Clip3( 0, InpPicHeightInLumaSamples / InpSubHeightC – 1, cTop + yP ) x = Clip3( 0, InpPicWidthtInLumaSamples / InpSubWidthC – 1, cLeft + xP ) if( nnpfc_component_last_flag = = 0 ) { inputTensor[ 0 ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = InpC( CroppedCbPic[ y ][ x ] ) inputTensor[ 0 ][ 1 ][ yP + overlapSize ][ xP + overlapSize ] = InpC( CroppedCrPic[ y ][ x ] ) } else { inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = InpC( CroppedCbPic[ y ][ x ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 1 ] = InpC( CroppedCrPic[ y ][ x ] ) } } |
| 1 | for ( yP = –overlapSize; yP < inpPatchHeight + overlapSize; yP++) for ( xP = –overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { inpCbVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, InpPicHeightInLumaSamples / InpSubHeightC, InpPicWidthtInLumaSamples / InpSubWidthC, CroppedCbPic ) ) inpCrVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, InpPicHeightInLumaSamples / InpSubHeightC, InpPicWidthtInLumaSamples / InpSubWidthC, CroppedCrPic ) ) if( nnpfc_component_last_flag = = 0 ) { inputTensor[ 0 ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = inpCbVal inputTensor[ 0 ][ 1 ][ yP + overlapSize ][ xP + overlapSize ] = inpCrVal } else { inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = inpCbVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 1 ] = inpCrVal } } |
| 2 | for ( yP = –overlapSize; yP < inpPatchHeight + overlapSize; yP++) for ( xP = –overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { yY = Clip3( 0, InpPicHeightInLumaSamples – 1, cTop + yP ) xY = Clip3( 0, InpPicWidthtInLumaSamples – 1, cLeft + xP ) yC = yY / InpSubHeightC xC = xY / InpSubWidthC if( nnpfc_component_last_flag = = 0 ) { inputTensor[ 0 ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = InpY( CroppedYPic[ yY ][ xY ] ) inputTensor[ 0 ][ 1 ][ yP + overlapSize ][ xP + overlapSize ] = InpC( CroppedCbPic[ yC ][ xC ] ) inputTensor[ 0 ][ 2 ][ yP + overlapSize ][ xP + overlapSize ] = InpC( CroppedCrPic[ yC ][ xC ] ) } else { inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = InpY( CroppedYPic[ yY ][ xY ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 1 ] = InpC( CroppedCbPic[ yC ][ xC ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 2 ] = InpC( CroppedCrPic[ yC ][ xC ] ) } } |
| 2 | for ( yP = –overlapSize; yP < inpPatchHeight + overlapSize; yP++) for ( xP = –overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { yY = cTop + yP xY = cLeft + xP yC = yY / InpSubHeightC xC = xY / InpSubWidthC inpYVal = InpY( InpSampleVal( yY, xY, InpPicHeightInLumaSamples, InpPicWidthtInLumaSamples, CroppedYPic ) ) inpCbVal = InpC( InpSampleVal( yC, xC, InpPicHeightInLumaSamples / InpSubHeightC, InpPicWidthtInLumaSamples / InpSubWidthC, CroppedCbPic ) ) inpCrVal = InpC( InpSampleVal( yC, xC, InpPicHeightInLumaSamples / InpSubHeightC, InpPicWidthtInLumaSamples / InpSubWidthC, CroppedCrPic ) ) if( nnpfc_component_last_flag = = 0 ) { inputTensor[ 0 ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = inpYVal ) inputTensor[ 0 ][ 1 ][ yP + overlapSize ][ xP + overlapSize ] = inpCbVal ) inputTensor[ 0 ][ 2 ][ yP + overlapSize ][ xP + overlapSize ] = inpCrVal ) } else { inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = inpYVal ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 1 ] = inpCbVal ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 2 ] = inpCrVal ) } } |
| 3 | for ( yP = –overlapSize; yP < inpPatchHeight + overlapSize; yP++) for ( xP = –overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { yTL = Clip3( 0, InpPicHeightInLumaSamples – 1, cTop + yP * 2 ) xTL = Clip3( 0, InpPicWidthtInLumaSamples – 1, cLeft + xP * 2 ) yBR = Clip3( 0, InpPicHeightInLumaSamples – 1, yTL + 1 ) xBR = Clip3( 0, InpPicWidthtInLumaSamples – 1, xTL + 1 ) y = Clip3( 0, InpPicHeightInLumaSamples / 2 – 1, cTop / 2 + yP) x = Clip3( 0, InpPicWidthtInLumaSamples / 2 – 1, cLeft / 2 + xP) if( nnpfc_component_last_flag = = 0 ) { inputTensor[ 0 ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = InpY( CroppedYPic[ yTL ][ xTL ] ) inputTensor[ 0 ][ 1 ][ yP + overlapSize ][ xP + overlapSize ] = InpY( CroppedYPic[ yTL ][ xBR ] ) inputTensor[ 0 ][ 2 ][ yP + overlapSize ][ xP + overlapSize ] = InpY( CroppedYPic[ yBR ][ xTL ] ) inputTensor[ 0 ][ 3 ][ yP + overlapSize ][ xP + overlapSize ] = InpY( CroppedYPic[ yBR ][ xBR ] ) inputTensor[ 0 ][ 4 ][ yP + overlapSize ][ xP + overlapSize ] = InpC( CroppedCbPic[ y ][ x ] ) inputTensor[ 0 ][ 5 ][ yP + overlapSize ][ xP + overlapSize ] = InpC( CroppedCrPic[ y ][ x ] ) inputTensor[ 0 ][ 6 ][ yP + overlapSize ][ xP + overlapSize ] = 2(SliceQPY – 42)/6 } else { inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = InpY( CroppedYPic[ yTL ][ xTL ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 1 ] = InpY( CroppedYPic[ yTL ][ xBR ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 2 ] = InpY( CroppedYPic[ yBR ][ xTL ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 3 ] = InpY( CroppedYPic[ yBR ][ xBR ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 4 ] = InpC( CroppedCbPic[ y ][ x ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 5 ] = InpC( CroppedCrPic[ y ][ x ] ) inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 6 ] = 2(SliceQPY – 42)/6 } } |
| 3 | for ( yP = –overlapSize; yP < inpPatchHeight + overlapSize; yP++) for ( xP = –overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) { yTL = cTop + yP * 2 xTL = cLeft + xP * 2 yBR = yTL + 1 xBR = xTL + 1 yC = cTop / 2 + yP xC = cLeft / 2 + xP inpTLVal = InpY( InpSampleVal( yTL, xTL, InpPicHeightInLumaSamples, InpPicWidthtInLumaSamples, CroppedYPic ) ) inpTRVal = InpY( InpSampleVal( yTL, xBR, InpPicHeightInLumaSamples, InpPicWidthtInLumaSamples, CroppedYPic ) ) inpBLVal = InpY( InpSampleVal( yBR, xTL, InpPicHeightInLumaSamples, InpPicWidthtInLumaSamples, CroppedYPic ) ) inpBRVal = InpY( InpSampleVal( yBR, xBR, InpPicHeightInLumaSamples, InpPicWidthtInLumaSamples, CroppedYPic ) ) inpCbVal = InpC( InpSampleVal( yC, xC, InpPicHeightInLumaSamples / 2, InpPicWidthtInLumaSamples / 2, CroppedCbPic ) ) inpCrVal = InpC( InpSampleVal( yC, xC, InpPicHeightInLumaSamples / 2, InpPicWidthtInLumaSamples / 2, CroppedCrPic ) ) if( nnpfc_component_last_flag = = 0 ) { inputTensor[ 0 ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = inpTLVal inputTensor[ 0 ][ 1 ][ yP + overlapSize ][ xP + overlapSize ] = inpTRVal inputTensor[ 0 ][ 2 ][ yP + overlapSize ][ xP + overlapSize ] = inpBLVal inputTensor[ 0 ][ 3 ][ yP + overlapSize ][ xP + overlapSize ] = inpBRVal inputTensor[ 0 ][ 4 ][ yP + overlapSize ][ xP + overlapSize ] = inpCbVal inputTensor[ 0 ][ 5 ][ yP + overlapSize ][ xP + overlapSize ] = inpCrVal inputTensor[ 0 ][ 6 ][ yP + overlapSize ][ xP + overlapSize ] = 2(SliceQPY – 42)/6 } else { inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = inpTLVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 1 ] = inpTRVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 2 ] = inpBLVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 3 ] = inpBRVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 4 ] = inpCbVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 5 ] = inpCrVal inputTensor[ 0 ][ yP + overlapSize ][ xP + overlapSize ][ 6 ] = 2(SliceQPY – 42)/6 } } |
| 4..255 | reserved |

nnpfc_out_sample_idc equal to 0, 1, 2, or 3 indicates that the sample values output by the post-processing filter are binary16, binary32, binary64, or binary128 floating point values, respectively, as specified in IEEE 754-2019. Functions OutY and OutC for converting luma sample values and chroma sample values, respectively, output by the post-processing, to integer values at bit depths BitDepthY and BitDepthC, respectively, are specified as follows:

OutY( x ) = Clip3( 0, ( 1  <<  BitDepthY ) − 1, Round( x * ( ( 1  <<  BitDepthY ) − 1 ) ) )	(6)

OutC( x )= Clip3( 0, ( 1  <<  BitDepthC ) − 1, Round( x * ( ( 1  <<  BitDepthC ) − 1 ) ) )	(7)

nnpfc_out_sample_idc equal to 4 indicates that the sample values output by the post-processing filter are unsigned integer. Functions OutY and OutC are specified as follows:

if outTensorBitDepth >= BitDepthY

OutY( x ) = Clip3( 0, ( 1  <<  BitDepthY ) − 1, ( x + ( 1  <<  ( shift − 1 ) ) )  >>  shift )	(15)
where shift = outTensorBitDepth − BitDepthY

Otherwise,

OutY( x ) = x  <<  ( BitDepthY − outTensorBitDepth )	(16)

if outTensorBitDepth >= BitDepthC,

OutC( x )= Clip3( 0, ( 1  <<  BitDepthC ) − 1, ( x + ( 1  <<  ( shift − 1 ) ) )  >>  shift )	(17)
where shift = outTensorBitDepth − BitDepthC

Otherwise,

OutC( x ) = x  <<  ( BitDepthC − outTensorBitDepth )	(18)

The variables outTensorBitDepth is derived from the syntax elements nnpfc_out_tensor_bitdepth_minus8 as described below.

The value of nnpfc_out_sample_idc shall be in the range of 0 to 255, inclusive. Values of nnpfc_out_sample_idc greater than 4 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_out_sample_idc.

nnpfc_out_tensor_bitdepth_minus8 plus 8 specifies the bit depth of sample values in the output integer tensor. The value of outTensorBitDepth is derived as follows:

outTensorBitDepth = nnpfc_out_tensor_bitdepth_minus8 + 8	(19)

It is a requirement of bitstream conformance that the value of nnpfc_out_tensor_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_out_order_idc indicates the output order of samples resulting from the post-processing filter. Table XYZ3A contains an informative description of nnpfc_out_order_idc values. The semantics of nnpfc_out_order_idc in the range of 0 to 3, inclusive, are specified in Table XYZ3B, which specifies a process for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensors outputTensor for different values of nnpfc_out_order_idc and a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensors. When nnpfc_purpose is equal to 2 or 4, nnpfc_out_order_idc shall not be equal to 3. The value of nnpfc_out_order_idc shall be in the range of 0 to 255, inclusive. Values of nnpfc_out_order_idc greater than 3 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_out_order_idc.

Table XYZ3A – Informative description of nnpfc_out_order_idc values

| nnpfc_out_ order_idc | Description |
| --- | --- |
| 0 | Only the luma matrix is present in the output tensor, thus the number of channels is 1. |
| 1 | Only the chroma matrices are present in the output tensor, thus the number of channels is 2. |
| 2 | The luma and chroma matrices are present in the output tensor, thus the number of channels is 3. |
| 3 | Four luma matrices and two chroma matrices are present in the output tensor, thus the number of channels is 6. This nnpfc_out_order_idc can only be used when the chroma format is 4:2:0. |
| 4..255 | reserved |

Table XYZ3B – Process for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensors outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensors

| nnpfc_out_ order_idc | Process StoreOutputTensors( ) for deriving sample values in the filtered picture from the output tensors |
| --- | --- |
| 0 | for ( yP = 0; yP < outPatchHeight; yP++) for ( xP = 0; xP < outPatchWidth; xP++ ) yY = cTop * outPatchHeight / inpPatchHeight + yP xY = cLeft * outPatchWidth / inpPatchWidth + xP if ( yY < nnpfc_pic_height_in_luma_samples && xY < nnpfc_pic_width_in_luma_samples ) if( nnpfc_component_last_flag = = 0 ) FilteredYPic[yY ][ xY ] = OutY( outputTensor[ 0 ][ 0 ][ yP ][ xP ] ) else FilteredYPic[ yY ][ xY ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 0 ] ) |
| 1 | for ( yP = 0; yP < outPatchCHeight; yP++) for ( xP = 0; xP < outPatchCWidth; xP++ ) { xSrc = cLeft * horCScaling + xP ySrc = cTop * verCScaling + yP if ( ySrc < nnpfc_pic_height_in_luma_samples / outSubHeightC && xSrc < nnpfc_pic_width_in_luma_samples / outSubWidthC ) if( nnpfc_component_last_flag = = 0 ) { FilteredCbPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ 0 ][ yP ][ xP ] ) FilteredCrPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ 1 ][ yP ][ xP ] ) } else { FilteredCbPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ 0 ] ) FilteredCrPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ 1 ] ) } } |
| 2 | for ( yP = 0; yP < outPatchHeight; yP++) for ( xP = 0; xP < outPatchWidth; xP++ ) { yY = cTop * outPatchHeight / inpPatchHeight + yP xY = cLeft * outPatchWidth / inpPatchWidth + xP yC = yY / outSubHeightC xC = xY / outSubWidthC yPc = ( yP / outSubHeightC ) * outSubHeightC xPc = ( xP / outSubWidthC ) * outSubWidthC if ( yY < nnpfc_pic_height_in_luma_samples && xY < nnpfc_pic_width_in_luma_samples) if( nnpfc_component_last_flag = = 0 ) { FilteredYPic[ yY ][ xY ] = OutY( outputTensor[ 0 ][ 0 ][ yP ][ xP ] ) FilteredCbPic[ yC ][ xC ] = OutC( outputTensor[ 0 ][ 1 ][ yPc ][ xPc ] ) FilteredCrPic[ yC ][ xC ] = OutC( outputTensor[ 0 ][ 2 ][ yPc ][ xPc ] ) } else { FilteredYPic[ yY ][ xY ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 0 ] ) FilteredCbPic[ yC ][ xC ] = OutC( outputTensor[ 0 ][ yPc ][ xPc ][ 1 ] ) FilteredCrPic[ yC ][ xC ] = OutC( outputTensor[ 0 ][ yPc ][ xPc ][ 2 ] ) } } |
| 3 | for( yP = 0; yP < outPatchHeight; yP++ ) for( xP = 0; xP < outPatchWidth; xP++ ) { ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP if ( ySrc < nnpfc_pic_height_in_luma_samples / 2 && xSrc < nnpfc_pic_width_in_luma_samples / 2 ) if( nnpfc_component_last_flag = = 0 ) { FilteredYPic[ ySrc * 2 ][ xSrc * 2 ] = OutY( outputTensor[ 0 ][ 0 ][ yP ][ xP ] ) FilteredYPic[ ySrc * 2 ][ xSrc * 2 + 1 ] = OutY( outputTensor[ 0 ][ 1 ][ yP ][ xP ] ) FilteredYPic[ ySrc * 2 + 1 ][ xSrc * 2 ] = OutY( outputTensor[ 0 ][ 2 ][ yP ][ xP ] ) FilteredYPic[ ySrc * 2 + 1 ][ xSrc * 2 + 1] = OutY( outputTensor[ 0 ][ 3 ][ yP ][ xP ] ) FilteredCbPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ 4 ][ yP ][ xP ] ) FilteredCrPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ 5 ][ yP ][ xP ] ) } else { FilteredYPic[ ySrc * 2 ][ xSrc * 2 ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 0 ] ) FilteredYPic[ ySrc * 2 ][ xSrc * 2 + 1 ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 1 ] ) FilteredYPic[ ySrc * 2 + 1 ][ xSrc * 2 ] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 2 ] ) FilteredYPic[ ySrc * 2 + 1 ][ xSrc * 2 + 1] = OutY( outputTensor[ 0 ][ yP ][ xP ][ 3 ] ) FilteredCbPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ 4 ] ) FilteredCrPic[ ySrc ][ xSrc ] = OutC( outputTensor[ 0 ][ yP ][ xP ][ 5 ] ) } } |
| 4..255 | reserved |

A base post-processing filter for a cropped decoded output picture picA is the filter that is identified by the NNR post-filter SEI message that has a particular nnpfc_id value, has nnpfc_mode_idc equal to 1 or 2, and pertains to the picture picA. A base post-processing filter for a cropped decoded output picture picA is the filter that is identified by the first NNR post-filter SEI message, in decoding order, that has a particular nnpfc_id value within a CLVS.

If there is another NNR post-filter SEI message that has the same nnpfc_id value, has nnpfc_mode_idc equal to 3, and pertains to the picture picA, the base post-processing filter is updated by decoding the ISO/IEC 15938-17 bitstream in that NNR post-filter SEI message to obtain a post-processing filter PostProcessingFilter( ). If there is another neural-network post-filter characteristics SEI message that has the same nnpfc_id value, has nnpfc_mode_idc equal to 1, has different content than the neural-network post-filter characteristics SEI message that defines the base post-processing filter, and pertains to the picture picA, the base post-processing filter is updated by decoding the ISO/IEC 15938-17 bitstream in that neural-network post-filter characteristics SEI message to obtain a post-processing filter PostProcessingFilter( ). Otherwise, the post-processing processing filter PostProcessingFilter( ) is assigned to be the same as the base post-processing filter.

The following process is used to filter the cropped decoded output picture with the post-processing filter PostProcessingFilter( ) to generate the filtered picture, which contains Y, Cb, and Cr sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, respectively, as indicated by nnpfc_out_order_idc.

if( nnpfc_inp_order_idc  = = 0 )
	for( cTop = 0; cTop < InpPicHeightInLumaSamples; cTop += inpPatchHeight )
		for( cLeft = 0; cLeft < InpPicWidthInLumaSamples; cLeft += inpPatchWidth ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}

else if( nnpfc_inp_order_idc  = = 1 )
	for( cTop = 0; cTop < InpPicHeightInLumaSamples / InpSubHeightC; cTop += inpPatchHeight )
		for( cLeft = 0; cLeft < InpPicWidthInLumaSamples / InpSubWidthC; cLeft += inpPatchWidth ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}

else if( nnpfc_inp_order_idc  = = 2 ) {
	for( cTop = 0; cTop < InpPicHeightInLumaSamples; cTop += inpPatchHeight)
		for( cLeft = 0; cLeft < InpPicWidthInLumaSamples; cLeft += inpPatchWidth) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
}

else if( nnpfc_inp_order_idc  = = 3 )
	for( cTop = 0; cTop < InpPicHeightInLumaSamples; cTop += inpPatchHeight * 2 )
		for( cLeft = 0; cLeft < InpPicWidthInLumaSamples; cLeft += inpPatchWidth * 2 ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
}

nnpfc_reserved_zero_bit shall be equal to 0.

nnpfc_uri[ i ] contains the i-th byte of a NULL-terminated UTF-8 character string, as specified in IETF Internet Standard 63. The UTF-8 character string contains a URI, with syntax and semantics as specified in IETF Internet Standard 66, identifying the neural network used as the post-processing filter.

nnpfc_payload_byte[ i ] contains the i-th byte of a bitstream conforming to ISO/IEC 15938-17. The byte sequence nnpfc_payload_byte[ i ] for all present values of i shall be a complete bitstream that conforms to ISO/IEC 15938-17.

nnpfc_parameter_type_flag equal to 0 indicates that the neural network uses only integer parameters. nnpfc_parameter_type_flag equal to 1 indicates that the neural network may use floating point or integer parameters.

nnpfc_log2_parameter_bit_length_minus3 equal to 0, 1, 2, and 3 indicates that the neural network does not use parameters of bit length greater than 8, 16, 32, and 64, respectively.

nnpfc_num_parameters_idc indicates the maximum number of neural network parameters for the post processing filter in units of a power of 2048. nnpfc_num_parameters_idc equal to 0 indicates that the maximum number of neural network parameters is not specified.

If the value of nnpfc_num_parameters_idc is greater than zero, the variable maxNumParameters is derived as follows:

maxNumParameters = (2048  <<  nnpfc_num_parameters_idc) − 1	(20)

It is a requirement of bitstream conformance that the number of neural network parameters of the post-processing filter shall be less than or equal to maxNumParameters.

nnpfc_num_kmac_operations_idc greater than 0 specifies that the maximum number of multiply-accumulate (MAC) operations per sample of the post-processing filter is less than or equal to nnpfc_num_kmac_operations_idc * 1000.

nnpfc_num_kmac_operations_idc equal to 0 specifies that the maximum number of MAC operations of the network is not specified.

## Neural-network post-filter activation SEI message

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_id | ue(v) |
| } |  |

This SEI message specifies the neural-network post-processing filter that may be used for post-processing filtering for the current picture.

The neural-network post-processing filter activation SEI message persists only for the current picture.

NOTE – There may be several neural-network post-processing filter activation SEI messages present for the same picture, for example, when the post-processing filters are meant for different purposes or filter different colour components.

nnpfa_id specifies that the neural-network post-processing filter specified by one or more neural-network post-processing filter characteristics SEI messages that pertain to the current picture and have nnpfc_id equal to nnfpa_id may be used for post-processing filtering for the current picture.

## Other

In subclause 8.1, add the following, highlighted tables row in Table 4:

Table 4 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| … |  |
| Neural-network post-filter characteristics | Specified by the syntax of the SEI message |
| Neural-network post-filter activation | The current picture |

# VVC proposal

## Payload type

In subclause D.2.1, add the following, yellow-highlighted table row:

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| ... |  |
| else if( payloadType = = 208 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_characteristics( payloadSize ) |  |
| else if( payloadType = = 209 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_activation( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| ... |  |

In subclause D.2.2, replace 207 with 209 as indicated with yellow highlights:

The list VclAssociatedSeiList is set to consist of the payloadType values 3, 19, 45, 47, 129, 132, 137, 142, 144, 145, 147 to 150, inclusive, 153 to 156, inclusive, 165, 168, 177, 179, 180, 200 to 202, inclusive, and 204 to 209207, inclusive.

## Use of neural-network post-filter characteristics SEI message (subclause D.9.x of VVC)

For purposes of interpretation of the neural-network post-filter characteristics SEI message, the following variables are specified:

–	The variables InpPicWidthInLumaSamples and InpPicHeightInLumaSamples are derived for the referenced PPS as follows:

InpPicWidthInLumaSamples = pps_pic_width_in_luma_samples –	SubWidthC * ( pps_conf_win_left_offset + pps_conf_win_right_offset )	(1)

InpPicHeightInLumaSamples = 	pps_pic_height_in_luma_samples –	SubHeightC * ( pps_conf_win_top_offset + pps_conf_win_bottom_offset )	(2)

–	The variables CroppedYPic[ y ][ x ] and chroma sample arrays CroppedCbPic[ y ][ x ] and CroppedCrPic[ y ][ x ], when present, are set to be the 2-dimensional arrays of decoded sample values of the 0th, 1st, and 2nd component, respectively, of the cropped decoded output picture to which the neural-network post-filter characteristics SEI message applies.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

–	InpSubWidthC is set equal to SubWidthC.

–	InpSubHeightC is set equal to SubHeightC.

–	SliceQPY is set equal to SliceQpY.

When an NNR post-filter SEI message with nnpfc_mode_idc equal to 1 or 2 and an NNR post-filter SEI message with nnpfc_mode_idc equal to 3 are present in the same picture unit and both NNR post-filter SEI messages have the same value of nnpfc_id, both NNR post-filter SEI messages shall be present in the same SEI NAL unit.

When a neural-network post-filter characteristics SEI message with the same nnpfc_id and different content are present in the same picture unit, both neural-network post-filter characteristics SEI messages shall be present in the same SEI NAL unit.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).