| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0245-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0118 (AHG7: GDR implementation for ECM 4.0) |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Tangi Poirier | Email: | tangi.poirier@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This document reports the crosscheck contribution of JVET-Z0118[1] on GDR implementation in ECM-4.0. The partial results perfectly match those provided in JVET-Z0118.

# Introduction

Contribution JVET-Y0163[2] of the previous meeting, introduced a new GDR implementation using normative changes. The contribution JVET-Z0118 presents the integration in ECM-4.0 and is enabled with all loop filters tools. In this contribution it is proposed to set the pixels in dirty areas as “not available” for CUs in clean area, and if needed they are set to 	2BD-1, where BD is bit depth, which will give prediction for CUs in clean area of the current picture more freedom over the current picture and reference pictures. For CUs in clean area of a current picture, coding information in dirty areas of the current picture and reference pictures are considered “not available”, which will prevent CUs in clean area of the current picture from using coding information in dirty area of the current picture and reference pictures.

# Analysis

The new GDR implementation doesn’t need to check the validity of intra modes or motion vectors used by the clean region by setting the pixels in dirty area as unavailable at both encoder and decoder. It also keeps 2 version of reference frames, one used by the dirty region where all information is available, as shown in Figure 1 and one for clean region where the dirty region as been set to a default value, Figure 2.

Figure 1: reference frame used by dirty region

Figure 2: reference frame used by clean region

The proposed method is faster to implement code-wise but needs more buffer for the prediction in the current implementation, in an alternative implementation more checks are needed at the decoder. Whereas VTM-like implementation from JVET-Y1062, doesn’t need checks at decoder but need to adapt each tools individually.

# Experimental results

Simulations were conducted over JVET test sequences. GDR tests are performed using low-delay B configuration with the following setting:

- The number of reference pictures is set to 2

- No intra QP offset and flat qp are used

- GOP size is set to 1

- Sequence start with an IDR, then the GDR period is set to FrameRate + 4

For comparison purpose, anchor bitstreams are encoded using an IDR frame instead of GDR frame.

Table 1:Performance of proposed GDR in LDB

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 1.50% | 3.04% | 3.35% | 113% | 114% |
| Class E | 5.87% | 9.76% | 10.79% | 114% | 126% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 2.31% | 5.10% | 6.37% | 114% | 122% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 2: Performance of proposed GDR in LDP

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 1.29% | 3.01% | 3.62% | 115% | 123% |
| Class E | 5.08% | 9.14% | 10.81% | 114% | 122% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 1.97% | 4.49% | 5.80% | 111% | 114% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

Results of JVET-Z0118 have been confirmed

.

# References

[1] S. Hong, L. Wang, K. Panusopone, “AHG7: GDR Implementation for ECM 4.0”, JVET-Z0118, Apr. 2022

[2] L. Wang, S. Hong, K. Panusopone, M. M. Hannuksela, J. Lainema (Nokia), “AHG 7: GDR without encoder constraints for ECM”, JVET-Y0163, Jan. 2022

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).