| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0247-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-Z0072 (AHG10/AHG12: Enhanced reference picture structures) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Christian Helmrich and Christian Bartnik VCA Dept, Einsteinufer 37, 10587 Berlin | Email: | christian.helmrich@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-Z0072 (AHG10/AHG12: Enhanced reference picture structures for ECM and VTM). The cross-checkers confirm that the software changes provided by the proponent match the description in JVET-Z0072 and that the reported BD-rate results are reproduced.

# Experimental Results

The Random Access (RA) and Low-Delay B (LB) configuration changes proposed by the proponent in JVET-Z0072 have been crosschecked, using the unmodified VTM 16.0 and ECM 4.0 sources as baseline. The proponent’s contribution only modifies the RA and LB configuration files, not the source code itself.

The results of the crosscheck simulations, performed according to the VTM SDR common test conditions of JVET-Y2010 and ECM common test conditions of JVET-Y2017, respectively, are tabulated as follows.

VTM CTC:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-16.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.17% | -0.26% | 108% | 103% |
| Class A2 | -0.34% | -0.49% | -0.50% | 109% | 100% |
| Class B | -0.64% | -0.73% | -0.68% | 105% | 100% |
| Class C | -0.68% | -0.68% | -0.63% | 104% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.52% | -0.56% | -0.55% | 106% | 99% |
| Class D | -0.43% | -0.34% | -0.31% | 104% | 102% |
| Class F | -0.37% | -0.34% | -0.31% | 103% | 98% |

|  | Low-Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-16.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.73% | -2.46% | -2.05% | 101% | 97% |
| Class C | -2.51% | -3.90% | -3.79% | 101% | 99% |
| Class E | -1.96% | -3.62% | -3.01% | 101% | 104% |
| Overall | -1.63% | -3.23% | -2.87% | 101% | 99% |
| Class D | -1.98% | -3.84% | -3.78% | 100% | 95% |
| Class F | -2.74% | -3.44% | -3.36% | 101% | 99% |

ECM CTC:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | TBD | TBD | TBD | TBD | TBD |
| Class A2 | TBD | TBD | TBD | TBD | TBD |
| Class B | -0.66% | -0.90% | -0.77% | 100% | 99% |
| Class C | -0.54% | -0.77% | -0.74% | 106% | 101% |
| Class E |  |  |  |  |  |
| Overall | TBD | TBD | TBD | TBD | TBD |
| Class D | -0.20% | -0.35% | -0.41% | 100% | 100% |
| Class F | -0.40% | -0.48% | -0.46% | 102% | 97% |

|  | Low-Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | TBD | TBD | TBD | TBD | TBD |
| Class C | -2.77% | -3.86% | -3.79% | 100% | 98% |
| Class E | -1.76% | -2.57% | -3.36% | 103% | 99% |
| Overall | TBD | TBD | TBD | TBD | TBD |
| Class D | -1.27% | -2.19% | -2.00% | 98% | 98% |
| Class F | -2.47% | -2.69% | -2.68% | 101% | 100% |

# Conclusion

The crosscheckers have verified the description of JVET-Z0072 and its implementation. It is confirmed that the software configuration changes proposed by the proponent match the description in JVET-Z0072, and the BD-rate results of the cluster simulations produced by the crosscheckers match those reported in JVET-Z0072. The encoding run-times obtained by the crosscheckers are close to the proponent’s as well.