| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z2024-v1 |
| --- | --- |

| Title: | Exploration Experiment on Enhanced Compression beyond VVC capability (EE2) |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | EE description |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Guichun Li Karam Naser Jacob Ström Martin Winken Xiaoyu Xiu Kai Zhang | Tel: Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com guichunli@tencent.com Karam.Naser@InterDigital.com jacob.strom@ericsson.com martin.winken@hhi.fraunhofer.de xiaoyuxiu@kwai.com zhangkai.video@bytedance.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document describes Exploration Experiments (EEs) planned to be performed between 26th and 27th JVET meetings to evaluate enhanced compression tools beyond VVC capability.

# Introduction

EE focus is to evaluate individual coding technologies and their combinations. Contributions improving compression efficiency further is highly encouraged.

EE related discussions shall happen on JVET and JVET-CE reflectors.

EE tests should be implemented on top the ECM software, ECM-5.0 is used as an anchor in the tests.

Tests shall be performed according to the CTC described in JVET-Y2017.

For SCC tests, TGM tests are required, and palette is enabled in both anchor and test.

For RPR tests, in addition to ECM CTC the tests are performed following JVET-Q2015, where only LB configuration is mandatory.

AI and RA test configurations are required for intra tool testing, while RA and LB test configurations are required for inter tool testing. LP configuration is optional. In LB and LP configurations, the sequences length is reduced to 5 seconds for all classes.

If encoder modification is included in EE tests, such encoder optimization, if applicable, introduced to the anchor should be tested.

# Timeline

T1 = 3 weeks (May 20, 2022) after JVET meeting: ECM is released

T2 = T1 + 1 week (May 27, 2022): EE description is finalized

T3 = T2 + 2 weeks (June 10, 2022): Initial software release for EE tests

T4 = JVET meeting start – 3 weeks (June 22, 2022): Software in EE branches is frozen

# List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Intra prediction |  |  |  |
| 1.1 | Convolutional cross-component intra prediction model and filter-based linear model | Nokia P. Astola Kwai Che-Wei Kuo |  |
| 1.2 | Gradient linear model | Kwai Che-Wei Kuo |  |
| 1.3 | Test 1.1 + Test 1.2 | Nokia P. Astola Kwai Che-Wei Kuo |  |
| 1.4a | Spatial GPM | OPPO F.Wang |  |
| 1.4b | Spatial GPM with restriction in inter-coded slices | OPPO F.Wang |  |
| 1.5 | Chroma intra modes derived from collocated luma blocks and neighboring chroma blocks | Qualcomm Y.-J. Chang |  |
| 1.6a | Weighted chroma prediction | Xidian Univ. J. Huo |  |
| 1.6b | Test 1.6a + Test 1.1 | Xidian Univ. J. Huo Nokia P. Astola |  |
| 1.7 | IBC adaptation for camera-captured content | Bytedance J. Xu |  |
| 2 Inter prediction |  |  |  |
| 2.1a | Regression based affine candidate derivation | Qualcomm Y. Zhang |  |
| 2.1b | Regression based affine candidate derivation with increased affine candidate list size | Qualcomm Y. Zhang |  |
| 2.2a | Motion compensation boundary padding | Xiaomi F. Le Léannec Qualcomm Z. Zhang |  |
| 2.2b | Motion compensation boundary padding with reduced memory usage | Xiaomi F. Le Léannec Qualcomm Z. Zhang |  |
| 2.3 | MMVD and affine MMVD extension | Bytedance M. Salehifar |  |
| 2.4 | Refined motion for ARMC | Bytedance Y. Wang |  |
| 2.5 | Diversity criterion for ARMC with merge candidates modifications | Canon G. Laroche |  |
| 2.6 | Test 2.4 + Test 2.5 | Canon G. Laroche Bytedance Y. Wang |  |
| 2.7a | GPM adaptive blending | KDDI Y. Kidani Kwai X. Xiaoyu |  |
| 2.7b | Test 2.7a + block-size dependent signalling | KDDI Y. Kidani Kwai X. Xiaoyu |  |
| 2.8 | Longer chroma filters for RPR | Ericsson K. Andersson |  |
| 2.9a | RPR luma filters 12-tap | Ericsson R. Yu |  |
| 2.9b | RPR luma filters 12-tap for non-affine blocks and RPR luma filters 10-tap for affine blocks | Ericsson R. Yu |  |
| 2.10a | Test 2.8 + Test 2.9a | Ericsson R. Yu |  |
| 2.10b | Test 2.8 + Test 2.9b | Ericsson R. Yu |  |
| 3 Screen content coding |  |  |  |
| 3.1 | IBC merge mode with block vector differences | Bytedance N. Zhang |  |
| 3.2 | IBC with reconstruction reordering | Bytedance Z. Deng |  |
| 4 Transform |  |  |  |
| 4.1a | Inter-MTS optimization | Qualcomm B. Ray |  |
| 4.1b | Inter-MTS optimization without KLT | Qualcomm B. Ray |  |
| 5 In-loop filtering |  |  |  |
| 5.1a | Adaptive filter shape switch for ALF | Bytedance W. Yin |  |
| 5.1b | Longer filter length for ALF | Bytedance W. Yin |  |
| 5.2 | Using samples before deblocking filter for ALF | Qualcomm N. Hu |  |
| 5.3a | Test 5.1a + Test 5.2 | Bytedance W. Yin Qualcomm N. Hu |  |
| 5.3b | Test 5.1b + Test 5.2 | Bytedance W. Yin Qualcomm N. Hu |  |

# Tools description

## Intra prediction

### Test 1.1: Convolutional cross-component intra prediction model (JVET-Z0064) and filter-based linear model (JVET-Z0140)

Convolutional cross-component model and filter-based linear model are investigated in this EE. In both methods, reconstructed luma samples are filtered to generate chroma prediction samples, which is activated by a PU level flag. Filter coefficients are derived for each chroma block separately using regression based MSE minimization on reference samples in the PU’s neighborhood.

In the convolutional cross-component model, 7-tap convolutional filter is applied. The filter input consists of 5 spatial luma samples, a nonlinear term, and a bias term.

In the filter-based linear model, multiple filters are supported and are switched in PU level. The filter is applied directly on the reconstructed luma samples, i.e., without down-sampling.

The test is performed using a configuration defined in JVET-Z0064. Additional configurations may be tested by selecting different aspects described in JVET-Z0064 and JVET-Z0140, and combinations of those. Such additional tests will be defined and described in the final version of this document.

### Test 1.2: Gradient linear model (JVET-Z0140)

Compared with CCLM, instead of using down-sampled luma samples, the gradient linear model utilizes luma sample gradients to derive the linear model. In other words, when the tested model is enabled, gradient filter is used to replace the existing down-sample filter in the CCLM process.

Control flags are signaled for chroma components to indicate whether gradient linear model is enabled or not; if the model is enabled, further flags are signaled to select one from multiple gradient patterns.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.1 | Convolutional cross-component intra prediction model and filter-based linear model | Nokia P. Astola Kwai Che-Wei Kuo |
| 1.2 | Gradient linear model | Kwai Che-Wei Kuo |
| 1.3 | Test 1.1 + Test 1.2 | Nokia P. Astola Kwai Che-Wei Kuo |

### Test 1.4: Spatial GPM (JVET-Z0124)

Spatial GPM intra prediction method is tested. It employs a template-reordered candidate list, where each candidate in the list comprises a combination of the partition mode and two intra prediction modes, and only signals the candidate index. The objective of this test is to improve the overall trade-off between coding efficiency and algorithm complexity.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.4a | Spatial GPM | OPPO F.Wang |
| 1.4b | Spatial GPM with restriction in inter-coded slices | OPPO F.Wang |

### Test 1.5: Chroma intra modes derived from collocated luma blocks and neighboring chroma blocks (JVET-Z0143)

In this test, the non-CCLM chroma intra mode list is constructed in the following order: the intra prediction modes of collocated luma blocks, the intra prediction modes of neighbouring chroma blocks, derived modes by offsetting +1 or -1 to the chroma intra modes in the non-CCLM chroma mode list, and default modes.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.5 | Chroma intra modes derived from collocated luma blocks and neighboring chroma blocks | Qualcomm Y.-J. Chang |

### Test 1.6: Weighted chroma prediction (JVET-Z0100)

Weighted chroma prediction method is tested, where predicted chroma samples of the current block are derived based on the weights and neighboring chroma samples. The weights are designed according to the difference between neighboring luma samples and the reconstructed luma samples of a coding block.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.6a | Weighted chroma prediction | Xidian Univ. J. Huo |
| 1.6b | Test 1.6a + Test 1.1 | Xidian Univ. J. Huo Nokia P. Astola |

### Test 1.7: IBC adaptation for camera-captured content (JVET-Z0157)

In this test, IBC adaptation, including encoder optimization and reference area adaptation for large CTU size will be investigated.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 1.7 | IBC adaptation for camera-captured content | Bytedance J. Xu |

## Inter prediction

### Test 2.1: Regression based affine candidate derivation (JVET-Z0125)

In this test, regression based affine candidate derivation method is tested. The input to the regression process is subblock motion information from previously coded affine CUs and neighboring inter coded subblocks.

Additionally, increased affine candidate list size is tested with the regression based affine candidate derivation. The ARMC process reorders all candidates from affine candidate list. However, only a part of the reordered candidates is checked in the SATD stage.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.1a | Regression based affine candidate derivation | Qualcomm Y. Zhang |
| 2.1b | Regression based affine candidate derivation with increased affine candidate list size | Qualcomm Y. Zhang |

### Test 2.2: Motion compensation boundary padding (JVET-Z0130)

Motion compensation boundary padding for reference picture boundary handling is tested. When a reference block locates partially or completely out of the picture boundary, the outside samples are replaced with the motion compensation-based padding.

In the implementation, the total padded area size is increased by 64 in Test 2.2a comparing to ECM to keep MV clipping, which implements repetitive padding, non-normative.

In the tests conducted, different values of the padding area size increase will be evaluated: the initial value 64 (Test 2.2a) and a reduced value (Test 2.2b), e.g. 16, targeting reduced memory consumption.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.2a | Motion compensation boundary padding | Xiaomi F. Le Léannec Qualcomm Z. Zhang |
| 2.2b | Motion compensation boundary padding with reduced memory usage | Xiaomi F. Le Léannec Qualcomm Z. Zhang |

### Test 2.3: MMVD and affine MMVD extension (JVET-Z0142)

In this test extra MMVD (and affine MMVD) positions are added for the bi-prediction cases. Also, extra base positions are added depending on the neighboring blocks. It also tests based on pruning similar candidates.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.3 | MMVD and Affine MMVD extension | Bytedance M. Salehifar |

### Test 2.4: Refined motion for ARMC (JVET-Z0145)

In this test addition motion refining is applied before candidates reordering. In the method, each merge candidate in the merge candidate list is refined using TM/multi-pass DMVR and the refined motion is used in ARMC to reorder the merge candidate list. ARMC with refined motion is applied to template matching merge (TM merge) mode, adaptive DMVR mode, and template matching for advanced motion vector prediction (TM-AMVP) mode. When multi-pass DMVR is used to derive the refined motion, only the first pass (i.e., PU level) of multi-pass DMVR is applied in reordering. TM can be extended to perform 1/16-pel MVD precision. Only the above or left template is used during the motion refinement of TM when the block is flat or narrow. When constructing the AMVP list, an MVP candidate with a TM cost larger than a threshold is skipped.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.4 | Refined motion for ARMC | Bytedance Y. Wang |

### Test 2.5: Diversity criterion and merge mode derivation modifications (JVET-Z0103)

Reordering based on a diversity criterion for ARMC and modifications of merge candidates derivation.

In the test, a reordering based on a diversity criterion is applied for ARMC. This reordering is applied for several modes or MV residual coding where ARMC or similar reordering are applied. In addition, the merge candidates derivation is adapted to this criterion and some pairwise candidates are added after a first ARMC and reordered with other candidates.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.8 | Diversity criterion for ARMC with merge candidates modifications | Canon G. Laroche |

### Test 2.6: Combination of ARMC related tests

A combination of Test 2.4 + Test 2.5 is tested.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.6 | Test 2.4 + Test 2.5 | Canon G. Laroche |

### Test 2.7: GPM adaptive blending (JVET-Z0059, JVET-Z0137)

An adaptive GPM blending scheme is investigated in the EE test. Specifically, besides the existing blending area (i.e., 2 sample on each side of the GPM partition split boundary), extra blending area sizes are added for the GPM model. The selected blending area size is signalled from encoder to decoder. Additionally, block-size dependent signaling will be tested to reduce the signaling overhead.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.7a | GPM adaptive blending | KDDI Y. Kidani Kwai X. Xiaoyu |
| 2.7b | Test 2.7a + block-size dependent signalling | KDDI Y. Kidani Kwai X. Xiaoyu |

### Test 2.8: Longer chroma filters for RPR (JVET-Z0069)

Increased chroma interpolation filter lengths to 6-tap for RPR is tested.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.8 | RPR chroma filters 6-tap | Ericsson K. Andersson |

### Test 2.9: Longer luma filters for RPR (JVET-Z0062)

Increased luma interpolation filter lengths for RPR is tested.

In Test 2.9a, the luma RPR filter length is 12-tap for both affine and non-affine blocks. In Test 2.7b, the luma RPR filter length is 12-tap for non-affine blocks and the luma RPR filter length is 10-tap for affine blocks.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.9a | RPR luma filters 12-tap | Ericsson R. Yu |
| 2.9b | RPR luma filters 12-tap for non-affine blocks and RPR luma filters 10-tap for affine blocks | Ericsson R. Yu |

### Test 2.10: Combination of RPR filter tests

Combinations of Test 2.8 and Test 2.9 are investigated.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 2.10a | Test 2.8 + Test 2.9a | Ericsson R. Yu |
| 2.10b | Test 2.8 + Test 2.9b | Ericsson R. Yu |

## Screen content coding

### Test 3.1: IBC merge mode with block vector differences (JVET-Z0152)

IBC merge mode with block vector differences (a.k.a. IBC-MBVD) is introduced. In IBC-MBVD, after an IBC base candidate is selected, it is further refined by the signalled BVDs information. An IBC-MBVD flag is signalled to specify whether IBC-MBVD mode is used for a CU. The base candidates are selected from the reordered IBC merge list and refinement positions are selected based on template costs.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.1 | IBC merge mode with block vector differences | Bytedance N. Zhang |

### Test 3.2: IBC with reconstruction reordering (JVET-Z0159)

A Reconstruction-Reordered IBC mode for screen content video coding is tested. When it is applied to an IBC coded block, the reconstruction block is flipped. Two symmetric flip methods, horizontal flip and vertical flip, are supported in this mode. Syntax flags are signalled for IBC AMVP, indicating whether the reconstruction block is flipped and how to flip it. For IBC merge, the flip type is inherited from neighbouring blocks without signalling. Moreover, the block vector candidate obtained from a neighbouring block can be adjusted according to a symmetric rule.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 3.2 | IBC with reconstruction reordering | Bytedance Z. Deng |

## Transform

### Test 4.1: Inter-MTS optimization (JVET-Z0147)

In this test, maximum CU size of Inter-MTS is signalled, and for smaller blocks (size less or equal to 16) DST7/DCT8 is replaced with separable KLT matrices.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 4.1a | Inter-MTS optimization | Qualcomm B. Ray |
| 4.1b | Inter-MTS optimization without KLT | Qualcomm B. Ray |

## In-loop filtering

### Test 5.1: Adaptive filter shape switch for ALF (JVET-Z0149)

In this test, a CTB-level adaptive filter shape switch method for ALF is tested. In the method, a filter shape index which is associated with luma filters is signalled for each APS. Each CTB could select the best luma filter shape from the available APSs. Longer filter length for ALF is also tested.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 5.1a | Adaptive filter shape switch for ALF | Bytedance W. Yin |
| 5.1b | Longer filter length for ALF | Bytedance W. Yin |

### Test 5.2: Using samples before deblocking filter for ALF (JVET-Z0146)

In this test, samples before deblocking filters are used as additional inputs for ALF. A final ALF sample is derived by weighting the regular ALF and the filter applied to the samples before the deblocking filter.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 5.2 | Using samples before deblocking filter for ALF | Qualcomm N. Hu |

### Test 5.3: Combination of ALF methods

Combination of the ALF Test 5.1 and Test 5.2 is tested.

List of tests to be performed

| # | Test | Tester |
| --- | --- | --- |
| 5.3a | Test 5.1a + Test 5.2 | Bytedance W. Yin Qualcomm N. Hu |
| 5.3b | Test 5.1b + Test 5.2 | Bytedance W. Yin Qualcomm N. Hu |