| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0002-v1 |
| --- | --- |

| Title: | JVET AHG report: Draft text and test model algorithm description editing (AHG2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JVET |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | B. Bross, C. Rosewarne, F. Bossen, J. Boyce, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye |  | benjamin.bross@hhi.fraunhofer.de chris.rosewarne@canon.com.au frank@bossentech.com jill.boyce@vimmerse.net adrian.browne@sony.com seunghwan3.kim@lge.com shanl@tencent.com ohm@ient.rwth-aachen.de garysull@microsoft.com alexismt@apple.com yekui.wang@bytedance.com yan.ye@alibaba-inc.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports on the work of the JVET ad hoc group on Draft text and test model algorithm description editing (AHG2) between the 26th meeting by teleconference, (20 – 29 April 2022) and the 27th meeting by teleconference, (13 – 22 July 2022).

# Mandates

At the 26th meeting of the ITU-T/ISO/IEC Joint Video Experts Team (JVET), an ad hoc group on Draft text and test model algorithm description editing (AHG2) was established with the following mandates:

- Produce and finalize draft text outputs of the meeting (JVET-Z1003, JVET-Z1005, JVET-Z1008, JVET-Z2005, and JVET-Z2006).

- Collect reports of errata for the VVC, VSEI, HEVC, AVC, CICP, the codepoint usage TR specification and the published HDR-related technical reports and produce the JVET-Z1004 errata output collection.

- Produce and finalize JVET-Z2002 VVC Test Model 17 (VTM 17) Algorithm and Encoder Descriptions.

- Coordinate with the test model software development AhG to address issues relating to mismatches between software and text.

- Collect and consider errata reports on the texts.

# Ad hoc group activity

## Output documents produced

### JVET-Z1003 Coding-independent code points for video signal type identification (Draft 1 of 3rd edition)

This document contains the draft text toward a 3rd edition of Coding-independent code points for video signal type identification (Rec. ITU-T H.273 | ISO/IEC 23091-2). Text modifications are provided for specification of code point identifiers for YCoCg-R colour representation with equal luma and chroma bit depths. The new code points are referred to as YCgCo-Re and YCgCo-Ro, where the number of bits added to a source RGB bit depth is 2 (i.e., even) and 1 (odd), respectively. Revision marking is provided to show modifications relative to the basis text (based on the 2021-07 edition of Rec. ITU-T H.273). Equation numbers and their cross-references that are maintained automatically have been updated without revision marking.

### JVET-Z1004 Errata report items for VVC, VSEI, HEVC, AVC, Video CICP, and CP usage TR

This document contains a list of reported errata items for VVC, VSEI, HEVC, AVC, Video CICP, and the TR on usage of video signal type code points, for tracking purposes. Some of the items have been confirmed by the JVET and have been agreed to require fixing, while some other items have not yet been confirmed. This document also provides publication status backgrounds of these standards.

Incorporated items at the JVET-Z meeting:

- For AVC, corrections to texts related to alpha blending corresponding to the changes adopted to VSEI from JVET-Z0119.

- Updated the summary and publication status for VVC.

- Updated the summary and publication status for VSEI.

- Updated the summary for HEVC.

### JVET-Z1005 New levels for HEVC (Draft 3)

This document contains the draft text for changes to the High Efficiency Video Coding (HEVC) standard (ITU-T H.265 | ISO/IEC 23008-2), for the addition of Levels 6.3, 7, 7.1, and 7.2. In addition, the document also contains some corrections to the previous edition of HEVC.

Draft 3 incorporated items:

- Increasing the max bit rate values for the High tier of levels 7.1 and 7.2 (JVET-Z0060)

- Clarifications on the alpha blending text description (JVET-Z0119)

- Text changes for the unlimited level for video profiles (JVET-Z0122)

- Upon ballot comment US-006, it was decided to increase the maximum frame rate to 960 fps (same value as in the corresponding VVC levels) in the high tier for levels 7.0 and higher.

- Correction of derived luma picture size numbers for 11520x6480 and 12288x6480 in Tables A-11, A-12, and A-13

- Further aspects of errata reports per JVET-Z1004

One issue noticed during the editing of this document is that the maximum luma picture size for new levels 7, 7.1 and 7.2 and the maximum luma sample rate for levels 7.1 and 7.2 may have been determined incorrectly. These values in the text were proposed in JVET-Y0072 (from A. Tourapis, D. Singer, and K. Kolarov), saying that the motivation was to enable “up to video resolutions of 8192x4320 at 120fps” (120 fps for level 7.2, and presumably half that frame rate for level 7.1). However, the proposed values for the specified limits are higher than the values necessary for achieving that goal, as they appear to have been computed by rounding up the luma picture width and height to multiples of 128. In fact, rounding only to multiples of 64 is used to derive all other values in Annex A (and values of MinCbSizeY greater than 64 are prohibited in HEVC). As can be observed in Table A.12 of the JVET-Y0072 proposal document, the drafted values thus support slightly higher frame rates than what is described in the document’s abstract (60.4 and 120.8 rather than 60 and 120 fps for levels 7.1 and 7.2, respectively). A maximum luma picture size of 141 557 760 would appear sufficient, but 142 606 336 was proposed and adopted into the draft text. To support the 8192x4320 picture size at maximum frame rates of 60 and 120 fps, the maximum luma sample rate limits could be reduced from 8 556 380 160 to 8 493 465 600 for level 7.1 and from 17 112 760 320 to 16 986 931 200 for level 7.2. The editors refrained from taking action on this issue since overprovisioning is not necessarily an error and since no input had been received on this issue in the previous meeting cycle. (Incorrect values derived from specified limits were corrected as noted above, but the apparent overprovisioning of the limits was left as-is.)

### JVET-Z1008 Additional color type identifiers for AVC and HEVC (Draft 1)

This document contains the draft text for the specification of additional colour type identifiers for AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10) and HEVC (Rec. ITU-T H.265 | ISO/IEC 23008-2). Text modifications are provided for specification of code point identifiers for YCoCg-R colour representation with equal luma and chroma bit depths. The new code points are referred to as YCgCo-Re and YCgCo-Ro, where the number of bits added to a source RGB bit depth is 2 (i.e., even) and 1 (odd), respectively. Revision marking is provided to show modifications relative to the basis texts (based on the 2021-08 edition of Rec. ITU-T H.264 and the 2020 edition of ISO/IEC 23008-2, respectively). Equation numbers and their cross-references that are maintained automatically have been updated without revision marking.

### JVET-Z2002 Algorithm description for Versatile Video Coding and Test Model 17 (VTM 17)

The JVET established the VVC Test Model 17 (VTM17) algorithm description and encoding method at its 26th meeting (20 – 29 April 2022, teleconference). This document serves as a source of general tutorial information on the VVC design and also provides an encoder-side description of VTM17. The VVC has been developed by a joint collaborative team of ITU-T and ISO/IEC experts known as the Joint Video Experts Team (JVET), which is a partnership of ITU-T Study Group 16 Question 6 (known as VCEG) and ISO/IEC JTC 1/SC 29/WG 11 (known as MPEG). This draft new standard has been designed with two primary goals. The first of these is to specify a video coding technology with a compression capability that is substantially beyond that of the prior generations of such standards, and the second is for this technology to be highly versatile for effective use in a broadened range of applications. Some key application areas for the use of this standard particularly include ultra-high-definition video (e.g., with 3840×2160 or 7620×4320 picture resolution and bit depth of 10 or 12 bits as specified in Rec. ITU-R BT.2100), video with a high dynamic range and wide colour gamut (e.g., with the perceptual quantization or hybrid log-gamma transfer characteristics specified in Rec. ITU-R BT.2100), and video for immersive media applications such as 360° omnidirectional video projected using a common projection format such as the equirectangular or cubemap projection format, in addition to the applications that have commonly been addressed by prior video coding standards.

Ed. Notes:

VVC Test Model 17 (VTM17) algorithm description and encoding method v1

- Incorporated JVET-Z0046: VTM Software Implementation for GREEN-MPEG SEI Messaging

- Incorporated JVET-Z0072: AHG10/AHG12: Enhanced reference picture structures for ECM and VTM

- Incorporated JVET-Z0099: AHG10: Deblocking in RDO and beta offset minus 2 for VTM

- Incorporated JVET-Z0111: Adaptively bypass affine ME in VTM

- Incorporated JVET-Z0120: AHG9: Shutter interval information SEI message for VSEI

- General editorial improvements throughout

At the time of preparing this AHG2 report, JVET-Z2002-v1 was available, with the following note included regarding a second version, which the editors plan to upload during the 27th meeting:

To be integrated into VVC Test Model 17 (VTM17) algorithm description and encoding method v2

- TBD: Incorporated JVET-Z0047: AHG13: Improvements of film grain analysis

- TBD: Incorporated JVET-Z0244: AHG9: NN post-filter SEI

### JVET-Z2005 New level and systems-related supplemental enhancement information for VVC (Draft 2)

This document contains a draft amendment for changes to the Versatile Video Coding (VVC) standard (ITU-T H.266 | ISO/IEC 23090-3). This amendment includes the support of an additional level of capability for the profiles that have previously been defined, thus expanding the range of application requirements addressed by the standard. This amendment also includes the support of two systems-related supplemental enhancement information (SEI) messages, for signalling of “green metadata” as to be specified in ISO/IEC 23001-11 and of an alternative video decoding interface for immersive media as to be specified in ISO/IEC 23090-13.

Draft 2 incorporated items:

- Updated text changes for the unlimited level for video profiles (JVET-Z0122)

- Fixes for tickets #1548 and #1549

- Fixes for tickets #1544 and #1547

### JVET-Z2006 Additional SEI messages for VSEI (Draft 1)

This document contains the draft text for changes to the versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7), to specify additional SEI messages, including the shutter interval information SEI message and SEI messages to facilitate neural-network-based post-processing filtering.

# Related input contributions

The following input contribution was noted as relevant to the work of this ad hoc group:

- JVET-AA0048 Some VVC text changes

# Remaining bug tickets

- #1555 Typo in 7.4.3.5 (pps_num_exp_tile_rows_minus1)

- #1556 Typo in clause 7.4.2.4.5 (Order of VCL NAL units and their association to coded pictures)

- #1558 Mismatch between spec & VTM for MipChromaDirectFlag derivation in YUV444

- #1560 Typo regarding "maxTbSize"

# Recommendations

The AHG recommends to:

- Approve JVET-Z1003, JVET-Z1004, JVET-Z1005, JVET-Z1008, JVET-Z2002, JVET-Z2005, and JVET-Z2006 documents as JVET outputs,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification text and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC text specification and Test Model documents,

- Ensure that, when considering changes to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC specification text is made available in a timely manner,

- Review AHG2 related contributions, bug tickets, and other AHG2 related inputs and act on them if found to be necessary.