| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0011-v1 |
| --- | --- |

| Title: | JVET AHG report: Neural network-based video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Elena Alshina Shan Liu Andrew Segall Franck Galpin Jonathan Pfaff Shanshe Wang Zhao Wang Mathias Wien Ping Wu Jizheng Xu | Email: | elena.alshina@huawei.com shanl@tencent.com asegall@amazon.com Franck.Galpin@InterDigital.com firstname.lastname@hhi.fraunhofer.de sswang@pku.edu.cn baixiu.wz@alibaba-inc.com wien@lfb.rwth-aachen.de pwuzte@gmail.com xujizheng@bytedance.com |
| Source: | AHG11 coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG11: Neural network-based video coding between the 26th meeting (20 – 29 April 2022) and the 27th meeting (13 – 22 July 2022), both held by teleconference.

# Introduction

The AHG was established with the following mandates:

- Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools and novel architectures.

- Refine the test conditions for NN-based video coding. Generate and distribute anchor encoding, and develop supporting software as needed.

- Study the impact of training (including the impact of loss function) on the performance of candidate technologies, and identify suitable materials for testing and training.

- Analyse complexity characteristics, perform complexity analysis, and develop complexity reductions of candidate technology.

- Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in library support and develop improvements as needed.

- Finalize and discuss the EE on neural network-based video coding.

- Coordinate with other relevant groups, including SC29/AG5 on the evaluation and assessment of visual quality and AHG12 on the interaction with ECM coding tools. In case of conducting subjective viewing, support shall be given to AHG4 for timely availability of test materials and test subjects.

- Investigate common software for development and verification NN-based video coding technologies.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, for email. Eight emails were exchanged on the reflector related to the AHG mandates.

## EE Coordination

The AHG finalized, conducted and discussed the EE on NN based video coding.  The final version of the EE description was uploaded to the document repository on May 17, 2022.

A summary report for the EE is available at this meeting as:

| JVET-AA0023 | EE1: Summary of Exploration Experiments on Neural Network-based Video Coding | E. Alshina, W. Chen, F. Galpin, Y. Li, Z. Ma, H. Wang, L. Wang |
| --- | --- | --- |

## Anchor Encoding

Anchors for the NN-based video coding activity were unchanged from those released on August 2, 2021.  The anchors were also made available on the Git repository used for the AHG activity: https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master.

## Study and Maintain SADL

The SADL library was updated during the AHG period.  Updates were announced on May 4, 2022; May 19, 2022; June 2, 2022; June 10, 2022; and July 12, 2022.  The software is available at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/sadl.

Change logs include:

Commit 43515f670d294367d70114ab3aaac62dc9179e9a

Add dynamic quantization documentation

Commit 1e27c61ec38c0ef75c69482925662be251b417ca

Changes:

change the Add layer quantizer policy: use the smallest quantizer

instead of the 2nd input one. BiasAdd layer keeps the same policy.

Commit 7876017ebb70c67916da06d757ddac1974428b7a

Corrections:

- carry over quantizer values for Expand and transpose layers (JVET-Z-EE-1.6)

- add conditions for special tensors multiplication case (JVET-Z0082)

- added leakyRelu case in naive_qunatization sample code (JVET-Z-EE-1.5)

Improvements:

- SSE42 for convolution in mod8 case (speedup JVET-Z0082)

Commit 29b68a048bc3ae968b8d1aa3e08319d5d37bd4e3

New:

- naive quantization example

- associated documentation

Improvements

- check on quantizer values for some layers

- better inference debug info

- in count_mac: also simulate an overflow

Commit 3d258f089761b1c906e94c51ec54c8ffd788a85c

New:

- split sample programs: count_mac, debug_model, sample_xxx

- added clang-format file

Improvements:

- improve SIMD support for float (AVX512, conv1x1 etc.)

- go back to HWCD loop order. Some gains on big models.

- nbOutputs() method in Model

Corrections:

- init is now re-entrant: it is possible to do inference with multiple input sizes by calling init with the new size

- remove all MAC counters in SIMD paths + add warning message

Cleaning:

- clean the naming for conv2d (simd, dispatch etc.)

- clean messages for SIMD support

- clean macro for debug

- clean sample_test.sh script

- clang format everything

- copyright year

- README update

- other minor cleanings

## Technical Evaluation

The AHG made meaningful progress on the mandate to evaluate and quantify potential NN based video coding technologies.  A summary of AHG11 related non-EE1 contributions provided as input to the 27th meeting is provided below:

|  | Title | Common Test Conditions | Results |  |  | Training Data |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RA | LDB | AI | CTC | Additional |
| Loop Filter |  |  |  |  |  |  |  |
| JVET-AA0080 | AHG11: Complexity reduction on neural-network loop filter | Yes | No | No | Yes | - | DIV2K |
| Post Filtering |  |  |  |  |  |  |  |
| JVET-AA0054 | AHG9: On Neural-network Post-filter Characteristics SEI Message | No | No | No | No | - | - |
| JVET-AA0055 | AHG9: Comments on Neural-network Post-filter Characteristics SEI Message | No | No | No | No | - | - |
| JVET-AA0056 | AHG9: On syntax gating in the neural-network post-filter characteristics SEI message | No | No | No | No | - | - |
| JVET-AA0067 | AHG9: Some specification improvements for neural-network post-filter characteristics SEI message | No | No | No | No | - | - |
| JVET-AA0083 | AHG9: NNR post-filter SEI message extension for flexible decoding capabilities | No | No | No | No | - | - |
| JVET-AA0100 | AHG9: On auxiliary input and separate colour description in the neural-network post-filter characteristics SEI message | No | No | No | No | - | - |
| JVET-AA0101 | AHG9: On processing order in the neural-network post-filter activation SEI message | No | No | No | No | - | - |
| JVET-AA0145 | AHG9: On decoupling neural-network post-filter activation SEI message | No | No | No | No | - | - |
| Super-Resolution |  |  |  |  |  |  |  |
| JVET-AA0065 | AHG11: CNN Filter for Super-Resolution with RPR functionality in VVC | Yes | Yes | No | Yes | BVI-DVC | DIV2K |
| JVET-AA0076 | AHG11: RPR-Based Super-Resolution Guided by Partition Information | Yes | Yes | No | Yes | BVI-DVC | DIV2K |
| JVET-AA0084 | AHG11: Neural Network based Super Resolution for Video Coding Using Multiple Side Information | Yes | Yes | No | Yes | BVI-DVC, TVD | DIV2K |
| Inter-Prediction |  |  |  |  |  |  |  |
| JVET-AA0082 | AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement | Yes | Yes | Yes | No | BVI-DVC | VIMEO |
| End-to-End |  |  |  |  |  |  |  |
| JVET-AA0063 | AHG11: A hybrid codec using E2E image coding combined with VVC video coding | No | - | - | - | - | - |

# Input contributions

There are 43 input contriubtions related to the AHG mandates. Twenty-six of the contributions are part of the EE activity, while the remaining 17 contributions are related to AHG11 but not part of the EE. The list of input contributions is provided below.

## EE and Related Input Contributions

| Reporting |  |  |
| --- | --- | --- |
| JVET-AA0023 | EE1: Summary of Exploration Experiments on Neural Network-based Video Coding | E. Alshina, W. Chen, F. Galpin, Y. Li, Z. Ma, L. Wang |
| EE Technology |  |  |
| JVET-AA0059 | EE1-3.1: Supplementary experiments based on JVET-Z0077 | Q. Qin, C. Jung (Xidian Univ.), D. Zou, M. Li (Oppo) |
| JVET-AA0066 | EE1-1.7: Content-adaptive post-filter based on SADL inference | R. Yang, M. Santamaria, F. Cricri, H. Zhang, J. Lainema, R. G. Youvalari, M. M. Hannuksela (Nokia) |
| JVET-AA0071 | EE1-2.1: A CNN-based Super Resolution Method Combined with GOP Level Adaptive Resolution | S. Peng, C. Fang, D. Jiang, J. Lin, X. Zhang (Dahua), J. Nam, S. Yoo, J. Lim, S. Kim (LGE) |
| JVET-AA0081 | EE1-1.2: NN intra model without attention, partitioning and boundary strength | J. Ström, D. Liu, M. Damghanian, K. Andersson, Y. Li, P. Wennersten, R. Yu (Ericsson) |
| JVET-AA0085 | EE1-1.1: The Performance of Single-Model Filter Trained on the VTM and ECM Reconstruction | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| JVET-AA0087 | EE1-1.4: Neural network based in-loop filter with 2 models | L. Wang, S. Lin, X. Xu, S. Liu (Tencent), F. Galpin (InterDigital) |
| JVET-AA0088 | EE1-1.5: Neural network based in-loop filter with a single model | L. Wang, S. Lin, X. Xu, S. Liu (Tencent), F. Galpin (InterDigital) |
| JVET-AA0111 | EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation | Y. Li, K. Zhang, J. Li, L. Zhang (Bytedance), H. Wang, M. Coban, A.M. Kotra, M. Karczewicz (Qualcomm), F. Galpin (InterDigital), K. Andersson, J. Ström, D. Liu, R. Sjöberg (Ericsson) |
| JVET-AA0122 | EE1-1.3: On BaseQP adjustment in CNNLF | Z. Xie, Y. Yu, H. Yu, D. Wang (Oppo) |
| EE Related |  |  |
| JVET-AA0074 | [EE1-related] Lighter WCDANN: CNN Based In-Loop Filters | H. Zhang, C. Jung (Xidian Univ.), D. Zou, M. Li (Oppo) |
| JVET-AA0089 | EE1-related: More refinements on EE1-1.4 and EE1-1.5 | L. Wang, S. Lin, X. Xu, S. Liu (Tencent), Z. Xie, Y. Yu, H. Yu, D. Wang (Oppo) |
| JVET-AA0090 | EE1-related: One luma model with IPB and skip for filtering intra and inter luma slices | D. Liu, J. Ström, M. Damghanian, P. Wennersten, Y. Li (Ericsson) |
| JVET-AA0094 | EE1-related: Deep In-Loop Filter in EE1-1.6 with Adaptive Input Samples | C. Zhou, Z. Lv, J. Zhang (vivo), W. Chen, J. Guo, B. Ai (BJTU) |
| JVET-AA0112 | EE1-1.6-related: Deep In-Loop Filter with Additional Input Information | Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AA0113 | EE1-1.6-related: RDO Considering Deep In-Loop Filter with SADL | J. Li, Y. Li, K. Zhang, L. Zhang (Bytedance) |
| JVET-AA0115 | EE1-1.6-related: ALF with Samples before Deep In-Loop Filter | J. Li, K. Zhang, Y. Li, L. Zhang (Bytedance) |
| JVET-AA0131 | EE1-related: CNN based in-loop filtering with large activation layer | H. Wang, S. Eadie, M. Coban, M. Karczewicz (Qualcomm) |
| Cross Checks |  |  |
| JVET-AA0172 | Cross-check of JVET-AA0081 (EE1-1.2: NN intra model without attention, partitioning and boundary strength) | M. Santamaria, F. Cricri (Nokia) |
| JVET-AA0174 | [EE1] Crosscheck of training stage for EE1-1.5 and EE1-1.6 tests | J. Sauer, B. Wang, E.Alshina (Huawei) |
| JVET-AA0178 | Crosscheck of JVET-AA0088 (EE1-1.5: Neural network based in-loop filter with a single model) | H. Wang (Qualcomm) |
| JVET-AA0179 | Crosscheck of JVET-AA0085 (EE1-1.1: The Performance of Single-Model Filter Trained on the VTM and ECM Reconstruction) | C. Zhou (vivo) |
| JVET-AA0181 | Cross-check of JVET-AA0111 (EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation) | K. Lin, C. Jia, S. Wang |
| JVET-AA0185 | Crosscheck of JVET-AA0122 (EE1-1.3: On BaseQP adjustment in CNNLF) | L. Wang (Tencent) |
| JVET-AA0186 | Crosscheck of JVET-AA0131 (EE1-related: CNN based in-loop filtering with large activation layer) | L. Wang (Tencent) |
| JVET-AA0200 | Crosscheck of JVET-AA0094 (EE1-related: Deep In-Loop Filter in EE1-1.6 with Adaptive Input Samples) | Y. Li |

## Non-EE Input Contributions

| Reporting |  |  |
| --- | --- | --- |
| JVET-AA0011 | JVET AHG report: Neural network-based video coding (AHG11) | E. Alshina, S. Liu, A. Segall, F. Galpin, J. Pfaff, S. S. Wang, Z. Wang, M. Wien, P. Wu, J. Xu (AHG chairs) |
| JVET-AA0047 | [AHG 11] Brief information about JPEG AI CfP status | E. Alshina, J. Ascenso, T. Ebrahimi, F. Pereira, T. Richter |
| Loop Filtering |  |  |
| JVET-AA0080 | AHG11: Complexity reduction on neural-network loop filter | J. N. Shingala, S. Kadaramandalgi, A. Shyam (Ittiam), T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy (Dolby) |
| Post Filtering |  |  |
| JVET-AA0054 | AHG9: On Neural-network Post-filter Characteristics SEI Message | S. Deshpande (Sharp) |
| JVET-AA0055 | AHG9: Comments on Neural-network Post-filter Characteristics SEI Message | S. Deshpande, A. Sidiya (Sharp) |
| JVET-AA0056 | AHG9: On syntax gating in the neural-network post-filter characteristics SEI message | M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia), T. Chujoh, Y. Yasugi, T. Ikai (Sharp), S. McCarthy, A. Arora, T. Shao, P. Yin, T. Lu, F. Pu, W. Husak (Dolby) |
| JVET-AA0067 | AHG9: Some specification improvements for neural-network post-filter characteristics SEI message | T. Chujoh, Y. Yasugi, T. Ikai (Sharp), M. Hannuksela, F. Cricri (Nokia), S. McCarthy, A. Arora, T. Shao, P. Yin, T. Lu, F. Pu, W. Husak (Dolby) |
| JVET-AA0083 | AHG9: NNR post-filter SEI message extension for flexible decoding capabilities | F. Galpin, T. Dumas, P. Bordes, E. François (InterDigital) |
| JVET-AA0100 | AHG9: On auxiliary input and separate colour description in the neural-network post-filter characteristics SEI message | T. Shao, A. Arora, P. Yin, S. McCarthy, T. Lu, F. Pu, W. Husak (Dolby), M. M. Hannuksela, F. Cricri, M. Santamaria Gomez (Nokia), T. Chujoh, Y. Yasugi, T. Ikai (Sharp) |
| JVET-AA0101 | AHG9: On processing order in the neural-network post-filter activation SEI message | T. Shao, A. Arora, P. Yin, S. McCarthy, T. Lu, F. Pu, W. Husak (Dolby) |
| JVET-AA0145 | AHG9: On decoupling neural-network post-filter activation SEI message | H.-B. Teo, J. Gao, C.-S. Lim, K. Abe, V. Drugeon (Panasonic) |
| Super Resolution |  |  |
| JVET-AA0065 | AHG11: CNN Filter for Super-Resolution with RPR functionality in VVC | S. Huang, C. Jung (Xidian Univ.), Y. Liu, M. Li (Oppo) |
| JVET-AA0076 | AHG11: RPR-Based Super-Resolution Guided by Partition Information | Q. Han, C. Jung (Xidian Univ.), Y. Liu, M. Li (Oppo) |
| JVET-AA0084 | AHG11: Neural Network based Super Resolution for Video Coding Using Multiple Side Information | R. Chang, L. Wang, X. Xu, S. Liu (Tencent) |
| Inter Prediction |  |  |
| JVET-AA0082 | AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement | Z. Liu, X. Xu, S. Liu (Tencent), J. Jia, Z. Chen (Wuhan Univ.) |
| End-to-End |  |  |
| JVET-AA0063 | AHG11: A hybrid codec using E2E image coding combined with VVC video coding | Y. He, B. Wang, E. Alshina, J. Sauer |
| Software |  |  |
| JVET-AA0086 | AHG11: Small Ad-hoc Deep-Learning Library (SADL) update | F. Galpin, T. Dumas, P. Bordes, E. François (InterDigital) |
| Cross Checks |  |  |

# Recommendations

The AHG recommends:

- Review all input contributions.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.