| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0023-v3 |
| --- | --- |

| Title: | EE1: Summary of Exploration Experiments on Neural Network-based Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, W. Chen, F. Galpin, Y. Li, Z. Ma, H. Wang, and L.Wang | Tel: Email: | elena.alshina@huawei.com chenwei06@kwai.com Franck.Galpin@InterDigital.com yue.li@bytedance.com mazhan@nju.edu.cn hongtaow@qti.qualcomm.com liqiangwang@tencent.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document summarizes Exploration Experiment 1 (EE1) tests performed between the JVET-Z and JVET-AA meetings to evaluate Neural Network-based Video Coding (NNVC) technologies, analyze their performance, and analyze their complexity aspects. In NN-based in-loop filter category BD-rate gain over AhG11 anchor goes up to 9~10% (depending on complexity). By content adaptive post-filter gain of 5% over AhG11 anchor was demonstrated. In Super Resolution category 1.5 % BD-rate average gain over AhG11 anchor can be achieved (among them more than 3% for 4K content). End-to-End AI video codec in this round of EE1 demonstrated almost 5% improvement, but still cannot reach VTM performance.

# Introduction

Group continues evaluation of new promising NN-based video coding technologies, answering questions and addressing suggestions from JVET members made during presentation NN-based technologies at JVET-Z meeting. Additionally in this round of EE the group focused on investigating platform-independent reproducibility (which is desirable feature for video codec [1]), drift-free loop operation by integerization of NNs, and usage of a software package which supports that.

The ultimate goal of the NNVC Exploration Experiments is the creation of AhG11 anchor based SW with examples of NN-tools implementation for further collaborative development of NN-based video coding tools. It was expected that 2 NN-based tools recommended by JVET to be added to common SW basis at JVET-Z meeting undergo procedure of cross-check for the training.

Tests were conducted in three categories: enhancement filters (in-loop or post), super-resolution and E2E NN-based video coding. One test was performed both on top of VTM and ECM anchors.

All proponents used AhG11 anchor [2] (VTM-11.0 + “newMCTF” patch [3], QP=22, 27, 32, 37, 42) and the reported template recommended by AhG11.

BD-rate gain over NNVC anchor in Random Access configuration vs computational complexity (in kMAC/pxl) and memory size for Model parameters (in MB) for typical representatives of tests in this EE1 are shown on Fig. 1 and Fig. 2 respectively.

Around 5% gain can be obtained with NN-based post-filter. Twice higher gain (~10%) can be achieved with NN-based in-loop filter. NN-based super-resolution provides gain primarily for 4K content, so in average performance is quite modest (1.5%). End-to-end video compression tested in this round of EE1 still cannot reach VTM performance.

Figure 1 Gain vs computational complexity for ttypical representatives of tests in EE1: loop-filters (EE1-1.4 “Tencent” architecture and EE1-1.6 “Bytedance” architecture), post filter (EE1-1.7) and NN-based super-resolution (EE1-2.1.)

Figure 2. Gain vs memory for typical representatives of tests in EE1: loop-filters (EE1-1.4 “Tencent” architecture and EE1-1.6 “Bytedance” architecture), post filter (EE1-1.7), NN-based super-resolution (EE1-2.1) and End-to-End AI video codec (EE1-3.1.2)

# Exploration experiments on Enhancement filters

Tests status is as follows:

- 7 tests have been completed (EE1-1.1...1.7);

- One test (EE1-1.8) was withdrawn

- 15 cross-checks registered (5 still not available), 3 cross-checks for training have been completed;

- For 1 test (EE1-1.2, JVET-AA0081) inference was successfully cross-checked; minor deviation (0.00004% BD-rate) in performance due to float point arithmetic was reported;

- Cross-checker of training for EE1-1.5 (JVET-AA0178) seems was able to complete training, performance difference with model from proponent is 0.04% (RA configuration).

- Cross-checker of training for EE1-1.5 (JVET-AA0217) seems was able to complete training, performance difference with model from proponent is ~0.02% (RA configuration).

- Cross-checker of training for EE1-1.6 (JVET-AA0181) seems was able to complete training, but performance difference with model from proponent is 0.39% (RA configuration).

- Cross-checker of training for EE1-1.5 and EE1-1.6 was not able to complete training (JVET-AA0174), but came up with practical suggestions for training procedure unification

- EE1-1.1 was partially cross-checked (only class D RA). Minor deviations {0.001%, 0.005%, 0.003%}  in BD-rate Y for EE1-1.1.1, EE1-1.1.2 and EE1-1.1.3, respectively, reported due to floating point arithmetic.

Among in-loop filters two architectures were recommended during last meeting to be considered for adoption to common SW base for NN-based video coding tools study. One architecture was originated by Tencent (later Interdigital joined design and OPPO working on further improvements for this architecture). This family of in-loop fillers is summarized in section 2.1. Another promising architecture was initiated by Bytedance further improved by Qualcomm, Ericsson and Interdigital. Test based on this architecture are summarized in section 2.2. Post-filter architecture was originated from one of Qualcomm’s loop filter proposals, Nokia adapted its weights for the post-filter use case and added the content-adaptation functionality, it is tested in EE1 as content adaptive post-filter and described in section 2.3.

## Family of in-loop filters (originated by Tencent).

There are 2 major variants: with one (model size 1.9 M parameters, Figure 3 right) and two models (total models sizes is 3.1 millions parameters, Figure 3 left). Prediction, Partitioning, QP and Slice Type (only for variant with 1 model) information are taken as extra inputs to this NN-based filters. All three color planes processed jointly.

Figure 3 NN-filter architecture from Tencent: 2 models (left) and one model (right).

Reference number for both those filters as tested during previous EE1 round are shown in grey font in Table 1. Filter parameters were partially fine-tuned, to be better with SADL implementation, so 0.2~0.3% performance improvement in RA configuration was achieved (if NN is quantized to Int 16).

Cross-check for training stage (major goal of this EE1 round) put into the box: EE1-1.5.0 (model trained and tested by proponent), EE1-1.5.1, 2, 3 (model trained by cross-checker and tested by proponent). Different cross-checkers (OPPO and Qualcomm) and two slightly different training strategies were used, but in all cases difference in within 0.04% BD-rate (in RA cfg).

Table 1 In-loop filters based in “Tencent” architecture vs VTM anchor.

|  |  |  |  |  |  | Random Access (CTC) |  |  | All Intra (CTC) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| test | Source | Precision | Total Parameters (Millions) |  | kMAC/pixel | Y | Cb | Cr | Y | Cb |  | Cr |
| EE1-1.4.1.0 | JVET-Z0094 | Float 32 | 3.1 | 401.0 |  | -8.8% | -19.9% | -19.4% | -7.4% | -16.8% | -17.3% |  |
| EE1-1.4.2.0 | JVET-Z0094 | Float (Int 16) | 3.1 | 401.0 |  | -8.8% | -19.9% | -19.4% | -7.4% | -16.8% | -17.3% |  |
| EE1-1.4.1 | JVET-AA0087 | Float 32 | 3.1 | 401.0 |  | -9.2% | -19.1% | -19.6% | -7.4% | -16.8% | -17.3% |  |
| EE1-1.4.2 | JVET-AA0087 | Float (Int 16) | 3.1 | 401.0 |  | -9.2% | -19.9% | -19.8% | -7.5% | -16.6% | -17.6% |  |
| EE1-1.3.1 | JVET-AA0122 | Float 32 | 3.1 | 401.0 |  | -9.0% | -20.0% | -19.5% | -7.4% | -16.8% | -17.3% |  |
| EE1-1.3.2 | JVET-AA0122 | Float 32 | 1.9 | 485.0 |  | -9.0% | -19.1% | -19.3% | -6.5% | -14.9% | -16.0% |  |
| EE1-1.5.0 | JVET-Z0091 | Float 32 | 1.9 | 485.0 |  | -8.68% | -18.6% | -19.0% | -6.50% | -14.9% | -16.0% |  |
| EE1-1.5.1.1 | JVET-AA0088 | Float 32 | 1.9 | 485.0 |  | -8.70% | -19.4% | -19.5% | -6.47% | -15.1% | -15.8% |  |
| EE1-1.5.1.2 | JVET-AA0088 | Float 32 | 1.9 | 485.0 |  | -8.64% | -18.0% | -19.3% | -6.35% | -14.4% | -15.8% |  |
| EE1-1.5.1.3 | JVET-AA0088 | Float 32 | 1.9 | 485.0 |  | -8.64% | -19.6% | -19.6% | -6.42% | -14.3% | -15.4% |  |
| EE1-1.5.2 | JVET-AA0088 | Float 32 | 1.9 | 485.0 |  | -8.8% | -18.9% | -19.0% | -6.5% | -15.4% | -16.5% |  |
| EE1-1.5.3 | JVET-AA0088 | Float (Int 16) | 1.9 | 485.0 |  | -9.0% | -18.5% | -19.2% | -6.5% | -15.1% | -16.4% |  |

In test EE1-1.3 (by OPPO) ~0.3% performance improvement was achieved by a slice-level adjustment for QP which used as input parameter for filtration. Instead of BaseQP (original design) selected by encoder and signalled per slice finalBaseQP will be used in filtration process.

It looks promising to combine two potential performance improvements for this architecture in the next round of EE1.

This architecture (original design, w/o improvements) was tested on top of ECM-4.0. For the reference performance vs VTM anchor is shown in grey font in Table 2. Overlap between ECM and BNN-based filter gain is about 3%. If VTM trained filter is directly applied to ECM (Test EE1-1.1.1) the performance is almost the same as if filter specifically trained for ECM artifacts (Test EE1-1.1.2). Interestingly the fine tuning of VTM trained filter for ECM generated artifacts (Test EE1-1.1.3) provides the best performance (almost 0.3% extra gain). This might indicate that longer training for “Tencent” filter might bring additional performance improvement.

Table 2 In-loop filters based in “Tencent” architecture vs ECM anchor

|  |  |  |  |  |  | Random Access (CTC) |  |  | All Intra (CTC) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| test | Source | Precision | Total Parameters (Millions) |  | kMAC/pixel | Y | Cb | Cr | Y | Cb |  | Cr |
| vs VTM | JVET-Z0091 | Float 32 | 1.9 | 485.0 |  | -8.7% | -18.6% | -19.0% | -6.5% | -14.9% | -16.0% |  |
| EE1-1.1.1 | JVET-AA0085 | Float 32 | 1.9 | 485.0 |  | -5.5% | -13.2% | -13.7% | -3.9% | -8.5% | -10.4% |  |
| EE1-1.1.2 | JVET-AA0085 | Float 32 | 1.9 | 485.0 |  | -5.6% | -13.0% | -13.0% | -3.6% | -7.4% | -8.9% |  |
| EE1-1.1.3 | JVET-AA0085 | Float 32 | 1.9 | 485.0 |  | -5.9% | -13.7% | -13.7% | -4.0% | -8.9% | -10.3% |  |

## Family of in-loop filters (originated by Bytedance)

NN-filter architecture used for tests in this sub-category is shown on Figure 4. Prediction, Boundary Strength, Partitioning and QP information are taken as extra inputs to this NN-based filters. Also attention Residual Block is used as basis element.

Figure 4 NN-filter architecture from Bytedance.

Table 3 In-loop filters based in “Bytedance” architecture vs VTM anchor.

|  |  |  |  |  |  | Random Access (CTC) |  |  | All Intra (CTC) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| test | Source | Precision | Total Parameters (Millions) |  | kMAC/pixel | Y | Cb | Cr | Y | Cb |  | Cr |
|  | JVET-Z0106 | Float 32 | 6.24 | 649.0 |  |  |  |  | -7.6% | -18.5% | -19.1% |  |
|  | JVET-Y0143 | Float 32 | 6.24 | 649.0 |  | -9.80% | -21.2% | -21.2% | -7.4% | -19.3% | -19.4% |  |
| EE1-1.2.1 | JVET-AA0081 | Float 32 | 6.21 | 635.0 |  | -9.82% | -21.2% | -21.2% | -7.6% | -18.5% | -19.1% |  |
| EE1-1.2.2 | JVET-AA0081 | Float 32 | 6.20 | 625.0 |  | -9.79% | -21.2% | -21.1% | -7.5% | -18.6% | -19.2% |  |
| EE1-1.6.1.0 | JVET-Z0113 | Float 32 | 6.24 | 649.0 |  | -10.18% | -22.2% | -22.1% | -7.24% | -19.7% | -20.0% |  |
| EE1-1.6.1 | JVET-AA0111 | Float 32 | 6.24 | 649.0 |  | -9.79% | 22.3% | -22.8% | -7.34% | -20.4% | -21.0% |  |
| EE1-1.6.2 | JVET-AA0111 | Float 32 | 6.24 | 539.0 |  | -10.2% | -22.1% | -21.8% | -7.3% | -20.2% | -20.5% |  |
| EE1-1.6.3 | JVET-AA0111 | Int 16 | 6.24 | 539.0 |  | -10.2% | -22.1% | -21.7% | -7.3% | -20.1% | -20.6% |  |

Reference numbers for performance complexity as technology was tested at the last meeting are shown in grey font. Training stage cross-check is in the box. Model was trained by cross-checker (PKU) and tested by proponent. Difference to the original proposal performance is 0.39% (in RA cfg).

The major goal of Test EE1-1.2 was to search for the better complexity performance trade-off. In Test EE1-1.2.1 partitioning and attention are removed from design. In Test EE-1.2.2 additionally Boundary Strength removes from the design. Tests were performed for AI, RA and LDB, but unfortunately only cross-checked for the all-intra configuration, due to other configurations coming in late. But at least for Intra coding those elements of this NN-filter design (partitioning, attention and BS) can be removed providing 4% saving in computation complexity, with 0.1% performance degradation. A summary diagram of the BDR and complexity can be seen below, for the AI configuration.

Test EE1-1.6.1 is supposed to be verification of training procedure. Model was trained by cross-checker and tested by original proponent. It should be noticed that model trained by proponent provides 0.4% lower gain than original model by proponent.

In Tests EE1-1.6.2 and 3 Chroma filters was forced to be the same for both Chroma components. This resulted in impressive reduction of computational complexity: 649 kMAC/pxl to 539 kMAC/pxl. This was possibility noticed last meeting and confirmed during this EE1 round.

Quantizing NN-filter to 16 bits was performed in test EE1-1.6.3. Similar to other attempts for NN-filter parameters quantizer to 16 bits integer, almost no performance variation compared to 32bit float precision was found.

## Status of training cross-check

Several attempts for the training process cross-check were taken. Status of this cross-check is summarized in Table 4 . Training was conducted by proponent and cross-checker and “cross-tested” (proponent tested model generated by cross-checker).  “TR” – training, “TS” – test, “C” – cross-checker, “P” –proponent.

It was easier to match performance of “Tencent” NN-filter. Several cross-checkers reported the problem with data loader for “Bytedance” NN-filter training scripts. Eventually one of the cross-checkers (PKU, JVET-AA0181) modified data loader and was able to complete training.

Procedure for the training cross-check requires discussion during the meeting. It is desirable to have training procedure unified for different NN-based technologies (at least for enhancement filters). This can be part of discussion about potential common SW for NNVC tools.

Table 4. Cross-check of training, Random Access test results are show.

## Family of post-filter

NN-based post-filter architecture is shown on Figure 5. Multiplier parameters (applied after adding the bias to the convolution output, but before the non-linear activation function)

σW*x+b∙m,

are content adaptive and transferred together within the video bitstream (using SEI message syntax).

Figure 5 NN-based Post-filter architecture.

Quantizing to 16 and 32 Int precision and implementation with SADL and comparison with C++ TensorFlow implementation was the purpose of the study, towards an eventual adoption into the common SW base. Results are shown in Table 4. Again it is shown that quantizing to 16 bits integer doesn’t have noticeable effect on the performance. SADL appears to be slower than TensorFlow, likely because it was not optimized for Float 32 and Int 32. Also filter architecture operates with basic blocks 72 or 24, which are not multiple of 16, so SADL was not optimized for those sizes (SIMD used instead).

Table 5 Complexity and performance on Content adaptive PNN-based Post-filter vs AhG11 anchor (different implementations).

|  |  |  |  |  |  | Random Access (CTC) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| test | Source | Precision | Total Parameters (Millions) |  | kMAC/pixel | Y | Cb | Cr | Dec. Time | Comments |  |
| EE1-1.7.0 | JVET-AA0066 | Float 32 | 0.4 | 34.0 |  | -5.0% | -18.9% | -17.3% | 46 | C++ TF |  |
| EE1-1.7.1 | JVET-AA0066 | Float 32 | 0.4 | 34.0 |  | -5.0% | -18.9% | -17.3% | 158 | SADL, Float 32 |  |
| EE1-1.7.2 | JVET-AA0066 | Int 32 | 0.4 | 34.0 |  | -5.0% | -18.9% | -17.3% | 336 | SADL, Int 32 |  |
| EE1-1.7.3 | JVET-AA0066 | Int 16 | 0.4 | 34.0 |  | -4.6% | -17.0% | -16.1% | 304 | SADL, Int 16 |  |

## Open question on kMAC/pxl computation

For all NN-based filters studied in this EE1 extended block is fed to the NN-filter. And block w/o extension is output of the filter. So multiplication operations are performed on higher amount of samples than actual output. Assuming whole picture is processed the effect of block extension is negligible. Under this assumption kMAC/pxl was computed in all tests and reported in the summary (including this EE summary). But in reality NN-filter will be implemented to process signal on a block basis. Two proponents reported kMAC/pxl both assuming picture level and block level processing (~27% higher). It is recommended to clarify which kMAC/pxl shall be computed and update NNVC CTC (or at least EE1 description) accordingly. Block based processing seems to be a reasonable assumption.

# NN-based super-resolution

In this round of EE1 only one test was performed. Adaptive coded frame size and QP selection was combined with NN-based up-sampling filter on a clean way.

(a)

(b)

Figure 6 Architecture of NN-based filter for super-resolution (a) with Advanced Residual Block (b).

Compared to previous round of EE1 (shown in Table 5 in grey font) performance improvement of ~0.2% in average is demonstrated, also computational complexity and memory size for the model size have been reduced (854  361 kMAC/pxl, 7  2.9 millions). This is definitely a good move, but still NN-based super-resolution is relatively heavy in terms of computational complexity. Likely architecture of NN-based super-resolution filter (Shown on Figure 6, 16 Advanced residual blocks(!)) can be simplified.

Table 6 BD-rate performance vs AhG11 anchor for NN-based super-resolution filter.

|  |  |  |  |  |  | Random Access (CTC) |  |  | All Intra (CTC) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| test | Source | Precision | Total Parameters (Millions) |  | kMAC/pixel | Y | Cb | Cr | Y | Cb |  | Cr |
| EE1-2.1.1 | JVET-Z0065 | Int 16 | 0.0 | 0.0 |  | -1.0% | 2.6% | 2.5% | -0.6% | 3.1% | 2.7% |  |
| EE1-2.1.0 | JVET-Z0098 | Float 32 | 7.0 | 854.0 |  | -1.3% | -0.6% | -0.8% | -1.1% | -0.4% | -0.7% |  |
| EE1-2.1 | JVET-AA0071 | Float 32 | 2.9 | 361.0 |  | -1.5% | 1.0% | 1.1% | -1.2% | 1.4% | 1.3% |  |

Gain is reported in average across all classes. It should be noticed that major gain is observed for 4K classes.

# End-to-End AI video coding

In this round of EE1 one technology was tested. NN architecture which was used for End-to-End AI video codec is shown on Figure 7.

Figure 7 End-to-End Ai video codec architecture.

The purpose of experiment was to replace first (Intra) frame coding by BPG (original design, Test EE1-3.1.1) by VTM11.0 Intra coding (Test EE1-3.1.2). Results were provided w/o class A2. Computational complexity information is not found in the submission.

RD-curves for anchor and tests cross each other, difference to target rates in 20-30% (which makes BD-rate measurement not reliable). Similar problems appeared and have been solved in NN-based super-resolution. Hopefully they will be resolved for End-to-End video coded in a future.

Table 7. Performance (w/o A2 class) and complexity for EE1 tests in End-to-End AI video codec category.

|  |  |  |  |  |  | Random Access (CTC) |  |  | All Intra (CTC) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| test | Source | Precision | Total Parameters (Millions) |  | kMAC/pixel | Y | Cb | Cr | Y | Cb |  | Cr |
| EE1-3.1.1(**) | JVET-AA0059 | Float 32 | 81.0 | ? |  | 9.8% | 9.1% | 8.0% |  |  |  |  |
| EE1-3.1.2(**) | JVET-AA0059 | Float 32 | 81.0 | ? |  | 4.3% | 4.0% | 3.7% |  |  |  |  |

(**) class A2 is missed

Test results in Table 1 confirm that replacement of BPG by VTM I-frame coding improves performance in random access configuration by more than 5%. It should be noticed that Intra frames in E2E Ai video coding solution are inserted more often compared to anchor (intra period 64 for almost all test videos).

It also should be noticed that, for example, in CVPR 2022 CLIC challenge in video coding section, none of End-to-End AI video coding solution was able to compete with VTM or ECM based solutions. Considering this only 4.3% drop relatively to AhG11 anchor (VVC) is not that bad.

Likely distinguishing features of this E2E AI based video coding bi-directional motion fusion. Motion field and residual are coded are picture.

# Conclusions

- For the training cross-check in two cases training was completed by non-proponent. Performance difference in RA cfg between model trained by cross-checker and proponent is 0.04% (for “Tencent” architecture) and 0.39% (for “Bytedance” architecture).

- .

- SADL allows transparent and reproducible implementation of NN-based tools, but so far it is slower than libtorch or/and TensorFlow.

- Performance improvement or/and complexity reduction for both two major architectures of NN-based in-loop filter have been demonstrated in this EE1 round.

- Content adaptive NN-based post-filter provides 5% gain with moderate complexity (but content dependent parameters).

- End-to-End AI based video coding still cannot reach VVC performance, but progressing quickly.

- The performance NN-based super-resolution was improved by 0.2%, also design was significantly simplified.

# Recommendation

- Discuss cross-check for training stage and re-fine procedure during the JVET-AA meeting. Encourage further efforts on training stage verification.

- Discuss the adoption and conditional adoption of tools into the common SW base.

- Encourage modifications of SADL library for missing accelerated path and features.

- Computational complexity measurement (kMAC/pxl) shall be clarified and aligned with actual implementation (assumption for the block-base implementation is recommended).

- More fruitful ideas are expected from EE1-related proposals, which shall be reviewed in details.

- Select promising technologies among EE1 to undergo training stage cross-check during next EE1 round.