| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0052-v2 |
| --- | --- |

| Title: | AHG13: On VSEI film grain profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Muhammed Coban, Marta Karczewicz Miloš Radosavljević Mickael Raulet | Email: | yonghe@qti.qualcomm.com, milosr@xiaomi.com m.raulet@aterme.com |
| Source: | Qualcomm Inc., Xiaomi, Aterme |  |  |

_____________________________

# Abstract

This contribution proposes two VSEI profiles, frequency filtering film grain synthesis profile and auto-regressive film grain synthesis profile, for the film grain characteristics (FGC) SEI message. The profiles specify constraints on FGC SEI and the requirements for the capabilities needed to enable the use of FGC SEI.

# Introduction

In previous JVET meeting, JVET-Z0132 proposed to explore the potential approaches to encourage the use of FGC SEI message based on two facts,

- An increased market demand for film grain synthesis technologies

- An alternative film grain synthesis model was broadly deployed recently

AHG13 reported three kinds of conformance:

- Hard conformance, an implementer must follow a specific set of guidelines for an expected output

- Soft conformance, an implementer should follow the guidelines but may have other options for implementation

- Not required, implementations are completely optional

There are currently no required implementations for the decoder to apply FGC SEI based film grain synthesis, and no indication of the decoder implementations for capability exchange and session negotiation. As a result, the FGC SEI based film grain synthesis technologies have not been practiced in most AVC, HEVC and VVC decoders.

In the previous JVET meeting discussions, it is generally agreed that the film grain synthesis should allow some degree of freedom to implementers, so hard conformance would be inappropriate.

A soft conformance example is the auto-regressive film grain synthesis model specified in AOM, it has been broadly implemented in majority decoders recently.

There are various film grain synthesis implementations provided in the film grain synthesis technical report, each targets specific use cases and design benefits with the similar perceptual quality. It would be desirable to specify constraints and soft conformance requirements for the capability needed to enable the use of FGC SEI message.

# Proposal

This contribution proposes two VSEI profiles, frequency filtering film grain synthesis profile and auto-regressive film grain synthesis profile, for the FGC SEI.

It is proposed to adopt the provided specification text to VSEI amendment.

# Patent rights declaration(s)

Qualcomm Inc. Xiaomi and Aterme, Inc. do not have any current or pending patent rights relating to the technology described in this contribution.