| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: by teleconference, 13–22 July 2022 | Document: JVET-AA0054-v2 |
| --- | --- |

| Title: | AHG9: On Neural-network Post-filter Characteristics SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Some modifications are proposed to the Neural-network post-filter characteristics SEI. Message. Following is proposed:

- Proposal 1: It is proposed to specify signalling of external URI information in Neural network post-filter characteristics SEI message. It is asserted that this allows defining neural network information externally.

- Proposal 2: It is proposed to signal one flag instead of two flags (nnpfc_out_sub_width_c_flag and nnpfc_out_sub_height_c_flag) to specify output chroma format information. This saves 1 bit.

- Proposal 3: A sanity check constraint is proposed to disallow signalling invalid values. In particular, it is proposed to be disallowed to signal nnpfc_purpose equal to 2 or 4 indicating upsampling of chroma format, when the input is monochrome or 4:4:4.

# Introduction

JVET-Z2006 [1] specifies the syntax and semantics for Neural network post-filter characteristics SEI message and for neural-network post-filter activation SEI message. Neural network post-filter characteristics SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

# Proposal

In this section we describe the proposed syntax and semantics. The proposed changes are shown in red color compared to JVET-Z2006-v1.

## Proposal 1

JVET-Z2006-v1 does not include a method to specify neural network post filter data via an external URI. In JVET-Z0052 such a URI is signalled by syntax elements nnrpf_uri[ i ].

It is asserted that the goal of specifying an URI to point to external neural network data is to allow different formats of neural network post processing filters to be defined by external entities based on their application/ preference. However it is asserted that only signaling a URI with nnrpf_uri[ i ] syntax elements does not specify what is the format of neural network data in the specified URI. Thus it is asserted that the URI nnrpf_uri[ i ] does not uniquely identify how to interpret the neural network data pointed by it.

It is proposed that an additional URI be signalled to uniquely identify the identity and format of the neural network data specified by nnrpf_uri[ i ] URI. For this purpose, it is proposed that a tag URI be signalled using syntax elements nnrpf_uri_tag[ i ], which are signalled only when nnrpf_mode_idc specifies that the neural network data is specified via external URI, i.e. only when nnrpf_uri[ i ] syntax elements are signalled. nnrpf_uri_tag[ i ] elements represent a 'tag' URI, which allows uniquely defining the format of neural network data specified by nnrpf_uri[ i ] values without needing a central registration authority.

Thus, in the proposed approach:

- A URI is signalled to externally specified neural network post filter information.

- A Tag URI is signalled to identify the identity and format of the externally specified neural network post filter information.

8.28 Neural-network post-filter characteristics SEI message

8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| nnpfc_purpose | ue(v) |
| if( nnpfc_purpose = = 2 ¦ ¦ nnpfc_purpose = = 4 ) { |  |
| nnpfc_out_sub_width_c_flag | u(1) |
| nnpfc_out_sub_height_c_flag | u(1) |
| } |  |
| if( nnpfc_purpose = = 3 ¦ ¦ nnpfc_purpose = = 4 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| /* input and output formatting */ |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_sample_idc | ue(v) |
| if( nnpfc_inp_sample_idc = = 4 ) |  |
| nnpfc_inp_tensor_bitdepth_minus8 | ue(v) |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_out_sample_idc | ue(v) |
| if( nnpfc_out_sample_idc = = 4 ) |  |
| nnpfc_out_tensor_bitdepth_minus8 | ue(v) |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_padding_type | ue(v) |
| nnpfc_complexity_idc | ue(v) |
| if( nnpfc_complexity_idc > 0 ) |  |
| nnpfc_complexity_element( nnpfc_complexity_idc ) |  |
| } |  |
| if( nnrpf_mode_idc = = 2 ) { |  |
| i = 0 |  |
| do |  |
| nnrpf_uri_tag[ i ] | b(8) |
| while( nnrpf_uri_tag[ i++ ] != 0 ) |  |
| i = 0 |  |
| do |  |
| nnrpf_uri[ i ] | b(8) |
| while( nnrpf_uri[ i++ ] != 0 ) |  |
| } |  |
| /* filter specified or updated by ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

    - Neural-network post-filter characteristics SEI message semantics

This SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

Use of this SEI message requires the definition of the following variables:

–	Cropped decoded output picture width and height in units of luma samples, denoted herein by InpPicWidthInLumaSamples and InpPicHeightInLumaSamples, respectively.

–	Luma sample array CroppedYPic[ y ][ x ] and chroma sample arrays CroppedCbPic[ y ][ x ] and CroppedCrPic[ y ][ x ], when present, of the cropped decoded output picture for vertical coordinates y and horizontal coordinates x, where the top-left corner of the sample array has coordinates y equal to 0 and x equal to 0.

–	Bit depth BitDepthY for the luma sample array of the cropped decoded output picture.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the cropped decoded output picture.

–	Chroma subsampling ratio relative to luma denoted as InpSubWidthC and InpSubHeightC.

–	When nnpfc_inp_order_idc is equal to 3, the initial luma quantization parameter value SliceQPY.

When this SEI message specifies a neural network that may be used as a post-processing filter, the semantics specify the derivation of the luma sample array FilteredYPic[ y ][ x ] and chroma sample arrays FilteredCbPic[ y ][ x ] and FilteredCrPic[ y ][ x ], as indicated by the value of nnpfc_out_order_idc, that contain the output of the post-processing filter.

nnpfc_id contains an identifying number that may be used to identify a post-processing filter. The value of nnpfc_id shall be in the range of 0 to 232 − 2, inclusive.

Values of nnpfc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders encountering a value of nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore it.

nnpfc_mode_idc equal to 0 specifies that the post-processing filter associated with the nnpfc_id value is determined by external means not specified in this Specification.

nnpfc_mode_idc equal to 1 specifies that the post-processing filter associated with the nnpfc_id value is a neural network represented by the ISO/IEC 15938-17 bitstream contained in this SEI message.

nnrpf_mode_idc equal to 2 specifies that the post-processing filter associated with the nnpfc_id value is a neural network identified by a specified tag Uniform Resource Identifier (URI) (nnrpf_uri_tag[ i ]) and  neural network information URI (nnrpf_uri[ i ]).

The value of nnpfc_mode_idc shall be in the range of 0 to 255, inclusive. Values of nnpfc_mode_idc greater than 1 2 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_mode_idc.

When the current CLVS contains a preceding neural-network post-filter characteristics SEI message, in decoding order, that has the same value of nnpfc_id equal to the value of nnpfc_id in this SEI message, at least one of the following conditions shall apply:

–	This SEI message has nnpfc_mode_idc equal to 1 and the same content as the preceding neural-network post-filter characteristics SEI message except that the values of nnpfc_mode_idc and nnpfc_payload_byte[ i ] may differ in order to provide a neural network update.

–	This SEI message has the same content as the preceding neural-network post-filter characteristics SEI message.

When this SEI message is the first neural-network post-filter characteristics SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, it specifies a base post-processing filter that pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS. When this SEI message is not the first neural-network post-filter characteristics SEI message, in decoding order, that has a a particular nnpfc_id value within the current CLVS, this SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or the next neural-network post-filter characteristics SEI message having that particular nnpfc_id value, in output order, within the current CLVS.

…

nnrpf_uri_tag[ i ] contains the i-th byte of a NULL-terminated UTF-8 character string which is a tag URI. The UTF-8 character string contains a URI, with syntax and semantics as specified in IETF RFC 4151, uniquely identifying the format and associated information about neural network used as the post-processing filter specified by nnrpf_uri[ i ] values.

Note: nnrpf_uri_tag[ i ] elements represent a 'tag' URI, which allows uniquely defining the format of neural network data specified by nnrpf_uri[ i ] values without needing a central registration authority.

nnrpf_uri[ i ] contains the i-th byte of a NULL-terminated UTF-8 character string, as specified in IETF Internet Standard 63. The UTF-8 character string contains a URI, with syntax and semantics as specified in IETF Internet Standard 66, identifying the neural network information (e.g. data representation) used as the post-processing filter.

## Proposal 2

In JVET-Z2006-v1, two flags nnpfc_out_sub_width_c_flag and nnpfc_out_sub_height_c_flag are signalled when nnpfc_purpose is equal to 2 or 4 (i.e. when chroma upsampling neural network filter information is signalled). These two flags specify the output chroma information.

It is proposed to signal one flag instead of two flags (nnpfc_out_sub_width_c_flag and nnpfc_out_sub_height_c_flag) to specify output chroma format information. It is asserted that one flag is sufficient for the required derivations. This saves 1 bit and avoids to unnecessarily signalling of more syntax elements than required.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| .. | ue(v) |
| if( nnpfc_purpose = = 2 ¦ ¦ nnpfc_purpose = = 4 ) { |  |
| nnpfc_out_sub_c_flag | u(1) |
| nnpfc_out_sub_width_c_flag | u(1) |
| nnpfc_out_sub_height_c_flag | u(1) |
| } |  |
| .. |  |
| } |  |

nnpfc_out_sub_c_flag equal to 1 specifies that outSubWidthC is equal to 1 and outSubHeightC is equal to 1. nnpfc_out_sub_c_flag equal to 0 specifies that outSubWidthC is equal to 2 and outSubHeightC is equal to 1. When nnpfc_out_sub_c_flag is not present, outSubWidthC is inferred to be equal to InpSubWidthC and outSubHeightC is inferred to be equal to InpSubHeightC. If InpSubWidthC is equal to 2 and InpSubHeightC is equal to 1, nnpfc_out_sub_c_flag shall not be equal to 0.

nnpfc_out_sub_width_c_flag and nnpfc_out_sub_height_c_flag are used to derive the variables outSubWidthC and outSubHeightC, respectively, which specify the chroma subsampling ratio relative to luma in the picture resulting from the post-processing filtering. When not present, nnpfc_out_sub_width_c_flag and nnpfc_out_sub_height_c_flag are both inferred to be equal to 0. When nnpfc_out_sub_width_c_flag and nnpfc_out_sub_height_c_flag are present, the sum of nnpfc_out_sub_width_c_flag and nnpfc_out_sub_height_c_flag shall be greater than 0.

outSubWidthC = InpSubWidthC − nnpfc_out_sub_width_c_flag	(73)

outSubHeightC = InpSubHeightC − nnpfc_out_sub_height_c_flag	(74)

It is a requirement of bitstream conformance that outSubWidthC and outSubHeightC are both greater than 0.

## Proposal 3

It is asserted that currently it is allowed to signal nnpfc_purpose equal to 2 or 4 indicating upsampling of chroma format, even when the input is monochrome or 4:4:4. It is asserted that chroma upsampling should not be allowed to be signalled when input chroma format is monochrome or 4:4:4. A constraint is proposed to prevent this.

nnpfc_purpose indicates the purpose of post-processing filter as specified in Table 20. The value of nnpfc_purpose shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_purpose that do not appear in Table 20 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_purpose.

When InpSubWidthC is equal to 1 and InpSubHeightC is equal to 1, nnpfc_purpose shall not be equal to 2 or 4.

# Examples

Some examples are provided in this section regarding the proposed usage of the tag URI (proposal 1).

As an example an external XYZ corporation may define a nnrpf_uri_tag[ i ] value which specifies a tag URI:

tag:xyzcorp.com,2022:nnfilter1

Then the data format for the neural network post processing filter information specified by URI formed from nnrpf_uri[ i ] values will be as documented by external XYZ corporation for their unique tag (tag:xyzcorp.com,2022:nnfilter1).

The same external XYZ corporation may define additional filters which may be identified by example tag URIs like:

tag:xyzcorp.com,2022:nnfilter2

tag:xyzcorp.com,2022:nnfilter3

tag:xyzcorp.com,2023:adv-nnfilterA

Similarly, a neural network post filter defined by another GHJ corporation may use example tag URIs as follows.

tag:ghj.com,2022:nnf/pp1

tag:ghj.com,2022:nnf/pp2

# Conclusion

This document proposes modifications to neural network post processing characteristics SEI. It is proposed to adopt the proposals in the next version of “Additional SEI messages for VSEI” draft text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-Z2006-v1, Additional SEI messages for VSEI (Draft 1), Online, April 2022

[2] JVET-Z0052, AHG9: NNR post-filter SEI message, Online, April 2022