| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 27th Meeting: by teleconference, 13–22 July 2022 | Document: JVET-AA0055-v2 |
| --- | --- |

| Title: | AHG9: Comments on Neural-network Post-filter Characteristics SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Ahmed Sidiya | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document contains comments on Neural-network post-filter characteristics SEI message. Following is proposed:

- Proposal A: It is proposed to allow signalling additional padding types for the neural network.

- Proposal B: A new value is proposed to indicate the type of neural network parameters. Also, the syntax element nnpfc_parameter_type is changed from a flag to a two-bit idc for this.

- Proposal C: It is proposed to signal frame rate upsampling as an additional purpose for post processing filter. Additionally, it is proposed to conditionally signal two syntax elements for frame rate upsampling purpose.

- Proposal D: Some sanity check constraints are proposed for patch size related syntax elements (nnpfc_constant_patch_size_flag, nnpfc_patch_width_minus1, nnpfc_patch_height_minus1) to disallow signalling invalid values.

In v2 proposal A, B text is updated as agreed.

# Introduction

JVET-Z2006 [1] specifies the syntax and semantics for Neural network post-filter characteristics SEI message and for neural-network post-filter activation SEI message. Neural network post-filter characteristics SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

This document contains proposals for additional signalling of information in Neural-network post-filter characteristics SEI. Following is proposed:

- Proposal A: It is proposed to allow signalling additional padding types for the neural network.

- Proposal B: A new value is proposed to indicate the type of neural network parameters. Also, the syntax element nnpfc_parameter_type is changed from a flag to a two-bit idc for this.

- Proposal C: It is proposed to signal frame rate upsampling as an additional purpose for post processing filter. Additionally, it is proposed to conditionally signal two syntax elements for frame rate upsampling purpose.

- Proposal D: Some sanity check constraints are proposed for patch size related syntax elements (nnpfc_constant_patch_size_flag, nnpfc_patch_width_minus1, nnpfc_patch_height_minus1) to disallow signalling invalid values.

# Proposal

In this section we describe the proposed syntax and semantics.

## Proposal A

It is proposed to allow signalling additional padding types for the neural network.

It is asserted that it is common to use wrap-around and fixed padding (of a specific padding value) for the input or features of a neural networks. Depending upon the post filter, one of these padding type may be used instead of currently define padding types. As an example, wrap-around padding may be used for omnidirectional content. Also fixed padding has been shown effective in certain type of neural networks (e.g. GAN).

The proposed changes are shown in red color compared to JVET-Z2006-v1.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| nnpfc_purpose | ue(v) |
| if( nnpfc_purpose = = 2 ¦ ¦ nnpfc_purpose = = 4 ) { |  |
| nnpfc_out_sub_width_c_flag | u(1) |
| nnpfc_out_sub_height_c_flag | u(1) |
| } |  |
| if( nnpfc_purpose = = 3 ¦ ¦ nnpfc_purpose = = 4 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| /* input and output formatting */ |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_sample_idc | ue(v) |
| if( nnpfc_inp_sample_idc = = 4 ) |  |
| nnpfc_inp_tensor_bitdepth_minus8 | ue(v) |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_out_sample_idc | ue(v) |
| if( nnpfc_out_sample_idc = = 4 ) |  |
| nnpfc_out_tensor_bitdepth_minus8 | ue(v) |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ){ |  |
| nnpfc_luma_padding_val | ue(v) |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| nnpfc_complexity_idc | ue(v) |
| if( nnpfc_complexity_idc > 0 ) |  |
| nnpfc_complexity_element( nnpfc_complexity_idc ) |  |
| } |  |
| /* filter specified or updated by ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

nnpfc_padding_type specifies the process of padding when referencing sample locations outside the boundaries of the cropped decoded output. The value of nnpfc_padding_type shall be in the range of 0 to 15, inclusive.

Table 22 – Informative description of nnpfc_padding_type values

| nnpfc_padding_type | Description |
| --- | --- |
| 0 | zero padding |
| 1 | replication padding |
| 2 | reflection padding |
| 3 | wrap-around padding |
| 4 | fixed padding |
| 35..15 | reserved |

nnpfc_luma_padding_val specifies the luma value to be used for padding when nnpfc_padding_type is equal to 4.

nnpfc_cb_padding_val specifies the Cb value to be used for padding when nnpfc_padding_type is equal to 4.

nnpfc_cr_padding_val specifies the Cr value to be used for padding when nnpfc_padding_type is equal to 4.

The function InpSampleVal( y, x, picHeight, picWidth, croppedPic ) with inputs being a vertical sample location y, a horizontal sample location x, a picture height picHeight, a picture width picWidth, and sample array croppedPic returns the value of sampleVal derived as follows:

if( nnpfc_padding_type  = =  0 )
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal = 0
	else
		sampleVal = croppedPic[ y ][ x ]		(80)
else if( nnpfc_padding_type  = =  1 )
	sampleVal = croppedPic[ Clip3( 0, picHeight − 1, y ) ][ Clip3( 0, picWidth − 1, x ) ]
else if( nnpfc_padding_type  = =  2 ) /* nnpfc_padding_type  = =  2 */
	sampleVal = croppedPic[ Reflect( picHeight − 1, y ) ][ Reflect( picWidth − 1, x ) ]
else if( nnpfc_padding_type  = =  3 ) 
	if(y >= 0  &&  y < picHeight)
		sampleVal = croppedPic[ y ][ Wrap( picWidth − 1, x ) ] 
else if( nnpfc_padding_type  = =  4 ) 
		if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
			sampleVal[ 0 ]  = nnpfc_luma_padding_val			sampleVal[ 1 ]  = nnpfc_cb_padding_val
			sampleVal[ 2 ] = nnpfc_cr_padding_val

		else
			sampleVal = croppedPic[ y ][ x ]

Where

# In clause 5.8, add the following function definition immediately before the definition of the function Round( x ) and adjust equation indices accordingly:

Reflect( y, z ) =  Min( -z, y );z < 0Max( y-(z-y), 0 );z > yz;otherwise

Wrap(y,z) =   Max( 0,  y+z+1 );z < 0Min( y, z-y-1 );z > yz;otherwise

## Proposal B

The neural network weights and biases may use different number of bits.

We propose to include a new value to indicate the type of neural network parameters.

Binary parameters represent binary neural networks where the weights and biases take on only two values. An example is a Binarized Neural Network (BNN) which uses weights and biases constrained to be -1 or 1.

It is asserted that current nnpfc_parameter_type_flag does not specifically support indicating use of binary neural networks. It is proposed to modify the nnpfc_parameter_type_flag to an indicator to support signalling this.

The proposed changes are shown in red color compared to JVET-Z2006-v1.

| nnpfc_complexity_element( nnpfc_complexity_idc ) { | Descriptor |
| --- | --- |
| if( nnpfc_complexity_idc = = 1 ) { |  |
| nnpfc_parameter_type_ flagidc | u(21) |
| if (nnpfc_parameter_type_idc ! = 2) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(8) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| } |  |
| } |  |

nnpfc_parameter_type_flagidc equal to 0 indicates that the neural network uses only integer parameters. nnpfc_parameter_type_flagidc equal to 1 indicates that the neural network may use floating point or integer parameters.

nnpfc_parameter_type_idc equal to 2 indicates that the neural network uses only binary parameters.

nnpfc_parameter_type_idc equal to 3 is reserved.

nnpfc_log2_parameter_bit_length_minus3 equal to 0, 1, 2, and 3 indicates that the neural network does not use parameters of bit length greater than 8, 16, 32, and 64, respectively. When nnpfc_parameter_type_idc is present and nnpfc_log2_parameter_bit_length_minus3 is not present the neural network does not use parameters of bit length greater than 1.

## Proposal C

It is asserted that the neural network post-processing filter can be used to upsample the frame rate for the decoded video. Thus it is proposed to allow signalling “frame rate upsampling” as an additional purpose for the post processing filter.

A neural network post-processing filter used for frame rate upsampling may take as input two or more decoded output pictures and generate one or more interpolated pictures in between those pictures.

The proposed changes are shown in red color compared to JVET-Z2006-v1.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| … | ue(v) |
| nnpfc_purpose | u(1) |
| if (nnpfc_purpose = = 5) { |  |
| nnpfc_number_of_input_pictures_minus1 | ue(v) |
| nnpfc_number_of_interpolated_pictures_minus1 | ue(v) |
| } |  |

nnpfc_purpose indicates the purpose of post-processing filter as specified in Table 20. The value of nnpfc_purpose shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_purpose that do not appear in Table 20 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore SEI messages that contain reserved values of nnpfc_purpose.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Unknown or unspecified |
| 1 | Visual quality improvement |
| 2 | Chroma upsampling from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format |
| 3 | Increasing the width or height of the cropped decoded output picture without changing the chroma format |
| 4 | Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |
| 5 | Frame rate upsampling |

nnpfc_number_of_input_pictures_minus1 plus 1 specifies the number of decoded output pictures used as input for the post-processing filter.

nnpfc_number_of_interpolated_pictures_minus1 plus 1 specifies the number of interpolated pictures generated by the post-processing filter.

## Proposal D

Some sanity check constraints are proposed for patch size related syntax elements (nnpfc_constant_patch_size_flag, nnpfc_patch_width_minus1, nnpfc_patch_height_minus1) to disallow signaling invalid values.

The proposed changes are shown in red color compared to JVET-Z2006-v1.

nnpfc_constant_patch_size_flag equal to 0 specifies that the post-processing filter accepts any  patch size that is a positive integer multiple of the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. When nnpfc_constant_patch_size_flag is equal to 0 the patch size width shall be less than or equal to InpPicWidthInLumaSamples. When nnpfc_constant_patch_size_flag is equal to 0 the patch size height shall be less than or equal to InpPicHeightInLumaSamples.  nnpfc_constant_patch_size_flag equal to 1 specifies that the post-processing filter accepts exactly the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input.

nnpfc_patch_width_minus1 + 1, when nnpfc_constant_patch_size_flag equal to 1, specifies the horizontal sample counts of the patch size required for the input to the post-processing filter. When nnpfc_constant_patch_size_flag is equal to 0, any positive integer multiple of ( nnpfc_patch_width_minus1 + 1 ) may be used as the horizontal sample counts of the patch size used for the input to the post-processing filter. The value of nnpfc_patch_width_minus1 shall be in the range of 0 to 32766, inclusive. nnpfc_patch_width_minus1 shall be less than InpPicWidthInLumaSamples.

nnpfc_patch_height_minus1 + 1, when nnpfc_constant_patch_size_flag equal to 1, specifies the vertical sample counts of the patch size required for the input to the post-processing filter. When nnpfc_constant_patch_size_flag is equal to 0, any positive integer multiple of ( nnpfc_patch_height_minus1 + 1 ) may be used as the vertical sample counts of the patch size used for the input to the post-processing filter. The value of nnpfc_patch_height_minus1 shall be in the range of 0 to 32766, inclusive. nnpfc_patch_height_minus1 shall be less than InpPicHeightInLumaSamples.

# Conclusion

This document proposes modifications to neural network post processing characteristics SEI. It is proposed to adopt the proposal in the next version of “Additional SEI messages for VSEI” draft text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-Z2006-v1, Additional SEI messages for VSEI (Draft 1), Online, April 2022