| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0057-v1 |
| --- | --- |

| Title: | EE2-1.1a: Convolutional cross-component intra prediction model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pekka Astola Jani Lainema Ramin G. Youvalari Alireza Aminlou Krit Panusopone | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports results of EE2 Test 1.1a on convolution cross-component model (CCCM) prediction. The proposed 7-tap convolutional filter maps luma values into chroma values when a CCCM prediction mode is activated by a PU level flag. The filter input consists of 5 spatial luma samples, a nonlinear term and a bias term. Filter coefficients are derived for each chroma block separately using regression based MSE minimization on reference samples in the PU’s neighborhood. The impact on coding efficiency and runtimes over ECM-5.0 is reportedly {for Y, U, V, EncT, DecT}:

AI { -1.39%, -2.81%, -2.83%, 101%, 101% }, RA { -0.79%, -2.10%, -2.42%, 101%, 100% }

The gains are asserted to be larger on higher resolution sequences and on TGM content, with AI-TGM { -2.86 %, -5.23 %, -5.07%, 103%, 101% } and RA-TGM { -1.59%, -2.47%, -2.38%, 102%, 100% }.

# Studied approach (JVET-Z0064)

## Introduction

It is proposed to apply convolutional cross-component model (CCCM) to predict chroma samples from reconstructed luma samples in a similar spirit as done by the current CCLM modes. As with CCLM, the reconstructed luma samples are down-sampled to match the lower resolution chroma grid when chroma sub-sampling is used.

Also, similarly to CCLM, there is an option of using a single model or multi-model variant of CCCM. The multi-model variant uses two models, one model derived for samples above the average luma reference value and another model for the rest of the samples (following the spirit of the CCLM design). Multi-model CCCM mode can be selected for PUs which have at least 128 reference samples available.

## Convolutional filter

The proposed convolutional 7-tap filter consist of a 5-tap plus sign shape spatial component, a nonlinear term and a bias term. The input to the spatial 5-tap component of the filter consists of a center (C) luma sample which is collocated with the chroma sample to be predicted and its above/north (N), below/south (S), left/west (W) and right/east (E) neighbors as illustrated below.

Figure 1: Spatial part of the convolutional filter

The nonlinear term P is represented as power of two of the center luma sample C and scaled to the sample value range of the content:

P = ( C*C + midVal ) >> bitDepth

That is, for 10-bit content it is calculated as:

P = ( C*C + 512 ) >> 10

The bias term B represents a scalar offset between the input and output (similarly to the offset term in CCLM) and is set to middle chroma value (512 for 10-bit content).

Output of the filter is calculated as a convolution between the filter coefficients ci and the input values and clipped to the range of valid chroma samples:

predChromaVal = c0C + c1N + c2S + c3E + c4W + c5P + c6B

## Calculation of filter coefficients

The filter coefficients ci are calculated by minimising MSE between predicted and reconstructed chroma samples in the reference area. Figure 2 illustrates the reference area which consists of 6 lines of chroma samples above and left of the PU. Reference area extends one PU width to the right and one PU height below the PU boundaries. Area is adjusted to include only available samples. The extensions to the area shown in blue are needed to support the “side samples” of the plus shaped spatial filter and are padded when in unavailable areas.

Figure 2: Reference area (with its paddings) used to derive the filter coefficients

The MSE minimization is performed by calculating autocorrelation matrix for the luma input and a cross-correlation vector between the luma input and chroma output. Autocorrelation matrix is LDL decomposed and the final filter coefficients are calculated using back-substitution. The process follows roughly the calculation of the ALF filter coefficients in ECM, however LDL decomposition was chosen instead of Cholesky decomposition to avoid using square root operations. The proposed approach uses only integer arithmetic.

## Bitstream signalling

Usage of the mode is signalled with a CABAC coded PU level flag. One new CABAC context was included to support this. When it comes to signalling, CCCM is considered a sub-mode of CCLM. That is, the CCCM flag is only signalled if intra prediction mode is LM_CHROMA_IDX (to enable single mode CCCM) or MMLM_CHROMA_IDX (to enable multi-model CCCM).

## Encoder operation

The encoder performs two new RD checks in the chroma prediction mode loop, one for checking single model CCCM mode and one for checking multi-model CCCM mode.

# Experimental results

Tables below summarize the coding efficiency and runtime impact of the proposed method in ECM 5.0 using the JVET common test conditions.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.22 % | -2.55 % | -4.80 % | 101 % | 100 % |
| Class A2 | -5.60 % | -4.24 % | -7.04 % | 101 % | 100 % |
| Class B | -0.58 % | -3.43 % | -1.64 % | 101 % | 101 % |
| Class C | -0.13 % | -0.87 % | -0.76 % | 103 % | 101 % |
| Class E | -0.35 % | -3.18 % | -1.41 % | 102 % | 102 % |
| Overall | -1.39 % | -2.81 % | -2.83 % | 101 % | 101 % |
| Class D | -0.10 % | -0.54 % | -0.64 % | 102 % | 100 % |
| Class F | -0.88 % | -3.20 % | -2.81 % | 102 % | 100 % |
| Class TGM | -2.86 % | -5.23 % | -5.07 % | 103 % | 101 % |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.15 % | -2.00 % | -3.94 % | 101 % | 99 % |
| Class A2 | -2.11 % | -1.97 % | -3.79 % | 101 % | 99 % |
| Class B | -0.35 % | -3.45 % | -2.15 % | 101 % | 100 % |
| Class C | -0.07 % | -0.58 % | -0.61 % | 101 % | 101 % |
| Class E |  |  |  |  |  |
| Overall | -0.79 % | -2.10 % | -2.42 % | 101 % | 100 % |
| Class D | -0.04 % | -0.75 % | -0.53 % | 100 % | 99 % |
| Class F | -1.14 % | -3.07 % | -2.84 % | 101 % | 99 % |
| Class TGM | -1.59 % | -2.47 % | -2.38 % | 102 % | 100 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).