| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0058 |
| --- | --- |

| Title: | EE2-2.7: GPM adaptive blending (JVET-Z0059, JVET-Z0137) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani, Haruhisa Kato, Kyohei Unno, Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan Ning Yan, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Che-Wei Kuo, Xianglin Wang (Kwai) | Tel: Email: Email: | +81 70 3508 5829 yo-kidani@kddi.com yanning03@kwai.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution reports the results of EE2 Test 2.7 on geometric partitioning mode (GPM) adaptive blending. Specifically, besides the existing blending area (i.e., 2 samples on each side of the GPM partition split boundary), four extra blending area sizes, i.e., quarter, half, double, and quadrupole of the existing area size, are added for the GPM mode. The selected blending area size is signalled at CU-level from encoder to decoder. Additionally, block-size dependent signaling will be tested to reduce the signaling overhead and encoder runtime.

Simulation test results of Test 2.7a and Test 2.7b on the top of ECM-5.0 in RA and LB are reported as follows:

- EE2 Test 2.7a:

  - Overall

    - RA (Overall): BD-rate Y/U/V: -0.25%/-0.29%/-0.33%; EncT: %; DecT: %

    - LB (Overall): BD-rate Y/U/V: -0.33%/-0.39%/-0.39%; EncT: 103%; DecT: 99%

  - ClassF

    - RA (Overall): BD-rate Y/U/V: -%/-%/-%; EncT: %; DecT: %

    - LB (Overall): BD-rate Y/U/V: -%/-%/-%; EncT: %; DecT: %

  - Class TGM

    - RA (Overall): BD-rate Y/U/V: -%/-%/-%; EncT: %; DecT: %

    - LB (Overall): BD-rate Y/U/V: -%/-%/-%; EncT: %; DecT: %

- EE2 Test 2.7b:

  - Overall

    - RA (Overall): BD-rate Y/U/V: -0.17%/-0.12%/-0.16%; EncT: %; DecT: %

    - LB (Overall): BD-rate Y/U/V: -0.23%/-0.55%/-0.29%; EncT: 101%; DecT: 100%

  - ClassF

    - RA (Overall): BD-rate Y/U/V: -%/-%/-%; EncT: %; DecT: %

    - LB (Overall): BD-rate Y/U/V: -%/-%/-%; EncT: %; DecT: %

  - Class TGM

    - RA (Overall): BD-rate Y/U/V: -%/-%/-%; EncT: %; DecT: %

    - LB (Overall): BD-rate Y/U/V: -%/-%/-%; EncT: %; DecT: %

Introduction

In current ECM design, geometric partitioning mode (GPM) generates the final prediction samples with weighted average, i.e., blending of the two different inter predictions or inter and intra predictions. The two integer blending matrices (W0 and W1) are utilized for the GPM blending process. The weights in the GPM blending matrices contain the value range of [0, 8], and are derived from the ramp function based on the displacement from a predicted sample position to the GPM partitioning boundary as shown in Figure 1.

Figure 1 Exemplified generation of a blending weight W0 of GPM

Specifically, the weights are given by a ramp function with the displacement (d) and the blending area size (τ) as shown in Figure 2.

Figure 2 The ramp function for the weights for GPM blending based based on the displacement (d) from a predicted sample position to the GPM partitioning boundary and the blending area size (τ).

In VVC spec and current ECM design, the blending area size is fixed (i.e., τ = 2 samples on each side of the GPM partition split boundary) while the fixed size is not always optimal for various types of video sequences, having room to improve the prediction accuracy.

To tackle the problem, GPM adaptive blending methods were proposed in JVET-Z0059 [1] and JVET-Z0137 [2]. In this contribution, a total of two tests on GPM adaptive blending are conducted according to the latest EE2 description [3].

Proposed methods

Test 2.7a: GPM adaptive blending

In this test, the GPM adaptive blending scheme is investigated. Specifically, besides the existing blending area, extra blending area sizes, i.e., quarter, half, double, and quadrupole of the existing area size (τ/4, τ/2, 2τ, and 4τ), are added for the GPM mode as shown in Figure 3. The selected blending area size is signalled at CU-level from encoder to decoder. Furthermore, the extended weighting precision is proposed, that is the maximum value of the weighs is changed from 8 to 32 to accommodate the extended blending area sizes.

Figure 3 The ramp function for the weights for GPM blending based based on the displacement (d) from a predicted sample position to the GPM partitioning boundary and the blending area size (τ).

Test 2.7b: Test 2.7a + Block-size dependent signalling

In the test, the block-size dependent signalling is additionally introduced into the Test 2.7a method to reduce the signalling overhead and encoder runtime. Specifically, the selectable blending area sizes are restricted based on the current block size. When the shorter side of the current block is equal to or smaller than 16 samples, the three narrower area sizes (τ/4, τ/2, and τ) can be selected; otherwise, the three wider widths τ, 2τ, and 4τ) can be selected.

Simulation results

The proposed methods are implemented on top of ECM-5.0[4]. The simulation tests were conducted following the common test condition and evaluation procedures for enhanced compression tool testing [5], and palette mode is enabled in Class F and Class TGM according to the latest EE2 description [3]. Test results of Test 2.7a and Test 2.7b under RA and LB configurations are summarized in Table 1 and Table 2. The GPM adaptive blending provides -0.25%/-0.33% coding gains in RA/LB compared to ECM-5.0 as shown in Table 1. Especially, the significant coding gains by -%/-% for Class F and Class TGM are observed in RA/LB, which are equivalent to the coding gains by SCC specific tools in EE2 summary report.

In contrast, the GPM adaptive blending plus block-size dependent signalling retains -0.17%/-0.23% coding gains of Test2.7a in RA/LB, while reduce encoder runtime %/2% in RA/LB as shown in Table 2. More detailed results can be found in the attached spreadsheets.

Table 1 Test 2.7a results over ECM-5.0.

|  |  |  | RA |  |  |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.27% | -0.41% | -0.39% | 102% | 100% |  |  |  |  |  |
| A2 | -0.07% | 0.06% | -0.03% | % | % |  |  |  |  |  |
| B | -0.20% | -0.44% | -0.41% | 101% | 99% | -0.22% | -0.40% | -0.24% | 102% | 99% |
| C | -0.42% | -0.27% | -0.42% | 101% | 99% | -0.49% | -0.49% | -0.49% | 103% | 99% |
| E |  |  |  |  |  | -0.28% | -0.23% | -0.52% | 102% | 101% |
| Overall | -0.25% | -0.29% | -0.33% | % | % | -0.33% | -0.39% | -0.39% | 103% | 99% |
| D | -0.32% | -0.33% | -0.01% | 101% | 99% | -0.45% | 0.64% | 0.63% | 102% | 100% |
| F | % | % | % | % | % | % | % | % | % | % |
| TGM | % | % | % | % | % | % | % | % | % | % |

Table 2 Test 2.7b results over ECM-5.0.

|  |  |  | RA |  |  |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A1 | -0.19% | -0.22% | -0.32% | 101% | 100% |  |  |  |  |  |
| A2 | -0.08% | 0.04% | 0.06% | % | % |  |  |  |  |  |
| B | -0.13% | -0.24% | -0.21% | 100% | 100% | -0.14% | -0.45% | -0.33% | 101% | 100% |
| C | -0.28% | 0.00% | -0.12% | 101% | 100% | -0.33% | -0.59% | -0.34% | 102% | 100% |
| E |  |  |  |  |  | -0.24% | -0.66% | -0.14% | 100% | 100% |
| Overall | -0.17% | -0.12% | -0.16% | % | % | -0.23% | -0.55% | -0.29% | 101% | 100% |
| D | -0.28% | -0.22% | -0.18% | 101% | 100% | -0.47% | 0.09% | -0.23% | 102% | 100% |
| F | % | % | % | % | % | % | % | % | % | % |
| TGM | % | % | % | % | % | % | % | % | % | % |

Supplemental results

Table 3 and Table 4 shows the Test 2.7a and Test 2.7b results for Class F and Class TGM where the palette mode is disabled, i.e., following the CTC[5]. More detailed results can be found in the attached spreadsheets.

Table 3 Test 2.7a results over ECM-5.0.

|  |  |  | RA |  |  |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| F | -0.81% | -0.66% | -0.69% | 100% | 99% | -1.08% | -0.49% | -0.85% | 102% | 99% |
| TGM | -2.51% | -1.96% | -1.87% | 99% | 97% | -3.83% | -3.19% | -3.12% | 100% | 96% |

Table 4 Test 2.7b results over ECM-5.0.

|  |  |  | RA |  |  |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| F | -0.68% | -0.50% | -0.47% | 99% | 100% | -0.91% | -0.62% | -0.42% | 101% | 99% |
| TGM | -2.40% | -1.82% | -1.73% | 99% | 97% | -3.58% | -3.00% | -2.95% | 100% | 96% |

Conclusion

In this contribution, GPM adaptive blending method were proposed. The proposed methods can bring some coding gains with a good trade-off between coding performance and complexity. It is recommended to adopt the proposed methods into the next ECM.

Reference

- Y. Kidani, H. Kato, K. Kyohei, and K. Kawamura, “Non-EE2: Adaptive width for GPM blending area,” JVET-Z0059, 20-29 April 2022.

- H. Gao, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, and X. Wang, “Non-EE2: Adaptive blending for GPM,” JVET-Z0137, 20-29 April 2022.

- V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-Y2017, by teleconference, 20-29 April 2022.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-5.0

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, by teleconference, 12-21, January. 2022.

- V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, and K. Zhang, “EE2: Summary Report on Enhanced Compression beyond VVC capability,” JVET-Z0024, by teleconference, 20-29 April 2022.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).