| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0061 |
| --- | --- |

| Title: | EE2-3.1: IBC Merge Mode with Block Vector Differences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1, Jizheng Xu2, Kai Zhang2, Mehdi Salehifar2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhangna.cynthia@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document reports results for EE2-3.1 on IBC merge mode with block vector differences (IBC-MBVD) as an extension of MMVD. In IBC-MBVD, after an IBC base candidate is selected, it is further refined by the signalled BVDs information. An IBC-MBVD flag is signalled to specify whether IBC-MBVD mode is used for a CU.

On top of the ECM5.0 (PLT=1), simulation results of the proposed methods are reported as below:

AI: Class F -0.39%,100%,101%; Class TGM -1.03%,101%,101%

RA: Class F -0.35%,100%,98%; Class TGM -0.99%,101%,100%

LB: Class F -0.35%,100%,98%; Class TGM -0.93%,100%,99%

# Introduction

In JVET- Z0152 [1], it is proposed to extend the MMVD mode to the IBC merge mode.

# Proposed methods

In this contribution, IBC merge mode with block vector differences is tested per the description of EE2.

The distance set is {1-pel, 2-pel, 4-pel, 8-pel, 12-pel, 16-pel, 24-pel, 32-pel, 40-pel, 48-pel, 56-pel, 64-pel, 72-pel, 80-pel, 88-pel, 96-pel, 104-pel, 112-pel, 120-pel, 128-pel}, and the BVD directions are two horizontal and two vertical directions.

The base candidates are selected from the first five candidates in the reordered IBC merge list. And based on the SAD cost between the template (one row above and one column left to the current block) and its reference for each refinement position, all the possible MBVD refinement positions (20×4) for each base candidate are reordered. Finally, the top 8 refinement positions with the lowest template SAD costs are kept as available positions, consequently for MBVD index coding.

# Experimental Results

The proposed methods have been implemented on top of ECM-5.0 [2]. Testing results under AI/RA/LB configurations with PLT on are summarized in Table 1. The simulations were performed following the common test conditions [3]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method compared to ECM-5.0 (PLT on).

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.39% | -0.37% | -0.39% | 100% | 101% |
| Class TGM | -1.03% | -0.93% | -0.98% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.35% | -0.08% | -0.22% | 100% | 98% |
| Class TGM | -0.99% | -1.05% | -1.14% | 101% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.35% | -1.75% | -1.32% | 100% | 98% |
| Class TGM | -0.93% | -0.94% | -1.02% | 100% | 99% |

- Conclusions

In this contribution, IBC merge mode with block vector differences is proposed. The proposed method can bring some coding gain with a good trade-off between coding performance and complexity. It is recommended to adopt the proposed method into ECM.

# References

[1] N. Zhang, J. Xu, K. Zhang, M. Salehifar, L. Zhang, “EE2-related: IBC Merge Mode with Block Vector Differences,” JVET-Z0152, Apr. 2022.

[2] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-5.0

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).