| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0062-v2 |
| --- | --- |

| Title: | EE2-3.3: Combination of EE2-3.1 and EE2-3.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1, Zhipin Deng1, Kai Zhang2, Jizheng Xu2, Mehdi Salehifar2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhangna.cynthia@bytedance.com zhipin.deng@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document reports results for EE2-3.3 on combination of EE2-3.1 and EE2-3.2. In EE2-3.1, IBC merge mode with block vector differences (a.k.a. IBC-MBVD) is introduced. In EE2-3.2, reconstruction-reordered IBC (a.k.a. RRIBC) is introduced.

On top of the ECM5.0 (PLT=1), simulation results of the proposed methods are reported as below:

AI: Class F -1.88%,103%,101%; Class TGM -4.36%,101%,	101%

RA: Class F -1.31%	,102%,101%; Class TGM -2.26%,102%,100%

LB: Class F -1.31%,102%,100%; Class TGM -1.53%,101%,99%

# Introduction

In JVET-Z0152 [1], it is proposed to extend the MMVD mode to the IBC merge mode.

In JVET-Z0159 [2], a Reconstruction-Reordered IBC (RRIBC) mode is proposed for screen content video coding to further improve the coding efficiency of IBC in ECM.

# Proposed methods

In the combination EE test, an IBC_MBVD coded block does not inherit flip type from a RRIBC coded neighbor block.

# Experimental Results

The proposed methods have been implemented on top of the ECM-5.0 [3]. Testing results under AI/RA/LB configurations with PLT on are summarized in Table 1. The simulations were performed following the common test conditions [4]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method compared to ECM-5.0 (PLT on).

|  |  | All Intra Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over ECM-5.0 |  |  |  |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |
| Class F |  | -1.88% |  | -2.19% |  | -1.91% |  | 103% |  | 101% |
| Class TGM |  | -4.36% |  | -4.24% |  | -4.22% |  | 101% |  | 101% |
|  |  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  |  | Over ECM-5.0 |  |  |  |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |
| Class F |  | -1.31% |  | -1.12% |  | -1.31% |  | 102% |  | 101% |
| Class TGM |  | -2.26% |  | -2.14% |  | -2.26% |  | 102% |  | 100% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over ECM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class F | -1.31% |  | -2.19% |  | -1.95% |  | 102% |  | 100% |  |
| Class TGM | -1.53% |  | -1.55% |  | -1.67% |  | 101% |  | 99% |  |

- Conclusions

In this contribution, combination of EE2-3.1 and EE2-3.2 is tested. The combination can bring some coding gain with a good trade-off between coding performance and complexity. It is recommended to adopt the combination method into ECM.

# References

[1] N. Zhang, J. Xu, K. Zhang, M. Salehifar, L. Zhang, “EE2-related: IBC Merge Mode with Block Vector Differences,” JVET-Z0152, Apr. 2022.

[2]	Z. Deng, K. Zhang, L. Zhang, “Non-EE2: Reconstruction-Reordered IBC for screen content coding”, JVET-Z0159, Apr. 2022.

[3] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-5.0

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).