| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0063-v2 |
| --- | --- |

| Title: | AHG 11: A hybrid codec using E2E image coding combined with VVC video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Y. He, B. Wang, E. Alshina, J. Sauer | Tel: Email: | +49 1588344067 {yu.he1, biao.wang, elena.alshina, johannes.sauer}@huawei.com |
| Source: | Huawei Technologies Dusseldorf GmbH |  |  |

# Abstract

This informative contribution presents a hybrid codec, where I frames are coded using an E2E (End-2-End) image codec while all remaining frames (P and B frames) are coded with VVC. As the chosen end-to-end neural-network based image codec provides a better than VVC coding performance on I frame, the coding performance of P and B frames are also improved due to inter-prediction propagation. Intra coding inside P and B slices remains the same as VVC. Compared to VVC 14.0 under random access configuration, experiments reportedly show that the proposed hybrid codec provides 4.1% BD-rate gain on Y component in random access configuration (3.7%, 5.6%, 3.4% BD-rate gain for ClassA1, ClassA2, and ClassB respectively). It should be noted that gain is demonstrated in random access configuration, while only 1 out of 64 frames coding is modified (for most of the sequences Intra period is 64). In demonstrated codec design the switching between “classical” (CPU) and “end-to-end neural-network based” coding (both CPU and GPU coding is possible) happens at picture level.

# Introduction

Image coding using AI-based learning approach has shown promising coding performance [1] [2]. For example, in JPEG-AI CfP several proposals demonstrated significant (20-30%) coding gain over VVC anchor under JPEG AI CTTC (common training and test conditions). On the other hand, video compression is more complex. In the common configuration RA (random access), frames are coded not in display order and it involves inter-prediction. It seems achieving state-of-the-art video coding performance (such as VVC) at this point is difficult for AI-based learning approach [4][5]. Therefore, this contribution made a try to combine the strong parts of E2E image codec and VVC. All I frames are coded using an E2E image codec, while the remaining P and B frames are kept untouched within the VVC coding framework, as shown in Figure 1.

Figure 1: coding with a hybrid approach, the GOP setup might differ

# Implementation of the hybrid video codec

The hybrid video codec is modified based on VTM-14.0. We kept VVC code untouched until the encoder starts to write NAL units to bitstream. For an I frame, we bypass its associated VCL NAL unit writing while let non-VCL units (such as Picture header, Slice header, etc.) write into bitstream as it is. The VCL NAL unit writing is replaced by the following steps:

- We call E2E image codec written in python script for the corresponding frame in the video according to configuration

- The E2E codec starts execute image coding and write the reconstructed image as an intermediate yuv file.

- The VVC codec collects the bit cost from the log file of E2E codec

- The VVC codec read the reconstructed intermediate yuv file, and overwrite the decoded picture buffer.

- With step 3 and 4, the bit cost and PSNR of image generated by E2E can be calculated by VVC codec.

In this design, the coding of I frame in VVC is largely reserved. In the very end stage of writing decided syntax elements, the image coding is replaced by E2E codec and its coding information is overwritten into VVC framework. The replacing of I image coding could be earlier, where as long as a frame type I is determined in VVC, the above five steps we could invoked. However, the current design facilitate us to do I frame coding adaptively, by comparing VVC I frame coding and E2E Image coding results.

It should be mentioned that switch between “classical” and E2E Neural Network based coding at high (picture / slice / tile, not block) level is beneficial for practical implementation of NN-based video coding technologies.

# End-to-end image codec design

The End-2-End image codec is adjusted for YUV4:2:0 coding variant of response to JPEG-AI CfP [3]. Submitted as JPEG AI CfP response model was trained with equally weighted MS-SSIM and MSE in loss function. For JVET contribution we used exactly the same network, but model trained purely with MSE. The only modification to the submitted to JPEG AI CfP End-2-End image codec is input/out-put data format. Instead reading/writing RGB 4:4:4 images, codec read /writes YUV 4:2:0 I-frame. Codec architecture is shown on (Fig. 2). Key elements of design are: Y and UV components are coded separately (as it is traditionally done in video coding); only in few positions in processing pipeline latent tensor which corresponds to Y component is used as auxiliary input for UV coding (information exchange between Y and UV networks is marked by blue arrows on Fig. 2); entropy network (marked by green on Fig. 2) is quantized (all temporal buffers and NN parameters are within 16 bits) and ensured against overflow (all registers do not exceed 32 bits), this allows achieving device interoperability. For a more detailed introduction of the End-2-End image codec, please refer to [3]

Fig. 2. End-2-end image codec architecture from [3].

# Experimental Results

Objective results

The experimental results on VTM-14 under RA configuration are shown in Table 1. The common test conditions defined in [6] are used to evaluate the proposed method except for QP setting. The QP set {32 37, 42, 47} are used instead of {22, 27, 32, 37}. This was done in order to match rates of Intra frames. JPEG AI CfP target rates are 0.06 ...0.75 bpp [7]. To match VVC anchor to target rates (Fig. 1), QP was selected for each sequence. Median QP is as follows QP=27~0.75 bpp, QP=31~0.5 bpp, QP=37~0.25 bpp , QP=42~0.12 bpp, , QP=46~0.06 bpp. With proposed QP settings we were able to have more or less close bits allocation between anchor (VTM-14.0) and the test (proposed hybrid codec). Fig. 3 presents the RD-curves of the anchor and the test for FoodMarket sequence.

It is observed that BD-rate –3.7%, –5.6%, –3.4% is achieved on Y components by average for sequences in A1, A2 and B classes, respectively. In general, the E2E image codec performs better for high resolution videos as most images in training dataset are high-resolution. This explains the marginal gain achieved in Class C and D sequences. In JPEG AI CfP all proponents must use JPEG AI training set, which doesn’t include screen content, so drop for ClassF is expected (models was not fed with screen content images during training).

Table 1: RA Results of Proposed Hybrid Codec (E2E AI I-frame inserted unconditionally)

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Test over VTM 14.0 |  |  |
|  | Y | U | V |
| Class A1 | -3.7% | -1.4% | -0.4% |
| Class A2 | -5.6% | 3.0% | 9.5% |
| Class B | -3.4% | 7.5% | 4.4% |
| Class C | 0.0% | 18.4% | 14.1% |
| Class E | -9.4% | 3.0% | -0.4% |
| Overall | -4.1% | 6.9% | 5.8% |
| Class D | -0.2% | 7.1% | 7.1% |
| Class F | 58.4% | 75.8% | 83.1% |

Encoder decoder run-time in this very non optimal hybrid implementation is not reliable. But in [3] VVC anchor and shown here E2E AI image codec were tested using same CPU. It was reported CPU decoding to be 34 of VVC. Assuming that Intra frame decoding is 34 slower and intra period is 64 for most of the test sequences, decoding time of this hybrid implementation should be 1.5 of VTM decoder.

Results in Table 1 obtained if All I-frames are coded as E2E AI (unconditionally, no multi-pass coding).  Performance can be even further improved if E2E AI I-frame coding is used not always, but content adaptively (with encoder decision to use E2E AI coding or VTM Intra coding). But this approach would require multi-pass.

Table 2. RA Results of Proposed Hybrid Codec (E2E AI I-frame inserted conditionally)

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Test over VTM 14.0 |  |  |
|  | Y | U | V |
| Class A1 | -3.7% | -1.3% | -0.5% |
| Class A2 | -5.6% | 3.0% | 9.5% |
| Class B | -3.4% | 7.6% | 4.3% |
| Class C | -1.5% | 14.7% | 9.8% |
| Class E | -9.4% | 2.9% | -0.6% |
| Overall | -4.4% | 6.1% | 4.8% |
| Class D | -1.4% | 7.1% | 6.8% |
| Class F | -1.5% | 13.0% | 12.7% |

This contribution concerns more about the coding performance than codec runtime at current stage. Therefore, we run simulation on a cluster without GPU. This would increase the runtime of E2E image codec a lot. Furthermore, the hybrid codec needs some I/O operation time for writing to/reading from the intermediate yuv file, as the expense of communication between E2E codec and VVC. The encoding/decoding runtime is probably sub-optimal and not shown.

Fig. 3. RD-curves for anchor (VTM14.0) and test (VTM14.0 with E2E AI Intra coding), FoodMarket, Random Access configuration.

Subjective results

We also perform a subjective comparison for reconstructed videos from the anchor and test. Figure 4 and 5 presents a crop from CatRobot at QP point 42 in a frame with POC equal to 31 from videos generated by Anchor and test, respectively. It can be observed that the image quality is significant improved in the test. In particular, the rabbit in the bottom right part blurs to a great extent in anchor, while in test its detail is largely reserved.

POC 31 is a frame far away from the first I frame POC 0. Therefore, the subjective results also shows that the improvements in I frame can be propagated to other frames in the hybrid codec.

Figure 4: CatRobot, QP 42, Anchor VTM 14.0, crop from POC 31

Figure 5:  CatRobot, QP 42, VTM 14.0 + E2E Intra, crop from POC 31

# Conclusions

In this contribution, a hybrid codec of an E2E image codec and a VVC codec is demonstrated. It is proposed to switch between “classical” and Neural Network based coding not lower than at picture / slice /tile level.  The E2E image codec is in charge of coding I frames while the remaining frames are coded by conventional VVC codec. The results show that 2-3% gain can be achieved by replacing only I-frames (one out of 64 frames for the majority of videos in CTC).  Proposal submitted only for the demonstration how video coding can benefit from output of JPEG AI standard project.

# Reference

- Zhengxue Cheng, Heming Sun, Masaru Takeuchi, and Jiro Katto. “Learned image compression with discretized Gaussian mixture likelihoods and attention modules”. In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition, pages 7939–7948, 2020.

- Zongyu Guo, Zhizheng Zhang, Runsen Feng, and Zhibo Chen. "Soft then hard: Rethinking the quantization in neural image compression"", In Proceedings of the 38th International Conference on Machine Learning, volume 139, pages 3920–3929. PMLR, 2021.

- Elena Alshina, Atanas Boev, Ze Cui, Yihui Feng, Georgy Gaikov, Tiansheng Guo, Sergey Ikonin, Panqi Jia , Alexander Karabutov, Ahmet Burakhan Koyuncu, Esin Koyuncu, Meng Li , Jue Mao, Sheng Qian, Johannes Sauer, Yibo Shi, Timofey Solovyev, Maxim Sychev, Jing Wang, Dequan Yu, Yin Zhao wg1m96016-REQ-Presentation of the Huawei response to the JPEG AI Call for Proposals: “Device agnostic learnable image coding using primary component extraction and conditional coding

- Oren Rippel, Alexander G Anderson, Kedar Tatwawadi, Sanjay Nair, Craig Lytle, and Lubomir Bourdev. “Elf-vc: Efficient learned flexible-rate video coding”, arXiv preprint 2104.14335, 2021

- Eirikur Agustsson, David Minnen, Nick Johnston, Johannes Balle, Sung Jin Hwang, and George Toderici. Scale-space flow for end-to-end optimized video compression. In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), June 2020

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th Meeting, Geneva, CH, 19–27 Mar. 2019

- E. Alshina, J. Ascenso, T. Ebrahimi, F. Pereira, T. Richter “[AHG 11] Brief information about JPEG AI CfP status”, JVET-AA0047, 27th Meeting, by teleconference, 13–22 July 2022

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).