| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0071-v2 |
| --- | --- |

| Title: | EE1-2.1：A CNN-based Super Resolution Method Combined with GOP Level Adaptive Resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuang Peng, Cheng Fang, Dong Jiang, Jucai Lin, Xue Zhang (Dahua) Junghak Nam, Sunmi Yoo, Jaehyun Lim, Seung Hwan Kim (LGE) | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com junghak.nam@lge.com |
| Source: | ZheJiang Dahua Technology CO.,LTD., LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the EE1-2.1 test results, which is a combination of JVET-Z0088 and JVET-Z0065 test 2.1.1. At each GOP, the encoder can adaptively select a scale factor from ×1.0 and ×2.0 and CNN-based super-resolution is utilized for is the latter case. Compared with VTM-11.0-NNVC, the experimental results show 3.06% and 3.27% BD-rate gains on average (A1 and A2) for luma, under AI and RA configurations, respectively.

Introduction

The JVET-Z0088 [1] shows significant coding gains by using the Convolutional Neural Network (CNN)-based super-resolution method combined with existing RPR functionality. In addition, the results shown in JVET-Z0065 [2] prove that RPR encoder control at GOP level may provide a better coding performance. This contribution tests the combination of JVET-Z0088 and JVET-Z0065 test 2.1.1.

Proposed method

GOP level encoding resolution decision

In this proposal, the resampling factors ×2.0 (half size) and ×1.0 (original size) are selected at GOP level and the scale factor is applied on all pictures in the GOP. To determine whether to resample the picture by ×2.0, the first picture is downscaled to quarter resolution and then resampled to the original resolution. The PSNR is calculated between the original picture and the down-up scaled picture. The scale factor is selected according to a threshold which is adaptively varied based on the initial QP. The pre-defined threshold is the same as that in JVET-Z0065.

CNN-based super-resolution Method

The architecture of the proposed CNN model is shown in Fig.1. The network contains 16 advanced residual blocks (ARB), several convolutional layers, a concatenate layer, a shuffle layer and a shortcut connection from conv1 to conv2. In addition, a global connection from input to output is included.

The parameters of each convolution layer are [cin, k, k, cout], where cin, k and cout represent the number of input channels, the size of convolution kernel and the number of output channels respectively.

The YRpr represents reconstruction up-sampled by RPR, YLR-Rec represents reconstruction, YLR-Pred represents prediction, and QP represents quantization parameter, which are used as the input of the model. QP is expanded to a matrix of the same shape as YLR-Rec before entering the concatenate layer. The output of the model is the super-resolution reconstruction YSR.

In addition, the architecture of the designed ARB is shown in Fig.2, where ReLU, GAP and Softmax are activation function, global average pooling layer and normalization function, respectively. The channel attention extraction module is circled by a dotted line in Fig.2.

Architecture of the proposed CNN model

Architecture of the designed ARB

Training and Inference

The training stage use BVI_DVC 4K dataset [3]. The data is down-sampled by RPR ×2.0 and then compressed by VTM.

Other details of the training and inference stage are as shown in Table 1 and Table 2, respectively.

Network Information for the proposed model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | NVIDIA A40 48GB |
|  | Framework: | PyTorch v1.9.1 |
|  | Number of GPUs per Task | 4 |
|  | Epoch: | ~1000 |
|  | Batch size: | 4Kx16 |
|  | Loss function: | L1 |
|  | Training time (for 1 model): | ~200h |
|  | Training data information: | BVI_DVC 4K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP = 22, 27, 32, 37, 42 |
| Optional |  |  |
|  | Number of iterations | ~100000 |
|  | Patch size | 128x128 |
|  | Learning rate: | 1.00E-04 |
|  | Learning rate update strategy | decayed at epochs 200, 600, 1000 with a factor of 0.5 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | randomly cropped, and rotation |
|  | Mini-batch selection process: |  |
|  | Training data update strategy: |  |
|  | Other information: |  |

Network Information for the proposed model in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | No GPU, Intel(R) Xeon(R) Gold 6242R CPU @ 3.10GHz |
|  | Framework: | Libtorch 1.3.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1,445,249/model |
|  | Total Number of Parameters (All Models) | 2,890,498 |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 11.02637482 |
|  | Multiply Accumulate (kMAC/pixel) | 361 |
|  | Calculation Method | 361 |
| Optional |  |  |
|  | Total Conv. Layers | 68 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 4Kx1 |
|  | Patch size | 512x512 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling | padding |
|  | Other information: |  |

Experimental results

The proposed CNN-based up-sampling filter is tested on top of VTM-11.0-NNVC[4] with the common test conditions[5].

For the anchor and the test, the QP sets are both {22, 27, 32, 37, 42}.

Table 3 and Table 4 shows the test results for AI and RA configurations, respectively. Under AI configuration, the proposed method can bring about 3.06% BD-rate gains on average (A1 and A2) for luma. Under RA configuration, the proposed method can bring about 3.27% BD-rate gains on average (A1 and A2) for luma.

Results of the proposed method under AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |
| sequences | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -3.58% | 2.29% | 2.52% | 98% | 611% |
| Class A2 | -2.54% | 4.01% | 3.14% | 68% | 244% |
| Class B | -0.59% | 1.08% | 1.25% | 68% | 184% |
| Class C | 0.00% | 0.00% | 0.00% | 59% | 55% |
| Class E | 0.00% | 0.00% | 0.00% | 65% | 66% |
| Average on A1 and A2 | -3.06% | 3.15% | 2.83% | 83% | 428% |
| Overall | -1.18% | 1.35% | 1.29% | 69% | 180% |
| Class D | 0.00% | 0.01% | 0.01% | 61% | 70% |
| Class F | 0.00% | 0.00% | 0.00% | 56% | 58% |

Results of the proposed method under RA configuration

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |
| sequences | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -5.61% | 1.55% | 2.74% | 79% | 2023% |
| Class A2 | -0.94% | 1.97% | 1.62% | 65% | 188% |
| Class B | -0.37% | 0.95% | 0.96% | 67% | 267% |
| Class C | 0.01% | 0.00% | 0.01% | 61% | 106% |
| Class E |  |  |  |  |  |
| Average on A1 and A2 | -3.27% | 1.76% | 2.18% | 72% | 1106% |
| Overall | -1.43% | 1.02% | 1.19% | 67% | 291% |
| Class D | 0.01% | 0.01% | 0.01% | 64% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 64% | 111% |

Conclusions

In this contribution, a combination of CNN-based super-resolution and adaptive GOP level encoding resolution decision is tested. The adaptive encoding resolution decision enables the encoder to select the encoding resolution between a original size and a half size. This method shows a promising coding performance.

References

- AHG11: A CNN-based Super Resolution Method Combined with Existing RPR Functionality, JVET-Z0088, S. Peng, D. Jiang, J. Lin, C. Fang, X. Zhang.

- EE1-2.1: RPR encoder with multiple scale factors, JVET-Z0065, J. Nam, S. Yoo, J. Lim, S. Kim.

- https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn , Di Ma, Fan Zhang, and David R. Bull.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-11.0-NNVC.

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

Patent rights declaration(s)

Dahua Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).