| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0072 |
| --- | --- |

| Title: | EE2-2.4: ARMC with refined motion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Kai Zhang2, Na Zhang1, Zhipin Deng1, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports test results for EE2-2.4 adaptive reordering of merge candidates (ARMC) with refined motion. With ARMC with refined motion, each merge candidate in the merge candidate list is refined using TM/multi-pass DMVR first and the refined motion will be used in ARMC to reorder the merge candidate list. When multi-pass DMVR is used to derive the refined motion, only the first pass (i.e., PU level) of multi-pass DMVR is applied in reordering. On top of ECM-5.0, simulation results of EE2-2.4 are reported as below:

RA: {-0.07%, -0.08%, -0.10%; 101%, 101%};

LDB: {-0.05%, -0.03%, 0.09%; 102%, 102%}.

Introduction

Adaptive reordering of merge candidates (ARMC) with refined motion was proposed in JVET-Z0145 [1] and established as Test-2.4 in JVET-Z EE2 [2]. With ARMC with refined motion, each merge candidate in the merge candidate list is refined using TM/multi-pass DMVR first and the refined motion is used in ARMC to reorder the merge candidate list. ARMC with refined motion is applied to template matching (TM) merge mode, adaptive DMVR mode, and template matching for advanced motion vector prediction (TM-AMVP) mode. When multi-pass DMVR is used to derive the refined motion, only the first pass (i.e., PU level) of multi-pass DMVR is applied in reordering. When template matching is used to derive the refined motion, the template size is set equal to 1. Only the above or left template is used during the motion refinement of TM when the block is flat (w > 2×h) or narrow (h > 2×w). TM is extended to perform 1/16-pel MVD precision. The first four merge candidates are reordered with the refined motion in TM merge mode. When constructing the AMVP list, an MVP candidate with a TM cost larger than a threshold, which is equal to five times of the cost of the first MVP candidate, is skipped.

Simulation results

ARMC with refined motion is implemented on top of ECM-5.0 [3]. Simulations are performed following the common test conditions of EE2 [4]. The simulation results of Test #2.4 compared to ECM-5.0 are shown in Table 1.

Table 1: Test results of Test #2.4

|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.04% | -0.19% | 101% | 100% |  |  |  |  |  |
| Class A2 | -0.10% | -0.11% | -0.02% | 101% | 100% |  |  |  |  |  |
| Class B | -0.05% | -0.10% | -0.05% | 101% | 100% | 0.02% | -0.17% | 0.19% | 102% | 102% |
| Class C | -0.09% | -0.12% | -0.14% | 102% | 102% | -0.05% | 0.05% | 0.00% | 102% | 103% |
| Class E |  |  |  |  |  | -0.17% | 0.08% | 0.02% | 102% | 101% |
| Overall | -0.07% | -0.08% | -0.10% | 101% | 101% | -0.05% | -0.03% | 0.09% | 102% | 102% |
| Class D | -0.06% | -0.27% | -0.12% | 102% | 102% | -0.01% | 0.41% | 1.16% | 103% | 103% |
| Class F | -0.02% | -0.01% | 0.07% | 101% | 101% | -0.37% | -0.16% | -0.14% | 101% | 101% |

Conclusions

In this contribution, test result of EE2-2.4 on ARMC with refined motion is reported. It is recommended to adopt ARMC with refined motion into ECM.

Reference

- Y. Wang, K. Zhang, N. Zhang, Z. Deng, and L. Zhang, “EE2-2.5 related: More test results for ARMC with refined motion,” Document of Joint Video Experts Team, JVET-Z0145, by teleconference, April 2022.

- V. Seregin, J. Chen, G. Li, K. Naser, J. Strom, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” Document of Joint Video Experts Team, JVET-Z2024, by teleconference, April 2022.

- ECM-5.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-5.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).