| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0076-v1 |
| --- | --- |

| Title: | AHG11: RPR-Based Super-Resolution Guided by Partition Information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Qihui Han Cheolkon Jung Yang Liu Ming Li | Email: | hanqihui2013@163.com zhengzk@xidian.edu.cn serena@oppo.com myron.li@oppo.com |
| Source: | Xidian University, OPPO |  |  |

_____________________________

- Abstract

In this contribution, we propose a CNN filter for RPR-based super-resolution guided by partition information. Since the blocking artifacts are closely related to block partitioning, the coding tree unit (CTU) partition map is helpful for predicting the blocking artifacts. Thus, we propose a reference spatial attention block (RSAB) for blocking artifact removal based on the CTU partition map. To make full use of correlation between luma and chroma, we use a U-Net backbone that extracts and fuses multi-scales features from an image. Through the U-Net backbone, we design a dilated convolution-based dense block with channel attention. The proposed CNN filter achieves {-9.25% (Y), 8.82% (Cb), -16.39% (Cr)} and {-4.67% (Y), -1.75% (Cb), -11.70% (Cr)} BD-rate changes in all intra (AI) and random access (RA) configurations, respectively.

Introduction

In this contribution, we combine a CNN with the reference picture resampling (RPR) functionality in VVC [1] to achieve super-resolution reconstruction with blocking artifact removal. We use the predicted frame and up-sampled frame by RPR functionality as input and utilize CTU partition information as reference to generate spatial attention for blocking artifact removal. To consider the correlation between the luma and chrominance components, features are extracted by three branches for luma and chroma components. Then, the extracted features are concatenated and fed into a U-net backbone. In this backbone, we adopt dilated convolution to reduce the number of parameters and expand the receptive field, and utilized channel attention to boost the inter-dependency among feature channels. Finally, the SR reconstruction results are generated by three reconstruction branches.

Proposed method

Network Architecture

The network architecture of the proposed CNN filter for RPR-based super-resolution is illustrated in Fig. 1. The proposed network is composed of four parts: feature extraction, reference information generation, mutual information processing and reconstruction.

Fig. 1 Illustration of the proposed network architecture for RPR-based SR guided by partition information.

Specifically, the feature extraction part consists of three convolutional layers, which are used to extract features of the inputs. The convolutional layer is followed by a ReLU activation function. The inputs are the predicted frame and chroma component after RPR upsampling.

The reference information generation (RIG) part consists of eight residual blocks. The first four blocks are performed for predicting the CTU partition information from reconstructed frame with an extra block, while the last three blocks are used for reference information generation. Sequentially, reference information features are input to several convolutional layer sets to generate different scales features as the input of the reference feature attention module. Each layer set consists of a convolutional layer with stride 2 and a convolutional layer followed by a ReLU.

The mutual information processing (MIP) part is based on a U-Net backbone. The module consists of convolutional layers, reference spatial attention blocks (RSAB) and dilated convolutional layers based dense blocks with channel attention (DDCAB). As shown in the Fig. 1, there are four different scales in the module, where the first three scales utilize two DDCABS followed by one RSAB and the last scale utilize four DDCABS followed by one RSAB.

The reconstruction part consists of three branch paths for luma and chroma components. For luma channel, the combined feature is upsampled and put to three convolutional layers followed by an addition operation with the reconstructed luma after RPR upsampling. For chroma channels, the combined feature is concatenated with the initial extracted features and then input to three convolutional layers.

The architecture of the proposed RSAB is shown in Fig. 2. Each block consists of three convolutional layers followed by a ReLU and a Sigmoid function. The reference features are put to the convolutional layers, ReLU and Sigmoid function sequentially. Finally, the input features are multiplied by the processed reference features.

Fig. 2 Network architecture of the proposed reference spatial attention block.

To reduce the number of parameters and expand the receptive field, we integrate dilated convolution and channel attention into a dense block as shown in Fig. 3. The block consists of two parts: 1) A dilated convolution based dense module, which consists of one convolutional layer and three dilated convolutional layers with dilation factors 2, 2, 4. Therefore, the receptive field is larger than normal convolutional layers while keeping the same number of parameters. 2) An optimized channel attention, which can boost the nonlinear relationship between input feature channels compared to the ordinary channel attention. Squeeze-and-excitation (SE) attention is mainly divided into three steps: squeeze, excitation and scale. Global average pooling is performed to squeeze the input feature map. In the excitation step, to avoid the conversion between matrices and vectors, two 1x1 convolutional layers followed by ReLU and sigmoid are utilized, respectively. Finally, we a dot product are performed to establish the intrinsic relationship of features using the adaptive channel weight maps.

Fig. 3 Architecture of the proposed dilated convolutional-based dense block with channel attention.

Training details

DIV2K [2] and BVI_DVC [3] datasets are used for training the proposed network. Partition maps are only utilized in the training procedure as an auxiliary information. For training dataset generation, we utilize official AHG11 anchor (VTM-11.0 [4] + new MCTF) [5] to compress the datasets with QPs {22, 27, 32, 37, 42} in the context of RPR scheme with factor 2.0. The decoded low-resolution frames, high-resolution frames after RPR scheme and partition map are generated for training. The low-resolution and high-resolution frames are cropped into 128x128 and 256x256 patches in 64 and 128 steps, respectively, for training data generation, while the original images are utilized as ground truth. We train five models for different QPs. Other details of the training stage are as follows:

Table 1. Network information of the proposed network in the training stage.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: GeForce RTX 3090 |
|  | Framework: | PyTorch v1.10 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 50 |
|  | Batch size: | 16 |
|  | Training time: | ~30h for each model |
|  | Training data information: | DIV2K and BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1 and L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | - |
|  | Other information: | - |

Inference

We train five models for different QPs {22, 27, 32, 37, 42}. The network information of the proposed network in the inference stage is as follows:

Table 2 Network information of the proposed network in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel(R) Core i9-12900KF CPU @ 2.40GHz |
|  | Framework: | LibTorch v1.7.1 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 2,493,351/model |
|  | Total Parameter Number | 12,466,755 |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 47.55689621 |
|  | Multiply Accumulate (MAC) |  |
| Optional | Total Conv. Layers | 85 |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 |
|  | Patch size | Whole frame |

Experimental results

The proposed super-resolution filter is evaluated on top of VTM-11.0 + new MCTF according to the common test conditions (CTCs) [6]. We evaluate the performance of anchor and the proposed network in the Random Access (RA) and All Intra (AI) configurations with the QPs {22, 27, 32, 37, 42}. Experimental results of AI and RA configurations are shown in Tables 3, 4, respectively. In the AI configuration, the proposed filter achieves {-9.25%, 8.82%, -16.39%} BD-rate changes on average for {Y, U, V} components, respectively. In the RA configuration, the proposed filter achieves {-4.67%, -1.75%, -11.70%} BD-rate changes on average for {Y, U, V} components, respectively.

Table 3 Results of the proposed method under AI configuration.

|  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27, 32, 37, 42) |  |  |
|  |  | Y | U | V |
| Cass A1 4K | Tango2 | -9.18% | -13.60% | -13.82% |
|  | FoodMarket4 | -3.74% | -0.86% | -2.87% |
|  | Campfire | -15.40% | 119.65% | -26.68% |
| Class A2 4K | CatRobot1 | -8.34% | -11.08% | -10.88% |
|  | DaylightRoad2 | -2.79% | -24.23% | -22.67% |
|  | ParkRunning3 | -16.02% | -16.98% | -21.40% |
| Average on A1 |  | -9.44% | 35.06% | -14.46% |
| Average on A2 |  | -9.05% | -17.43% | -18.32% |
| Overall |  | -9.24% | 8.82% | -16.39% |

Table 4 Results of the proposed method under RA configuration.

|  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27, 32, 37, 42) |  |  |
|  |  | Y | U | V |
| Cass A1 4K | Tango2 | -7.09% | -14.51% | -12.95% |
|  | FoodMarket4 | -5.57% | -2.90% | -6.36% |
|  | Campfire | -18.65% | 36.66% | -23.08% |
| Class A2 4K | CatRobot1 | 2.77% | -9.17% | -3.83% |
|  | DaylightRoad2 | 12.50% | -21.21% | -18.26% |
|  | ParkRunning3 | -11.95% | 0.63% | -5.72% |
| Average on A1 |  | -10.44% | 6.41% | -14.13% |
| Average on A2 |  | 1.11% | -9.92% | -9.27% |
| Overall |  | -4.67% | -1.75% | -11.70% |

Conclusions

This contribution proposes an attention-based super-resolution network for video compression guided by partition information. The CTU partition map is helpful for predicting the blocking artifacts and thus we propose a RSAB for blocking artifact removal based on the CTU partition map. A U-Net backbone that extracts and fuses multi-scales features is adopted to make full use of correlation between luma and chroma components. Through the U-Net backbone, a dilated convolution-based dense block with channel attention is designed to expand the receptive field and boost the inter-dependency among feature channels. The proposed CNN filter has achieved {-9.25% (Y), 8.82% (Cb), -16.39% (Cr)} and {-4.67% (Y), -1.75% (Cb), -11.70% (Cr)} BD-rate changes in AI and RA configuration, respectively.

References

[1] B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile video coding (draft 10),” JVET-S2001, 2020.

[2] E. Agustsson and R. Timofte, “Ntire 2017 challenge on single image super-resolution: Dataset and study,” in Proceedings of the IEEE Conference on Computer Vision and Pattern Recognition (CVPR) Workshops, July 2017.

[3]	Di Ma, Fan Zhang, and David Bull, “BVI-DVC: A Training Database for Deep Video Compression,” arXiv:2003.13552, 2020.

[4] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

[5] P. Wennersten, C. Hollmann, and J. Ström, “GOP-based temporal filter improvements,” JVET-V0056, 2021.

[6] S. Liu, A. Segall, E. Alshina, R.-L. Liao, “Common test conditions and evaluation procedures for neural network-based video coding technology,” JVET-W2016, 2021.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).