| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0080-v2 |
| --- | --- |

| Title: | AHG11: Complexity Reduction on Neural-Network Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jay N. Shingala, Shireesh Kadaramandalgi, Ajay Shyam Tong Shao, Arjun Arora, Peng Yin, Fangjun Pu, Taoran Lu, Sean McCarthy | Email: | {jay.shingala, shireesh.kadaramandalgi }@ittiam.com {tshao, pyin}@dolby.com |
| Source: | Ittiam Systems, Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to use CP decomposition to reduce the inference computational complexity of neural-network-based loop filtering. This contribution also proposes to fuse adjacent 1x1 convolutions to achieve additional reduction in computational complexity. The baseline model is taken from JVET-X0140 low complexity model (EE1-1.4.1), which shows BD-rate saving of 5.16%, 5.93%, 5.19% (Y, Cb, Cr, respectively) for AI compared to EE1 VTM anchor. Using block-level KMAC/Pixel as the complexity metric, the baseline model has worst case complexity of 33.6 KMAC/Pixel. The proposed CP decomposition with rank 24 is about 20.1 KMAC/Pixel. The complexity is further reduced to 16.2 KMAC/Pixel by fusing adjacent 1x1 convolutions, i.e., approximately half the complexity of the baseline model. Compared to the baseline model (JVET-X0140 EE1-1.4.1), the simulation shows that CP decomposition with rank 24 plus fusing adjacent 1x1 convolutions has the BD-Rate loss of 0.50%, 0.21%, 0.39% (Y, Cb, Cr, respectively) for AI. Compared to EE1 VTM anchor, the BD-rate saving is 4.68%, 5.72%, 4.81% (Y, Cb, Cr, respectively) for AI.

In v2, one additional method is added to further reduce the complexity of NNLF model, which retains the majority of the luma gains and achieves better coding efficiency for chroma. The method is to split architecture for luma and chroma components between the input network and output network, therefore different number of features can be used for luma and chroma components, indicated by (xL, yC). Combined with CP decomposition and fusion adjacent 1x1 convolutions, when compared against EE1 VTM anchor, the BD-Rate saving for AI (Y, Cb Cr, respectively) is:

- (24L, 8C): 4.99%, 7.92%, 7.59%, with worst case block level complexity of 17.7 KMAC/Pixel

- (20L, 8C): 4.59%, 7.93%, 7.18%, with worst case block level complexity of 14.2 KMAC/Pixel

- (16L, 8C): 4.32%, 8.28%, 7.66%, with worst case block level complexity of 11.3 KMAC/Pixel

# Introduction

Neural-network-based loop filters (NNLF) have shown great potential in improving coding efficiency. However, the complexity is still a concern for real applications. This contribution presents a study on NNLF inference complexity reduction using tensor decomposition methods.

Tensor decomposition is one of the approaches to reduce NN computational complexity [6]. There are several tensor decomposition methods. The depth-wise separable convolution (DSC) approach has been tried in several contributions, such as JVET-X0053[4]. In this contribution, CP decomposition [5] is proposed to reduce NN computational complexity. In addition, because the characteristics of luma and chroma components can be different, split luma and chroma architectures is proposed to reduce the NN computational complexity too. Both methods can be combined to reduce the complexity further.

Among many existing NNLF models [1][2][3], the JVET-X0140 low complexity model (EE1-1.4.1) [2] is chosen as the baseline model for our trial. This model has a good tradeoff on performance versus complexity. The worst case complexity is 33.6 KMAC/Pixel. The simulation results indicate that EE1-1.4.1 achieved 5.78%, 6.80% and 5.03% BD-rate saving for RA and 5.16%, 5.93%, 5.19% BD-rate saving for AI, for Y, Cb and Cr components, respectively.

Figure 1 shows the baseline JVET-X0140 NNLF architecture.

3x3 conv 3x3x10xM

Leaky ReLu

3x3 conv 3x3xKxL

1x1 conv 1x1xMxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

BS info

QStep

Yx4+U+V 72x72x10

NxN

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

Y’x4+U’+V‘ 64x64x6

N’xN’

n Hidden Layers

Figure 1 JVET-X0140 Baseline Model

# Proposal

## CP decomposition

The CP decomposition [5] method approximates a given CNN layer with 4 separable convolutions, as shown in Figure 2. The first convolution is pointwise convolution. The 2nd and 3rd layer are spatial convolutions in X and Y directions. The 4th convolution is again a pointwise convolution in channel dimension.

Regular convolution for output channel t can be written as:

Vx,y,t= i=x-δx+δj=y-δy+δs=1SKi-x+δ, j-y+δ, s, t Ui,j,s

where, U is input tensor with S channels, K is kernel of size δxδxS per output channel, and V is output tensor.

The CP rank R approximation for the above convolution for channel t can be written as:

Vx,y,t= r=1RKtt,ri=x-δx+δKxi-x+ δ, r j=y-δy+δKyj-y+δ, rs=1SKss, r U(i,j,s)

Where kernel K is approximated as

Ki,j,s, t= r=1RKxi-x+δ, r Kyj-y+δ,,r  Kss,r  Ktt,r
and Kx, Ky, Ks, Kt are δxR, δxR, SxR, TxR tensors along different dimensions

The complexity of CP decomposition in terms of MAC/pixel is R(S+2δ+T) as compared to STδ2 for the regular convolution.

If a recommended Rank R≅STS+T is chosen for decomposition, then the improvement factor will be approximately δ2.

For training, CP decomposition can reuse and fine-tune the model trained using regular convolution.

Figure 2  Illustration of CP Decomposition [5]

Application of CP-decomposition to 3x3 convolution layers of the baseline JVET-X0140 Model is shown in Figure 3. The 3x3 convolutions of each hidden layer are decomposed into 4 layers with rank R:

- 1st layer: 1x1xKxR pointwise convolution

- 2nd layer: 3x1xRxR separable convolution

- 3rd layer: 1x3xRxR separable convolution

- 4th layer: 1x1xRxK pointwise convolution

3x3 conv 3x3x10xM

Leaky ReLu

3x3 conv 3x3xKxL

1x1 conv 1x1xMxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

BS info

QStep

Yx4+U+V 72x72x10

NxN

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

Y’x4+U’+V‘64x64x6

N’xN’

n Hidden Layers

1x1 conv 1x1xKxR

3x1 Sep conv 3x1xRxR

1x1 conv 1x1xRxK

1x3 Sep conv 1x3xRxR

Rank R CP decomposition

Layers approximated with CP decomposition

Separable Convolution layers

Figure 3  CP Decomposition of JVET-X0140 baseline model

The complexity of the CP decomposed model is computed as follows:

- MACs in input and output conv layers: 3*3*10*M + 3*3*K*L

- MACs in first hidden layer: M*M + M*K + 2*K*R + 2*3*R

- MACs in other hidden layers: (n-1) * 2 *(K*M + [K*R + 3*R])

For K=24, M=72, R=24, L=6, n = 11, total MACs = 63504*(72*72) to process 1 CTU (128x128) samples.

- Thus, MACs/Pixel = (63504 * 72 * 72) / (128*128) = 20.093 KMAC/Pixel

## Fusion of adjacent 1x1 conv layers:

When decomposing each 3x3 conv layer using CP decomposition, the first decomposed layer (1x1xKxR) can be fused with the 1x1xMxK layer in the hidden unit, effectively resulting in a single layer (1x1xMxR). Similarly, the last CP decomposed layer (1x1xRxK) can be fused with the next hidden unit’s first layer (1x1xKxM) to realize the same functionality by a single (1x1xRxM) layer (as shown in Figure 4). This fusion is possible because there are no nonlinear activations present between the layers that are fused, resulting in further complexity reduction.

There can be minor differences in inference outputs because of differences in floating point operations order. The simulation shows that it has negligible difference in the coding performance of the fused NNLF filter.

The complexity of CP decomposition + fusion of adjacent 1x1 conv layers is computed as follows:

- MACs in input and output conv layers: 3*3*10*M + 3*3*K*L

- MACs in first hidden layer: M*M + M*R+ 2*3*R

- MACs in other hidden layers: (n-2) * [2*R*M + 2*3*R]

- MACs in last hidden layer:  2*R*M + R*K + 2*3*R

For K=24, M=72, R=24, L=6, n = 11, Total Macs = (7776 + 7056 + 32400 + 4176)*(72*72) = 51408*(72*72) to process 1 CTU (128*128) samples.

3x3 conv 3x3x10xM

Leaky ReLu

3x3 conv 3x3xKxL

1x1 conv 1x1xMxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

BS info

QStep

Yx4+U+V 72x72x10

NxN

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

Y’x4+U’+V‘64x64x6

N’xN’

n Hidden Layers

1x1 conv 1x1xKxR

3x1 Sep conv 3x1xRxR

1x1 conv 1x1xRxK

1x3 Sep conv 1x3xRxR

CP decomposition

Layers approximated with CP decomposition

Separable Convolution layers

1x1 conv 1x1xMxR

1x1 conv 1x1xRxM

Thus, MACs/Pixel = (51408 * 72 * 72) / (128*128) = 16.265 KMAC/Pixel

Figure 4  CP Decomposition + fusion of 1x1 conv layer of JVET-X0140 baseline mode

## Split luma and chroma architecture:

The joint Luma-Chroma loop filter model in Figure 1 tries to exploit any cross-channel dependencies of chroma on luma or vice-versa. On the other hand, luma and chroma components also have different signal characteristics and coding error induced distortions. To exploit both cross-channel dependencies and independent characteristics of luma-chroma components, we propose split luma and chroma architecture with reduced number of parameters. The input layer and the first hidden layer jointly process Luma-Chroma samples. The K output feature maps of the first hidden layer are then split into luma (KY) and chroma (KC) feature maps. The split luma and chroma features maps are then independently processed using nY hidden layers for luma and nC hidden layers for chroma which achieves better trade-off for coding gains vs complexity.

The new split architecture can be combined with CP decomposition and fusing of adjacent 1x1 conv layer to further reduce the complexity, as shown in Figure 5.

Ky

NxN

KC

Channel Concat

LY

NxN

LC

3x3 conv 3x3x10xM

Leaky ReLu

3x3 conv 3x3xKYxLY

1x1 conv 1x1xMxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

Leaky ReLu

3x3 conv 3x3xKYxKY

1x1 conv 1x1xMYxKY

BS info

QStep

Yx4+U+V 72x72x10

NxN

1x1 conv 1x1xKYxMY

Leaky ReLu

3x3 conv 3x3xKYxKY

1x1 conv 1x1xMYxKY

Y’x4+U’+V‘ 64x64x6

N’xN’

nY Hidden Layers (Luma)

nC Hidden Layers (Chroma)

1x1 conv 1x1xMCxKC

1x1 conv 1x1xKCxMC

3x3 conv 3x3xKCxKC

1x1 conv 1x1xMCxKC

3x3 conv 3x3xKCxKC

1x1 conv 1x1xKCxMC

Leaky ReLu

3x3 conv 3x3xKCxLC

Leaky ReLu

1x1 conv 1x1xKYxMY

Layers approximated with CP decomposition

Separable Convolution layers

3x3 conv 3x3xKYxKY

Rank RY CP decomposition

1x1 conv 1x1xKYxRY

3x1 Sep conv 3x1xRYxRY

1x1 conv 1x1xRYxKY

1x3 Sep conv 1x3xRYxRY

Illustration of CP decomposition in Luma layer hidden unit

Figure 5  Split luma and chroma model + CP Decomposition + fusion of 1x1 conv layer

The complexity of split luma and chroma model + CP decomposition + fusion of adjacent 1x1 conv layers is computed as follows:

- Total MACs in luma path:

  - Intermediate hidden layers = (nY - 2) * 2 * (RY * MY+ 3 * RY)

  - First hidden layer = (KY * MY + MY * RY + 2 * 3 * RY)

  - Last hidden layer = (MY * RY + RY * KY + 2 * 3 * RY)

  - Last Conv layer = 9 * KY * LY

- Total MACs in chroma path:

  - Intermediate hidden layers = (nC - 2) * 2 * (RC * MC + 3 * RC)

  - First hidden layer = (KC * MC + MC * RC + 2 * 3 * RC)

  - Last hidden layer = (MC * RC + RC * KC + 2 * 3 * RC)

  - Last conv layer = 9 * KC * LC

- Total MACs in common path = 9 * 10 * M + M * M + K * M + 2 * (3 * R + K * R)

The Total Macs = (Total MACs in luma path + Total MACs in chroma path + Total MACs in common path ) * (72 * 72) to process 1 CTU (128 * 128) samples.

- Thus, MACs/Pixel = Total Macs / (128 * 128)

In our simulation, K=24, M=72 is used for common path. For split luma and chroma path, we tested three settings:

- common parameters:

  - luma path: nY = 10, LY = 4

  - chroma path: nC = 10, LC = 2

- setting 1: (24L, 8C)

  - luma path: KY = 24, MY = 72, RY = 24

  - chroma path: KC = 8, MC = 24, RC = 8

- setting 2: (20L, 8C)

  - luma path: KY = 20, MY = 60, RY = 20

  - chroma path: KC = 8, MC = 24, RC = 8

- setting 3: (16L, 8C)

  - luma path: KY = 16, MY = 48, RY = 16

  - chroma path: KC = 8, MC = 24, RC = 8

The block-level complexity in KMACs/Pixel is shown in Table 1.

Table 1 Block-level complexity measure in KMacs/Pixel for split luma and chroma model + CP + fusion

| Split luma+chroma model | complexity in KMAC/Pixel |
| --- | --- |
| (24L, 8C) | 17.7 |
| (20L, 8C) | 14.2 |
| (16L, 8C) | 11.3 |

# Simulation results

Simulations are performed based on the common test conditions for neural-network-based video coding technology [8]. Only AI results are available. Table 1 shows our proposal CP decomposition + fusion of adjacent 1x1 conv compared against VTM EE1 VTM anchor. Table 2 shows our proposal CP decomposition + fusion of adjacent 1x1 conv compared against JVET-X0140 EE1-1.4.1 baseline model. Encoder time ratio is left blank as we do not have reliable timing for encoding. Only decoding time is shown here for comparison. From the AI results in Table 2, it can be seen that the proposed loop filter with CP decomposition and fusion of adjacent 1x1 convolutions shows a decoder time reduction of 24% compared to baseline JVET-X0140-1 for luma coding loss of 0.5%. The decoder run time reduction is not linear with the 2x reduction in MAC/pixel as the proposed neural network is not adequately optimized for decoding process.

Table 2  Simulation results of CP decomposition + fusion compared against EE1 VTM Anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0+V0056 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -4.56% | -4.48% | -4.23% |  | 2343% |
| Class A2 | -4.24% | -5.95% | -4.69% |  | 1858% |
| Class B | -4.23% | -5.90% | -4.97% |  | 1924% |
| Class C | -4.52% | -6.76% | -5.07% |  | 1498% |
| Class E | -6.21% | -5.02% | -4.88% |  | 2288% |
| Overall | -4.68% | -5.72% | -4.81% |  | 1925% |
| Class D | -4.94% | -5.07% | -4.86% |  | 1550% |

Table 3  Simulation results of CP decomposition + fusion compared against JVET-X0140 EE1-1.4.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-X0140 EE1-1.4.1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.17% | 0.16% | 0.16% |  | 76% |
| Class A2 | 0.57% | -0.27% | -0.41% |  | 76% |
| Class B | 0.54% | 0.36% | 0.35% |  | 76% |
| Class C | 0.52% | -0.04% | 0.33% |  | 76% |
| Class E | 0.69% | 0.82% | 1.58% |  | 76% |
| Overall | 0.50% | 0.21% | 0.39% |  | 76% |
| Class D | 0.45% | 0.18% | -0.05% |  | 74% |

Additional simulation for combined split luma and chroma model + CP decomposition + fusion of adjacent 1x1 conv compared to EE1 VTM anchor is shown in Table 4 for setting 1 (24L, 8C) , Table 5 for setting 2 (20L, 8C) and Table 6 for setting 3 (16L, 8C).

Table 4  Simulation results of split luma and chroma model + CP decomposition + fusion compared using (24L, 8C) against EE1 VTM Anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0+V0056 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -4.51% | -7.07% | -6.72% |  | 3133% |
| Class A2 | -4.58% | -9.10% | -6.39% |  | 2463% |
| Class B | -4.50% | -7.85% | -8.00% |  | 2558% |
| Class C | -5.05% | -8.52% | -8.68% |  | 2377% |
| Class E | -6.59% | -6.92% | -7.53% |  | 3558% |
| Overall | -4.99% | -7.92% | -7.59% |  | 2733% |
| Class D | -5.45% | -7.50% | -9.52% |  | 2643% |

Table 5  Simulation results of split luma and chroma model + CP decomposition + fusion compared using (20L, 8C) against EE1 VTM Anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0+V0056 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -4.37% | -7.15% | -6.18% |  | 2867% |
| Class A2 | -4.24% | -8.29% | -6.13% |  | 2259% |
| Class B | -4.15% | -7.98% | -7.62% |  | 2345% |
| Class C | -4.45% | -8.37% | -7.53% |  | 1846% |
| Class E | -6.10% | -7.71% | -8.03% |  | 2808% |
| Overall | -4.59% | -7.93% | -7.18% |  | 2355% |
| Class D | -4.89% | -7.35% | -8.43% |  | 1950% |

Table 6  Simulation results of split luma and chroma model + CP decomposition + fusion compared using (16L, 8C) against EE1 VTM Anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0+V0056 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -4.09% | -7.22% | -6.90% |  | 2514% |
| Class A2 | -3.99% | -8.46% | -6.50% |  | 1995% |
| Class B | -3.92% | -8.48% | -8.56% |  | 2078% |
| Class C | -4.23% | -8.97% | -8.23% |  | 1621% |
| Class E | -5.69% | -7.88% | -7.33% |  | 2463% |
| Overall | -4.32% | -8.28% | -7.66% |  | 2074% |
| Class D | -4.67% | -8.29% | -9.37% |  | 1712% |

# Training and inference information

The models are trained based on DIV2K [7] for AI only. The raw image/video data was compressed using AI configurations to create the training examples. As in JVET-X0140 EE1-1.4.1, joint models are used for luma and chroma components. 2 models are trained using the AI compressed data.

More information related to the training & inference of models can be found in the tables below.

Table 7  Inference information of CP decomposition + fusion

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  |  | CP-decomposed | CP-decomposed + 1x1 fusion |
| Mandatory | HW environment: |  |  |
|  | GPU Type | CPU only |  |
|  | Framework: | Tensorflow 2.8.0 |  |
|  | Number of GPUs per Task | 0 |  |
|  | Number of Parameters (Each Model) | 64902 | 52566 |
|  | Total Parameter Number | 129804 | 105132 |
|  | Parameter Precision (Bits) | Floating (32bits) |  |
|  | Memory Parameter (MB) | 0.507 | 0.411 |
|  | MAC/Pixel (Kilo) | 20.1 | 16.2 |
| Optional |  |  |  |
|  | Total Conv. Layers | 68 | 46 |
|  | Total FC Layers | 0 |  |
|  | Total Memory (MB) |  |  |
|  | Batch size: |  |  |
|  | Patch size | 72x72 (4 luma and 2 chroma planes) |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage (Total) |  |  |
|  | Peak Memory Usage (per Model) |  |  |
|  | Other information: |  |  |

Table 8  Training information of CP decomposition + fusion

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Nvidia GeForce RTX3090 |
|  | Framework: | Tensorflow 2.8.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~374 |
|  | Batch size: | 16 |
|  | Training time: | 124h |
|  | Training data information: | DIV2K |
|  | Training configurations for generating compressed training data (is different to VTM CTC): | QP=20,25,30,35,40,45 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 72x72 (4 luma and 2 chroma planes) |
|  | Learning rate: | Epoch : LR, 0-56: 2E-5, 56-374: 2E-6 |
|  | Optimizer: | ADAM |
|  | Loss function: | 12-1-1 Weighted L2 |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Other information: |  |

Table 8  Inference information of split luma and chroma model + CP decomposition + fusion

| Network Information in Inference Stage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Split LC (16L, 8C) | Split LC (20L, 8C) | Split LC (24L, 8C) |  |
| Mandatory | HW environment: |  |  |  |  |
|  | GPU Type | CPU only |  |  |  |
|  | Framework: | Tensorflow 2.8.0 |  |  |  |
|  | Number of GPUs per Task | 0 |  |  |  |
|  | Number of Parameters (Each Model) | 36974 | 46066 |  | 57014 |
|  | Total Parameter Number | 73948 | 92132 |  | 114028 |
|  | Parameter Precision (Bits) | Floating (32bits) |  |  |  |
|  | Memory Parameter (MB) | 0.282 | 0.368 |  | 0.435 |
|  | MAC/Pixel (Kilo) | 11.3 | 14.2 |  | 17.7 |
| Optional |  |  |  |  |  |
|  | Total Conv. Layers | 88 |  |  |  |
|  | Total FC Layers | 0 |  |  |  |
|  | Total Memory (MB) |  |  |  |  |
|  | Batch size: |  |  |  |  |
|  | Patch size | 72x72 (4 luma and 2 chroma planes ) |  |  |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |  |  |
|  | Peak Memory Usage (Total) |  |  |  |  |
|  | Peak Memory Usage (per Model) |  |  |  |  |
|  | Other information: |  |  |  |  |

# Conclusion

This contribution shows the potential of using split luma and chroma model combining with CP decomposition plus fusion of 1x1 convolutional layer to achieve computational complexity reduction by approximately half compared to the JVET-X0140 low complexity model (EE1-1.4.1). The AI luma gain is about 4.68% with worst case complexity of 16.2 KMAC/Pixel for CP decomposition fused model. When applying split luma-chroma architecture plus CP decomposition fused model, the AI luma gain is about 5% with worst case complexity of 17.7 KMAC/pixel for (24L,8C) split. The architecture retains majority of the luma gain and achieve higher chroma gain with worst case complexity further reduced to 14.2 KMAC/Pixel for (20L,8C) split and 11.3 KMAC/Pixel for (16L,8C) split, respectively. The contribution proposes to further study the complexity reduction framework for RA case.

# References

- Y. Li, K. Zhang, L. Zhang, H. Wang, J. Chen, K. Reuze, A.M. Kotra, M. Karczewicz, “EE1-1.6: Combined Test of EE1-1.2 and EE1-1.4”, JVET-X0066, teleconference, Oct 2021

- H. Wang, J. Chen, K. Reuze, A.M. Kotra, M. Karczewicz, “EE1-1.4: Tests on Neural Network-based In-Loop Filter with Constrained Computational Complexity”, JVET-X0140, teleconference, Oct 2021

- L. Wang, X. Xu, S. Liu, “EE1-1.1: neural network based in-loop filter with constrained storage and low complexity”, JVET-Y0078, teleconference, Jan 2022

- L. Wang, S. Lin, X. Xu, S. Liu, X. Li “E1-1.5: neural network based in-loop filter using depthwise separable convolution and regular convolution”, JVET-X0053, teleconference, Oct 2021

- V. Lebedev, Y. Ganin et al “Speeding-up Convolutional Neural Networks Using Fine-tuned CP-Decomposition”, ICLR 2015

- M. Jaderberg, A. Vedaldi et al “Speeding up Convolutional Neural Networks with Low Rank Expansions”, Proceedings of the British Machine Vision Conference (BMVC), 2014

- R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool, https://data.vision.ee.ethz.ch/cvl/DIV2K/,

- S. Liu, A. Segall, E. Alshina, R.-L. Liao, “Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-W2016, July 2021

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).