| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 27th Meeting, by teleconference, 13–22 July 2022 | Document: JVET-AA0086-v1 |
| --- | --- |

| Title: | AHG11: SADL update |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Franck Galpin Thierry Dumas Philippe Bordes Edouard François | Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This contribution presents updates in the Small AdHoc Deep Learning (SADL) library already described in JVET-W0181, JVET-Y0110 and JVET-Z0161.

# Overview

The table below summarizes the framework characteristics.

| Language | Pure C++, header only. |
| --- | --- |
| Footprint | ~5200 LOC, library ~200kB, no dependency |
| Optimization | Some SIMD at hot spots (best effort) |
| Compatibility | Onnx to sadl converter |
| Layer Supports | constants, add, maxPool, matMul, reshape, ReLU, conv2D, mul, concat, max, leakyReLU, shape, expand, preLU, flatten, transpose |
| Type support | float, int32, int16, int8 |
| Quantization | Support adaptive quantizer per layer |
| License | BSD 3-Clause |

# Changelog

Changes are also available in the repository at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/sadl

## Changes

- new sample programs:

  - count_mac: count number of MAC/samples, overflow, number of operations

  - debug_model: print information and issues on model (layers parameters, SIMD implementation etc.)

  - sample_generic, sample_simd256, sample_simd512: test a model at various level of optimization

  - naïve_quantization: convert a model from float to integer model knowing the layers quantizers

- added clang-format file

- change the Add layer quantizer policy: use the smallest quantizer instead of the 2nd input one. BiasAdd layer keeps the same policy.

## Improvements

- improve SIMD support for float (AVX512, conv1x1 etc.) (EE1.6, EE1.2 etc.)

- go back to HWCD loop order. Some gains on big models.

- New nbOutputs() method in Model

- Sanity check on quantizer values for some layers

- better inference debug info

- SSE42 for convolution in mod8 case (speedup JVET-Z0082)

- Update documentation

## Corrections

- init function is now re-entrant: it is possible to do inference with multiple input sizes by calling init with the new size

- remove all MAC counters in SIMD paths + add warning message

- carry over quantizer values for Expand and transpose layers (JVET-Z-EE-1.6)

- add conditions for special tensors multiplication case (JVET-Z0082)

- added leakyRelu case in naive_quantization sample code (JVET-Z-EE-1.5)

## Cleaning

- clean the naming for conv2d (simd, dispatch etc.)

- clean messages for SIMD support

- clean macro for debug

- clean sample_test.sh script

- clang format everything

- copyright year

- README update

- other minor cleanings

# Recommendations

## Normalization

Use power of 2 normalizer for inputs: it allows to prepare inputs of quantized models using only bit shifts.

Example, assuming a 10bits input:

- In float: input[k]=buffer[k]/1024.f     (not 1023!)

- In integer: input[k]=buffer[k]<<3      (for a quantizer q=13)

## Inputs/outputs preparation

Prepare inputs outside SADL. Do not use SADL layers just to prepare inputs (e.g. Concatenate, Reshape, Expand…). It makes later inputs quantization more difficult to control.

For the same reason, do not manipulate final output inside SADL. Especially, for residual model, perform the final addition (reconstruction+model output) outside the model, so that the quantization control is easier.

## Receptive field – translation equivariance

For flexibility purpose, a border at least equal to the receptive field and a translation equivariant model might be desirable.

Larger border increases the theoretical complexity but in practice, it also avoids costly testing for border cases or border padding. In practice, in the SADL implementation, it is usually faster to have a larger border and activate the skip_border option.

Translation equivariance also allows to reduce the computation overhead by processing larger blocks or increase the parallelism by processing smaller blocks.

More details can be found in JVET-AA0083.

## Dynamic quantization

Several approaches are possible to quantize the models. Here we show a simple approach where the model is fine-tuned taking into account the quantization policy of SADL. Note that initial quantizers q can be found by a range analysis of the weights and latents.

In the following:

- the layer Q denotes a quantization layer using the quantizer 2q

- the layer Q-1 denotes a dequantization layer using the quantizer 2q

- the clip layer clips the values to be in the range of an int16 (or the target integer).

- The box in grey represents the added layers.

The layer names are the same as in ONNX specifications.

### Conv2D, MatMul, Mul

Figure 1 In grey: layers to add to the original model to simulate the quantization

### Add, BiasAdd

Figure 2 In grey: layers to add to the original model to simulate the quantization

### Concat

Figure 3 In grey: layers to add to the original model to simulate the quantization

### Max

Figure 4 In grey: layers to add to the original model to simulate the quantization

### Others

Figure 5 In grey: layers to add to the original model to simulate the quantization

Other operations with only one input just keep the same quantizer as the input at the output.

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.